SOLUTION: FXM 05: Typical transmitting station

Prepare an electronic notice for notifying to the BR, for the recording in the Master Register of frequency 935.8 MHz used by several base stations in your country. is taken as notifying Administration)

Solution

- Frequency assignments relating to a number of stations may be notified in the form of the characteristics of a typical station and the intended geographical area of operation (No. 11.17 of the Radio Regulations). This provision doesn’t apply to all service types. However, it does apply to fixed and mobile services.

- To identify the notice type it is possible to use the utility “Wizard” from TerRaNotices.

- The class of station for base station is FB. The preface to the BR IFIC provides a complete list of Class of stations in Chapter IV, Section 6.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list in Chapter IV, Section 7.

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

- The Frequency does not fall within the bands shared on equal basis with space services, the values for Antenna gain and Radiated power shall be notified in dipole reference.

Electronic notice file:

- Open TerRaNotices and Select “Wizard” from File menu. ( TerRaNotices software can be installed from BRIFIC DVD)

The Wizard functionality of TerRaNotices, guides you to select the notice type to use according to the given criteria.

For this exercise we select INS as notifying Administration, and “Update of the Master International Frequency Register (MIFR) under Article 11 of the RR” as requested by the exercise.

- Next we specify the required action as “ADD”.  Note the candidate notice types at the bottom for the specified criteria at each stage.

- The Class of station and Station Type shall be selected from the ”Notice’s Parameter” menu  as “FB, Typical station”  for the Typical station and 935.8 MHz for the frequency. (Please refer to the PREFACE to BRIFIC which can be found within the BRIFIC DVD).

- Note that the Notice type selected by the wizard for this requirement is “T14”

- Then confirm the summary to generate the required Notice form.  And fill all the required fields to complete the notice.

- The complete Notice form is shown below with the following screen.

- Validate and save the text file containing the complete T14 notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.