LFMF 02: Request of publication in Part B.

Prepare an electronic notice file for notifying the publication of a notice in Part B of GE75 Plan

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices facility “Generate TB notices”  to request a publication of a notice in Part B of the GE75 Plan.

Solution

Use the TerRaNotices facility (Generate TB notices) to select the corresponding frequency notice from the GE75 Plan

- The notice type to use for requesting the publication of a notice in Part B of the plan GE75 is : TB8

- The Notifying Administrations shall make sure the required coordination is made between concerned Administrations.

- The request for publication in Part B can be submitted after coordination period.

Electronic notice file:

- Open TerRaNotices and choose Generate TB notices from Tools menu and select TB8 for requesting publication of a notice in Part B.

- Then continue with “New search” and select the Administration. For this example we will select IRN and Fragment GE75, and enter the required Frequency.

- The list of all notices T03/ADD with this Frequency will appear. Then we need to identify the required notice with the Geographical coordinates.  In this case there is 1 T03/ADD with Unique id “khoy” corresponding to the required Frequency and Geographical coordinate. Thus select it and proceed to generate the TB8.

- Generate the required TB8 notice.

- Validate and save the file ready to be submitted.

Please note that TerRaNotices tool is still under development for fragment LF/MF. To save the notice please click on “Ignore warnings and proceed” at the bottom. This notice is ready to be submitted to ITU via WISFAT.