| Radionavigation Satellite Service Res 609 CONSULTATION MEETING |  |
| --- | --- |
|  | Document RNSS-Consult/ |
|  | July 2018 |

Russian Federation

This document presents the results of the calculations of the aggregate equivalent power flux-density (epfd) produced by RNSS systems in the band 1164-1215 MHz.

Appendix 1 to this document contains comparison of the calculation results of the maximum epfd for a single radionavigation satellite service (RNSS) system performed on the basis of updated information that Administrations submitted to the Forum on Res. 609 to 15th Consultation meeting with the calculations of the maximum epfd performed by the administration of Russian Federation.

1.	Introduction

The maximum epfd level has been calculated using the methodology described in
ITU-R Recommendation M.1642 «Methodology for assessing the maximum aggregate equivalent power flux-density at an aeronautical radionavigation service station from all radionavigation-satellite service systems operating in the 1 164-1 215 MHz band».

2	Systems taken into account in the calculations

The conducted calculations of the aggregate epfd in this document took into account only those RNSS systems, that data confirmation was submitted for.

The calculations were conducted taking 32 networks/systems (7 NGSO ones and 25 GSO ones) in total into consideration.

Table. 1 Systems accounted in the calculations

| Имя сети РНСС | Администрация | Орбитальная позиция |
| --- | --- | --- |
| COMPASS-B-144.5E | CHN | 144.5 |
| COMPASS-B-84E | CHN | 84 |
| COMPASS-110.5E 1 | CHN | 110.5 |
| COMPASS-140E 1 | CHN | 140 |
| COMPASS-160E 1 | CHN | 160 |
| COMPASS-58.75E 1 | CHN | 58.75 |
| COMPASS-80E 1 | CHN | 80 |
| COMPASS-IGSO 1 | CHN | N-GSO |
| COMPASS-MEO 1 | CHN | N-GSO |
| MSATNAV-2 2 | F | N-GSO |
| INMARSAT-4A 143.5E | G | 143.5 |
| INMARSAT-4A 98W | G | -98 |
| INMARSAT GSO-2N | G | 64 |
| INSAT-NAVR(32.5) | IND | 32.5 |
| INSAT-NAV-55E | IND | 55 |
| INSAT-NAVR-83E (IRNSS) | IND | 83 |
| INSAT-NAVR-83E (GAGAN) | IND | 93.5 |
| INSAT-NAV-129.5E 3 | IND | 131.5 |
| INSAT-NAVR-GS | IND | N-GSO |
| QZSS | J | N-GSO |
| QZSS-GS3 | J | 123 |
| QZSS-GS4 | J | 127 |
| GLONASS-M 4 | RUS | N-GSO |
| LM-RPS-107.3W | USA | -107.3 |
| LM-RPS-133W | USA | -133 |
| NAVSTAR GPS-IIRF | USA | N-GSO |
| LUX-G6-2-E | LUX | 5 |
| LUX-G7-9-E2 | LUX | 31.5 |
| LUX-G9-38-A | LUX | -129 |
| NIGCOMSAT-1R | NIG | 42.5 |
| ALCOMSAT-24.8W | ALG | -24.8 |
| RAGGIANA-18 | PNG | -117 |

1 In accordance with initial data provided by Administration of China COMPASS-110.5E, COMPASS-140E, COMPASS-160E, COMPASS-58.75E, COMPASS-80E, COMPASS-IGSO и COMPASS-MEO satellite networks operate both in global mode and in regional mode, therefore EPFD levels were taken into account twice for each of these networks.

2 In accordance with § 5 of Terms of Reference for the Resolution 609 (WRC-03) Consultation Meetings, the following filings remain available for Galileo and shall be treated with MSATNAV-2 filing as a single planned RNSS system for purposes of performing the epfd calculations having the characteristics presented in this document: MSATNAV-3 and 4, GALILEO-NAV-2004, GALILEO-M-NAVSTAR, E-NSS-1, and SNS.

3 In accordance with initial data provided by Administration of India only one of the following satellite networks will be brought into use: INSAT-NAVR(120.5), INSAT-NAVR(121.5), INSAT-NAVR(123.5), INSAT-NAVR(126.5), INSAT-NAVR(127.5), INSAT-NAVR(129.5), INSAT-NAV(131.5). For the purpose of aepfd calculation the satellite network INSAT-NAV(129.5) can be considered.

4 In accordance with initial data provided by Administration of Russia, GLONASS-M satellite network uses two types of signal: FDMA and CDMA signals.

3.	Results of the aggregate epfd calculation

3.1 	Results of the aggregate epfd calculation based on the data on RNSS systems submitted by Administrations

For each system indicated in Table 1 Administrations submitted an epfd matrix with the 1° step via both latitude and longitude and a Spectral Adjustment Factor (SAF) mask.

The aggregate epfd level for each1 MHz in the band 1164-1215 MHz has been calculated based on the submitted data. The results are shown in Table 2 and Figure 1.

Table 2. Maximum level of aggregate epfd

| Frequency (MHz) | Max. aggregate epfd dB(W/m2)/MHz | Frequency (MHz) | Max. aggregate epfd (dB(W/m2)/MHz | Frequency (MHz) | Max. aggregate epfd dB(W/m2)/MHz | Frequency (MHz) | Max. aggregate epfd dB(W/m2)/MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -139,16 | 1177 | -123,13 | 1190 | -135,68 | 1203 | -125,68 |
| 1165 | -140,41 | 1178 | -123,56 | 1191 | -134,78 | 1204 | -124,80 |
| 1166 | -140,07 | 1179 | -124,39 | 1192 | -134,82 | 1205 | -124,48 |
| 1167 | -139,13 | 1180 | -124,75 | 1193 | -135,63 | 1206 | -123,68 |
| 1168 | -136,16 | 1181 | -125,84 | 1194 | -136,48 | 1207 | -122,54 |
| 1169 | -132,85 | 1182 | -127,65 | 1195 | -138,02 | 1208 | -123,59 |
| 1170 | -129,36 | 1183 | -130,14 | 1196 | -138,56 | 1209 | -125,30 |
| 1171 | -126,92 | 1184 | -132,86 | 1197 | -137,68 | 1210 | -125,90 |
| 1172 | -126,16 | 1185 | -136,21 | 1198 | -135,36 | 1211 | -127,03 |
| 1173 | -124,92 | 1186 | -139,00 | 1199 | -132,93 | 1212 | -128,30 |
| 1174 | -124,28 | 1187 | -141,30 | 1200 | -130,40 | 1213 | -130,16 |
| 1175 | -123,40 | 1188 | -139,85 | 1201 | -128,41 | 1214 | -132,44 |
| 1176 | -121,90 | 1189 | -137,64 | 1202 | -126,77 | 1215 | -135,70 |

Figure 1. Maximum level of aggregate epfd

3.2	Results of the aggregate epfd calculation base on the RNSS system data obtained by administration of Russian Federation

Input data (1° epfd matrix) for INSAT-NAVR-GS system was obtained by the administration of Russian Federation (see Appendix 1).

Based on this data as well as the normalized signal power spectrum, maximum aggregate epfd for each 1 MHz of the band 1164-1215 MHz was calculated. The results of this calculation are given in Table 3 and in Figure 2.

Table 3. Maximum level of Aggregate epfd

| Frequency (MHz) | Max. aggregate epfd dB(W/m2)/MHz | Frequency (MHz) | Max. aggregate epfd (dB(W/m2)/MHz | Frequency (MHz) | Max. aggregate epfd dB(W/m2)/MHz | Frequency (MHz) | Max. aggregate epfd dB(W/m2)/MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -139,16 | 1177 | -123,17 | 1190 | -135,77 | 1203 | -125,68 |
| 1165 | -140,41 | 1178 | -123,58 | 1191 | -134,76 | 1204 | -124,80 |
| 1166 | -140,07 | 1179 | -124,40 | 1192 | -134,84 | 1205 | -124,48 |
| 1167 | -139,13 | 1180 | -124,86 | 1193 | -135,62 | 1206 | -123,68 |
| 1168 | -136,16 | 1181 | -125,96 | 1194 | -136,48 | 1207 | -122,54 |
| 1169 | -132,84 | 1182 | -127,76 | 1195 | -138,02 | 1208 | -123,59 |
| 1170 | -129,44 | 1183 | -130,15 | 1196 | -138,57 | 1209 | -125,30 |
| 1171 | -126,97 | 1184 | -132,85 | 1197 | -137,68 | 1210 | -125,90 |
| 1172 | -126,27 | 1185 | -136,21 | 1198 | -135,36 | 1211 | -127,03 |
| 1173 | -124,94 | 1186 | -139,00 | 1199 | -132,93 | 1212 | -128,30 |
| 1174 | -124,30 | 1187 | -141,30 | 1200 | -130,40 | 1213 | -130,16 |
| 1175 | -123,47 | 1188 | -139,85 | 1201 | -128,41 | 1214 | -132,44 |
| 1176 | -122,03 | 1189 | -137,64 | 1202 | -126,77 | 1215 | -135,70 |

Figure 2. Maximum level of aggregate epfd

4.	Conclusion

The maximum aggregate epfd based on the RNSS data submitted by administrations in the band 1164-1215 MHz is -121.9 dBW/m2/MHz, i.e. a margin of 0.4 dB in respect of  Resolution 609 limit of –121.5 dBW/m2/MHz is ensured.

The maximum aggregate epfd in the frequency band 1164-1215 MHz obtained by the administration of Russian Federation is –122.03 dBW/m2/MHz, i.e. a margin of 0.53 dB in respect of  Resolution 609 limit of –121.5 dBW/m2/MHz is ensured.

The difference between the maximum aggregate epfd values based on the RNSS data submitted by administrations and obtained by the administration of Russian Federation in the frequency band 1164-1215 MHz is 0.13 dB.

Appendix 1

Comparison of the Single System Maximum epfd results obtained by applying different soft tools

1. Introduction

According to §9 of the Terms of Reference of the Resolution 609 Consultation Meeting:

“in the absence of an ITU-validated epfd calculation software that could be used for the calculation of the epfd produced by each RNSS system in the band 1164-1215 MHz, administrations will use their own software to calculate the epfd produced by each RNSS system separately”.

This Appendix contains comparisons of calculation results of the maximum epfd levels from a single RNSS system obtained by different soft tools.

The comparison was carried out for those RNSS systems which Administrations submitted updated data.

Updated information for the INSAT-NAVR-GS system was submitted to the Forum on Resolution 609 at the ITU website.

The maximum epfd was calculated with the software used by the administration of Russian Federation, and then the results were compared with the results provided by administration of India.

The software used by the administration of Russian Federation is based on the methodology described in the Annex 1 to Appendix 1 to ITU-R Recommendation M.1642.

It should also be noted that during the calculation appeared a number of questions related to the orbital positions of the non-GSO constellation of the INSAT-NAVR-GS system: according to the updated information provided by the administration of India, the INSAT-NAVR-GS constellation consists of 4 orbits with an inclination of 29 degrees. However, in accordance with RAAN values presented in the original data submitted by the administration of India to the 15th Consultation Meeting on Resolution 609, the number of orbital planes is six. In this regard, it is necessary to clarify the initial data on the INSAT-NAV-GS system.

2.    Comparison of the maximum epfd calculation results

For comparison, the maximum epfd values were selected for all latitudes with the 1° step obtained by applying different soft tools.

This section gives the results in terms of maximum epfd, for NGSO and GSO RNSS systems which Administrations submitted updated data.

The calculations were based on the input data concerning RNSS as submitted by Administrations to the Resolution 609 Forum at ITU website. The obtained results were then compared with the epfd results provided by Administrations in section I-9 of their contribution to the Resolution 609 Forum at ITU website.

INSAT-NAVR-GS

The epfd calculations were carried out by applying software of India (Admin) and Russian Federation (Rus).

The calculation results are shown in table and in Figure. (see Table A.1 and Fig.A.1).

It should also be noted that the Administration of the Russian Federation has some questions to the Indian Administration regarding the orbital characteristics of the non-GSO constellation of the INSAT-NAVR-GS system

Table A.1 The maximum epfd distribution

| Latitude (deg.) | Max epfd, dB(W/m2)/MHz Admin | Max epfd, dB(W/m2)/MHz Rus | Latitude (deg.) | Max epfd, dB(W/m2)/MHz Admin | Max epfd, dB(W/m2)/MHz Rus | Latitude (deg.) | Max epfd, dB(W/m2)/MHz Admin | Max epfd, dB(W/m2)/MHz Rus | Latitude (deg.) | Max epfd, dB(W/m2)/MHz Admin | Max epfd, dB(W/m2)/MHz Rus |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -90 | -138,53 | -136,13 | -44 | -129,18 | -129,47 | 2 | -128,25 | -128,38 | 48 | -127,96 | -128,14 |
| -89 | -138,36 | -136,24 | -43 | -129,14 | -129,51 | 3 | -128,23 | -128,37 | 49 | -127,94 | -128,05 |
| -88 | -138,22 | -136,13 | -42 | -129,1 | -129,53 | 4 | -128,24 | -128,36 | 50 | -127,87 | -127,96 |
| -87 | -137,87 | -136,14 | -41 | -129,07 | -129,55 | 5 | -128,22 | -128,35 | 51 | -127,85 | -127,88 |
| -86 | -137,66 | -136,15 | -40 | -129,03 | -129,56 | 6 | -128,2 | -128,36 | 52 | -127,81 | -127,81 |
| -85 | -135,76 | -136,09 | -39 | -128,92 | -129,56 | 7 | -128,17 | -128,36 | 53 | -127,77 | -127,76 |
| -84 | -135,48 | -135,1 | -38 | -128,87 | -129,55 | 8 | -128,15 | -128,36 | 54 | -127,7 | -127,7 |
| -83 | -135,26 | -134,99 | -37 | -128,86 | -129,55 | 9 | -128,13 | -128,36 | 55 | -127,66 | -128,44 |
| -82 | -135,1 | -134,8 | -36 | -128,85 | -129,46 | 10 | -128,16 | -128,37 | 56 | -128,1 | -128,41 |
| -81 | -134,93 | -134,25 | -35 | -128,77 | -129,46 | 11 | -128,14 | -128,36 | 57 | -128,79 | -128,59 |
| -80 | -134,81 | -134,14 | -34 | -128,75 | -129,45 | 12 | -128,12 | -128,36 | 58 | -128,83 | -129,06 |
| -79 | -134,67 | -134,03 | -33 | -128,68 | -129,43 | 13 | -128,09 | -128,36 | 59 | -129,35 | -129,35 |
| -78 | -134,55 | -133,92 | -32 | -128,64 | -129,41 | 14 | -128,07 | -128,35 | 60 | -129,88 | -129,57 |
| -77 | -134,45 | -133,77 | -31 | -128,62 | -129,39 | 15 | -128,05 | -128,37 | 61 | -130,15 | -129,88 |
| -76 | -134,33 | -133,6 | -30 | -128,6 | -129,36 | 16 | -128,03 | -128,37 | 62 | -130,44 | -130,03 |
| -75 | -134,19 | -133,41 | -29 | -128,57 | -129,32 | 17 | -128,01 | -128,39 | 63 | -130,6 | -130,16 |
| -74 | -134,09 | -133,23 | -28 | -128,55 | -129,3 | 18 | -128,01 | -128,39 | 64 | -130,77 | -130,21 |
| -73 | -133,85 | -133,09 | -27 | -128,53 | -129,27 | 19 | -127,99 | -128,4 | 65 | -130,89 | -130,33 |
| -72 | -133,65 | -132,94 | -26 | -128,51 | -129,24 | 20 | -127,98 | -128,5 | 66 | -130,98 | -130,44 |
| -71 | -133,47 | -132,78 | -25 | -128,49 | -129,21 | 21 | -127,95 | -128,51 | 67 | -131,04 | -130,55 |
| -70 | -132,64 | -132,54 | -24 | -128,47 | -129,17 | 22 | -127,93 | -128,52 | 68 | -131,08 | -130,66 |
| -69 | -132,45 | -132,44 | -23 | -128,46 | -129,15 | 23 | -127,91 | -128,53 | 69 | -131,1 | -130,82 |
| -68 | -132,43 | -132,27 | -22 | -128,46 | -129,11 | 24 | -127,89 | -128,52 | 70 | -131,25 | -130,9 |
| -67 | -132,39 | -132,15 | -21 | -128,46 | -129,08 | 25 | -127,89 | -128,53 | 71 | -131,88 | -131,06 |
| -66 | -132,34 | -132,02 | -20 | -128,45 | -129,04 | 26 | -127,9 | -128,54 | 72 | -131,98 | -131,2 |
| -65 | -132,25 | -131,89 | -19 | -128,45 | -128,93 | 27 | -127,9 | -128,55 | 73 | -132,14 | -131,34 |
| -64 | -132,15 | -131,75 | -18 | -128,44 | -128,89 | 28 | -127,9 | -128,57 | 74 | -132,23 | -131,45 |
| -63 | -132 | -131,61 | -17 | -128,43 | -128,86 | 29 | -127,9 | -128,57 | 75 | -132,31 | -131,6 |
| -62 | -131,84 | -131,52 | -16 | -128,43 | -128,82 | 30 | -127,9 | -128,58 | 76 | -132,43 | -131,77 |
| -61 | -131,56 | -131,29 | -15 | -128,43 | -128,79 | 31 | -127,9 | -128,59 | 77 | -132,54 | -131,92 |
| -60 | -131,3 | -131 | -14 | -128,42 | -128,75 | 32 | -127,9 | -128,58 | 78 | -132,64 | -132,07 |
| -59 | -130,67 | -130,77 | -13 | -128,42 | -128,73 | 33 | -127,92 | -128,58 | 79 | -132,76 | -132,29 |
| -58 | -130,08 | -130,36 | -12 | -128,43 | -128,7 | 34 | -127,97 | -128,57 | 80 | -132,89 | -132,49 |
| -57 | -130,04 | -129,85 | -11 | -128,42 | -128,67 | 35 | -127,96 | -128,57 | 81 | -132,98 | -132,61 |
| -56 | -129,03 | -129,64 | -10 | -128,42 | -128,65 | 36 | -128,04 | -128,54 | 82 | -133,13 | -132,79 |
| -55 | -128,77 | -129,58 | -9 | -128,36 | -128,62 | 37 | -128,03 | -128,61 | 83 | -133,27 | -132,95 |
| -54 | -128,81 | -128,85 | -8 | -128,36 | -128,6 | 38 | -128,02 | -128,59 | 84 | -133,46 | -133,12 |
| -53 | -128,86 | -128,79 | -7 | -128,35 | -128,56 | 39 | -128,04 | -128,57 | 85 | -133,67 | -134,69 |
| -52 | -128,89 | -128,88 | -6 | -128,35 | -128,54 | 40 | -128,13 | -128,55 | 86 | -135,4 | -134,81 |
| -51 | -128,93 | -128,99 | -5 | -128,34 | -128,5 | 41 | -128,15 | -128,52 | 87 | -135,57 | -134,91 |
| -50 | -128,94 | -129,11 | -4 | -128,34 | -128,47 | 42 | -128,16 | -128,49 | 88 | -135,87 | -135,02 |
| -49 | -128,98 | -129,21 | -3 | -128,3 | -128,46 | 43 | -128,18 | -128,45 | 89 | -136 | -135,35 |
| -48 | -128,99 | -129,28 | -2 | -128,29 | -128,44 | 44 | -128,21 | -128,4 | 90 | -136,16 | -135,38 |
| -47 | -129,06 | -129,36 | -1 | -128,3 | -128,42 | 45 | -128,16 | -128,34 | - | - | - |
| -46 | -129,1 | -129,43 | 0 | -128,31 | -128,39 | 46 | -128,11 | -128,32 | - | - | - |
| -45 | -129,14 | -129,43 | 1 | -128,28 | -128,39 | 47 | -128,05 | -128,24 | - | - | - |

Fig. A.1. The maximum epfd distribution by latitude

The maximum epfd matrix (step 1 degree) obtained by the administration of Russian Federation is presented in the file «INSAT-NAVR-GS.xlsx».

Thus the maximum epfd levels for the INSAT-NAVR-GS system obtained with the software of India and Russian Federation differ by 2.4  dB in the worst case.

Conclusion

Based on the analysis of the results shown in Section 2 of appendix 1 it could be made conclusion, that maximum difference of epfd values by latitude submitted by Administrations and obtained with the software used by the administration of Russian Federation, is 2.4 dB.

However, it should also be noted that there are a number of questions related to the orbital positions of the spacecrafts in the non-GSO constellation of the INSAT-NAVR-GS system, which could affect the correctness of the result obtained. In this regard, it is necessary to clarify the initial data of the INSAT-NAV-GS system.