# IRNSS (India)

## I-1	RNSS ITU publication reference

| RNSS network name | BR network ID | ITU publication reference | IFIC No |
| --- | --- | --- | --- |
| INSAT-NAVR(32.5) | 111540556 | API/A/7044 | 2705 |
| INSAT-NAVR(32.5) | 112520044 | CR/C/3122 | 2724 |
| INSAT-NAVR(32.5) | 312520108/112520044 | CR/C/3122 M-1 | 2728 |
| INSAT-NAVR(83) | 111540557 | API/A/7045 | 2705 |
| INSAT-NAVR(83) | 112520045 | CR/C/3123 | 2724 |
| INSAT-NAVR(83) | 312520109/112520045 | CR/C/3123 M-1 | 2728 |
| INSAT-NAVR(83) | 115500195 | Part I-S | 2817 |
| INSAT-NAVR(120.5)1 | 111540558 | API/A/7046 | 2705 |
| INSAT-NAVR(120.5)1 | 112520046 | CR/C/3124 | 2724 |
| INSAT-NAVR(120.5)1 | 312520110/112520046 | CR/C/3124 M-1 | 2728 |
| INSAT-NAVR(121.5)1 | 111540559 | API/A/7047 | 2705 |
| INSAT-NAVR(121.5)1 | 112520047 | CR/C/3125 | 2724 |
| INSAT-NAVR(121.5)1 | 312520111/112520047 | CR/C/3125 M-1 | 2728 |
| INSAT-NAVR(123.5)1 | 111540560 | API/A/7048 | 2705 |
| INSAT-NAVR(123.5)1 | 112520048 | CR/C/3126 | 2724 |
| INSAT-NAVR(123.5)1 | 312520112/112520048 | CR/C/3126 M-1 | 2728 |
| INSAT-NAVR(126.5)1 | 111540563 | API/A/7051 | 2705 |
| INSAT-NAVR(126.5)1 | 112520049 | CR/C/3127 | 2724 |
| INSAT-NAVR(126.5)1 | 312520113/112520049 | CR/C/3127 M-1 | 2728 |
| INSAT-NAVR(127.5)1 | 111540564 | API/A/7052 | 2705 |
| INSAT-NAVR(127.5)1 | 112520050 | CR/C/3128 | 2724 |
| INSAT-NAVR(127.5)1 | 312520114/112520050 | CR/C/3128 M-1 | 2728 |
| INSAT-NAVR(129.5)1 | 111540566 | API/A/7054 | 2705 |
| INSAT-NAVR(129.5)1 | 112520051 | CR/C/3129 | 2724 |
| INSAT-NAVR(129.5)1 | 312520115/112520051 | CR/C/3129 M-1 | 2728 |
| INSAT-NAV-A-GS2 | 107540528 | API/A/4720 | 2603 |
| INSAT-NAV-A-GS2 | 108520024 | CR/C/2160 | 2621 |
| INSAT-NAV-A-GS2 | 309520155/108520024 | CR/C/2160 M-1 | 2657 |
| INSAT-NAV-A-GS2 | 309540693/107540528 | API/A/4720 M-1 | 2658 |
| INSAT-NAV-A-GS2 | 112500050 | PART I-S | 2721 |
| INSAT-NAV-A-GS2 | 112500050 | PART III-S | 2733 |
| INSAT-NAV-A-GS2 | 112500050 | PART II-S | 2744 |
| INSAT-NAVR-GS2 | 111540555 | API/A/7043 | 2709 |
| INSAT-NAVR-GS2 | 112520052 | CR/C/3121 | 2724 |
| INSAT-NAVR-GS2 | 312520116/112520052 | CR/C/3121 M-1 | 2728 |
| INSAT-NAV-GS17 | 117520274 | API/C/224 | 2856 |
| INSAT-NAV-GS17 | 117520274 | CR/C/4544 | 2866 |

- Among the satellite networks between the orbital slot 120.5⁰E and 129.5⁰E, the orbital slot 129.5⁰E has been finalised and the satellite has been launched. Hence the data submitted for 129.5 to be used for aepfd calculation.

- INSAT-NAVR-GS, covers the four IRNSS geo-synchronous satellites in two orbital planes with an inclination of 29⁰ and equatorial crossings at 55⁰E for two satellites and 111.75⁰E for other two satellites.

- INSAT-NAV-GS17, covers the four IRNSS geo-synchronous satellites in two orbital planes with an inclination of 29⁰ and equatorial crossings at 32.5⁰E for two satellites and 129.5⁰E for other two satellites.

- In accordance with § 5 of the Resolution 609 Consultation Meeting Terms of Reference, the publication references for these two satellite networks can be treated as single system.

## I-2	Non-GSO satellite system constellation parameters

N: 	8 number of space stations of the non-GSO system

K:	4 number of orbital planes

h:	35786kms satellite altitude above the Earth at apogee (km)

hP:	35786kmssatellite altitude above the Earth at perigee(km), for non-GSO systems with elliptical orbits

I:	29⁰ inclination

| Satellite index I | RAAN i,0(degrees) |  | Argument of latitude ui,0(degrees) | Argument of perigee of the non-GSO system2 i,0 (degrees) |
| --- | --- | --- | --- | --- |
| 1 | 129.45 |  | 110.58 | 0 |
| 2 | 129.45 |  | 167.33 | 0 |
| 3 | 306.980 |  | 293.05 | 0 |
| 4 | 306.980 |  | 349.80 | 0 |
| 5 | 148.53 |  | 69.0 | 0 |
| 6 | 317.78 |  | 259.75 | 0 |
| 7 | 288.28 |  | 25.75 | 0 |
| 8 | 104.28 |  | 209.75 | 0 |
| Is the system operational? (Yes or No) 1 |  | No So far four NGSO satellites have been launched |  |  |  |
| 1 An operational system is one that has frequency assignments in the 1164-1215 MHz band that have been brought into use or that are anticipated to be brought into use within 18 months after the start of the tentatively-scheduled Consultation Meeting. See §§11 b), 12b) and 15 of the Consultation Meeting Terms of Reference for details. 2 Applicable to non-GSO satellites with elliptical orbits. |  |  |  |  |  |

## I-3	GSO satellites

## I-3A	GSO satellite system longitude

Lon GSO:-	32.5⁰E, 83⁰E, 129.5⁰E

## I-3B	GSO satellite system brought into use status

Satellite under INSAT-NAVR(83) was launched on 10 November 2014

Satellite under INSAT-NAVR(32.5) was launched on 10 March 2016

Satellite under INSAT-NAVR(129.5) was launched on April 28, 2016

## I-4	Maximum circular orbit non-GSO space station pfd versus the elevation angle at 40,000 feet above the Earth’s surface (worst 1 MHz)

All the non-GSO satellites of IRNSS have steerable beam. See sec I-6.

## I-5	GSO space station maximum pfd versus latitude and longitude at 40,000 feet above the Earth’s surface (worst 1 MHz)

There is no change in the pfd data submitted for INSAT-NAVR(32.5), INSAT-NAVR(83) and INSAT-NAVR(129.5) during the 14th consultation meeting.

## I-6	Maximum off-axis EIRP density for non-GSO space stations with steerable beams, and antenna pointing information (worst 1MHz)

For non-GSO systems with steerable beams, the following information is needed in order to calculate the epfd matrix:

- Satellite antenna pointing direction and other operational information to be used in calculating epfd values. The beams of all 8 NGSO satellites of IRNSS constellation will always look towards 83⁰E Longitude and 5⁰N Latitude.

- Off-axis EIRP density:

| Off-axis Angle (each 1°) | EIRP density (dBW in worst 1MHz) |
| --- | --- |
| 0 | 32.74 |
| 1 | 32.64 |
| 2 | 32.45 |
| 3 | 32.17 |
| 4 | 31.78 |
| 5 | 31.29 |
| 6 | 30.68 |
| 7 | 29.94 |
| 8 | 29.04 |
| 9 | 28 |
| 10 | 26.77 |
| 11 | 25.35 |
| 12 | 23.59 |
| 13 | 21.63 |
| 14 | 19.21 |

## I-7	Frequency/Satellite Matrix

All the 8 non-GSO satellites of IRNSS constellation will transmit the same signals at the centre frequency 1176.45 MHz.

## I-8	GSO/non-GSO satellite system spectrum

The level of spectrum emission in each 1 MHz relative to the spectrum value at the worst 1 MHz of the whole band (1 164-1 215 MHz) is provided below as the Spectral Adjustment Factor (SAF) in dB.

Normalised Signal Power Spectrum

| Center Frequency (MHz) | SAF (dB) | Center Frequency (MHz) | SAF (dB) | Center Frequency (MHz) | SAF (dB) | Center Frequency (MHz) | SAF (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -31.33 | 1177 | -10.95 | 1190 | -19.67 | 1203 | -40.45 |
| 1165 | -31.63 | 1178 | -14.39 | 1191 | -15.72 | 1204 | -38.82 |
| 1166 | -28.68 | 1179 | -17.88 | 1192 | -18.61 | 1205 | -46.63 |
| 1167 | -28.36 | 1180 | -8.19 | 1193 | -29.98 | 1206 | -43.17 |
| 1168 | -22.05 | 1181 | -5.96 | 1194 | -26.82 | 1207 | -50.74 |
| 1169 | -22.11 | 1182 | -10.3 | 1195 | -32.76 | 1208 | -47.71 |
| 1170 | -9.34 | 1183 | -22.99 | 1196 | -28.83 | 1209 | -52.71 |
| 1171 | -5.55 | 1184 | -21.87 | 1197 | -33.5 | 1210 | -44.26 |
| 1172 | -8.35 | 1185 | -28.26 | 1198 | -30.83 | 1211 | -42.3 |
| 1173 | -17.64 | 1186 | -28.14 | 1199 | -34.35 | 1212 | -46.13 |
| 1174 | -13.83 | 1187 | -31.49 | 1200 | -25.28 | 1213 | -56.37 |
| 1175 | -8.82 | 1188 | -31.39 | 1201 | -23.02 | 1214 | -54.15 |
| 1176 | 0 | 1189 | -31.14 | 1202 | -27.37 | 1215 | -58.77 |

## I-9  Results of the epfd calculation for an individual system in the worst 1 MHz of the 1 164-1 215 MHz band

Additional simulation data for non-GSO systems only:

| Time step (sec) | 60 |
| --- | --- |
| Number of time steps | 1440 |

Technical Contact Person

Name: Mr. S. Sayeenathan

Tel.: +91 80 2217 2309

Fax: +91 80 2351 8884

E-mail Address: sayee@isro.gov.in

Name: Mr. P.V.Kumaramohan

Tel.: +91 80 2217 2382

Fax: +91 80 2351 8884

E-mail Address: kumaramohan@isro.gov.in

Name: Mrs.Richa Chakravarty

Tel.: +91 80 2217 2320

Fax: +91 80 2351 8884

E-mail Address: richa@isro.gov.in