| Fifteenth Resolution 609 (Rev. WRC-07) Consultation Meeting | Document |
| --- | --- |
|  | 5 July 2018 |
|  | English only |

FRANCE

Galileo maximum aggregate epfd calculations for 
Fifteenth Resolution 609 (Rev. WRC-07) Consultation Meeting

This contribution presents the results of aggregate epfd calculations performed by Galileo in preparation to the Fifteenth Resolution 609 Consultation Meeting.

I Systems taken into account in the aggregate epfd calculation

From the analysis of input contributions to the Fifteenth Consultation Meeting posted on the Res609 Forum web site, Galileo notes that the novelty compared to previous meeting is on the NGSO component of the IRNSS system.

As an answer to the action mentioned in §7 of the “Record of Decisions Taken at the Fourteenth Resolution 609 (Rev. WRC-07) Consultation Meeting”, India provides an updated epfd matrix for INSAT-NAVR-GS/INSAT-NAV-GS17 satellite networks.

Table 1 below contains the list of systems that are considered by Galileo in the framework of the aggregate epfd calculations performed in preparation to the Fifteenth Resolution 609 Consultation Meeting.

| N° | RNSS system name | Administration | Orbital Position | Note |
| --- | --- | --- | --- | --- |
| 1 | MSATNAV-2 | F/GLS | NGSO | Confirmed |
| 2 | NAVSTAR GPS | USA | NGSO | Confirmed |
| 3 | GLONASS-M FDMA | RUS | NGSO | Confirmed |
| 4 | GLONASS-M CDMA | RUS | NGSO | Confirmed |
| 5 | COMPASS- MEO Regional signal | CHN | NGSO | Confirmed |
| 6 | COMPASS-MEO Global signal | CHN | NGSO | Confirmed |
| 7 | COMPASS-IGSO Regional signal | CHN | NGSO | Confirmed |
| 8 | COMPASS-IGSO Global signal | CHN | NGSO | Confirmed |
| 9 | N-SAT-HEO2/QZSS-1 | J | NGSO | Confirmed |
| 10 | INSAT-NAVR-GS/INSAT-NAV-GS17 | IND | NGSO | Updated |
| 11 | LM-RPS-107.3W | USA | -107.3 | Confirmed |
| 12 | LM-RPS-133W | USA | -133 | Confirmed |
| 13 | INMARSAT-4 143.5E/-4A 143.5E | G | 143.5 | Confirmed |
| 14 | INMARSAT-4 98W/-4A 98W | G | -98 | Confirmed |
| 15 | INMARSAT-GSO-2N/-4B 64E | G | 64 | Confirmed |
| 16 | COMPASS-58.75E Regional signal | CHN | 58.75 | Confirmed |
| 17 | COMPASS-58.75E Global signal | CHN | 58.75 | Confirmed |
| 18 | COMPASS-80E Regional signal | CHN | 80 | Confirmed |
| 19 | COMPASS-80E Global signal | CHN | 80 | Confirmed |
| 20 | COMPASS-110.5E Regional signal | CHN | 110.5 | Confirmed |
| 21 | COMPASS-110.5E Global signal | CHN | 110.5 | Confirmed |
| 22 | COMPASS-140E Regional signal | CHN | 140 | Confirmed |
| 23 | COMPASS-140E Global signal | CHN | 140 | Confirmed |
| 24 | COMPASS-160E Regional signal | CHN | 160 | Confirmed |
| 25 | COMPASS-160E Global signal | CHN | 160 | Confirmed |
| 26 | COMPASS-B-84E Regional signal | CHN | 84 | Confirmed |
| 27 | COMPASS-B-144.5E Regional signal | CHN | 144.5 | Confirmed |
| 28 | INSAT-NAVR(32.5) (IRNSS) | IND | 32.5 | Confirmed |
| 29 | INSAT-NAVR(83) (IRNSS) | IND | 83 | Confirmed |
| 30 | INSAT-NAVR(129.5) (IRNSS) | IND | 129.5 | Confirmed |
| 31 | INSAT-NAV(55) (GAGAN) | IND | 55 | Confirmed |
| 32 | INSAT-NAVR(83) (GAGAN) | IND | 83 | Confirmed |
| 33 | LUX-G6-2-E | LUX | 5 | Confirmed |
| 34 | LUX-G7-9-E2 | LUX | 31.5E | Confirmed |
| 35 | QZSS-GS4 | J | 127 | Confirmed |
| 36 | QZSS-GS3 | J | 123 | Confirmed |
| 37 | RAGGIANA-18 | PNG | -117 | Confirmed |
| 38 | ALCOMSAT-24.8W | ALG | -24.8 | Confirmed |
| 39 | NIGCOMSAT-1R | NIG | 42.5 | Confirmed |
| 40 | LUX-G9-38-A | LUX | -129 | Confirmed |

Table 1 - RNSS systems considered at the 15th Consultation Meeting

II Results of maximum aggregate epfd calculations

Galileo has carried out the calculation for the maximum aggregate epfd considering all networks listed in Table 1.

Results are given in Table 2 and in Figure 1. The maximum aggregate epfd in the band 1164-1215 MHz is –121.898 dB(W/m2·MHz), i.e. 0.398 dB below the Resolution 609 limit of –121.5 dB(W/m2·MHz).

| Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -139.162 | 1177 | -123.129 | 1190 | -135.682 | 1203 | -125.679 |
| 1165 | -140.405 | 1178 | -123.557 | 1191 | -134.781 | 1204 | -124.795 |
| 1166 | -140.073 | 1179 | -124.390 | 1192 | -134.820 | 1205 | -124.481 |
| 1167 | -139.132 | 1180 | -124.746 | 1193 | -135.625 | 1206 | -123.680 |
| 1168 | -136.157 | 1181 | -125.843 | 1194 | -136.477 | 1207 | -122.540 |
| 1169 | -132.851 | 1182 | -127.652 | 1195 | -138.016 | 1208 | -123.594 |
| 1170 | -129.365 | 1183 | -130.137 | 1196 | -138.565 | 1209 | -125.300 |
| 1171 | -126.915 | 1184 | -132.861 | 1197 | -137.684 | 1210 | -125.896 |
| 1172 | -126.161 | 1185 | -136.205 | 1198 | -135.364 | 1211 | -127.028 |
| 1173 | -124.924 | 1186 | -139.003 | 1199 | -132.933 | 1212 | -128.301 |
| 1174 | -124.276 | 1187 | -141.297 | 1200 | -130.399 | 1213 | -130.161 |
| 1175 | -123.396 | 1188 | -139.853 | 1201 | -128.409 | 1214 | -132.442 |
| 1176 | -121.898 | 1189 | -137.644 | 1202 | -126.773 | 1215 | -135.705 |

Table 2 - Aggregate epfd results

Figure 1 - Plot of aggregate epfd results

Galileo looks forward to further discussing this result during the meeting.

______________________________

France acts on behalf of all Galileo Administrations (GLS Administrations).