# UNITED STATES OF AMERICA

# Calculation Results for Aggregate System EPFD for the Fifteenth Resolution 609 (Rev. WRC-07) Consultation Meeting

# 1 Introduction

The purpose of this contribution is to provide and share the results of equivalent power flux density (epfd) calculations performed by the U.S. in preparation for the Fifteenth Resolution 609 Consultation Meeting (15th CM) to be held in Abuja, Nigeria, 2-4 October 2018. The following relevant materials are presented in this document:

- An Executive Summary is presented in Section 2.

- The U.S. calculation of the maximum RNSS aggregate epfd, according to § 11 d) of the CM Terms of Reference (MOD Geneva, September 2011) (ToR) is provided in Section 3.

- In Section 4, the single-system epfd results, obtained using the U.S. Resolution 609 calculation tool, are compared to other operator/administration results.

- Some outstanding topics to be discussed at the next CM is proposed in Section 5.

# 2 Executive Summary

## 2.1 Maximum Aggregate EPFD Results

For the maximum aggregate epfd results provided in this report, the single-system epfd results provided most recently by the notifying administrations were used. Based on this assumption, the maximum aggregate epfd value occurs at 1 175 MHz and is -121.90 dB(W/(m2∙MHz)), 0.40 dB below the Resolution 609 limit of -121.5 dB(W/(m2∙MHz)). The U.S. believes this value is the correct maximum aggregate epfd value to report out of the 15th CM. (See Section 3.1 below for more details.)

## 2.2 Fifteenth Consultation Meeting Single-System EPFD Results

India provided updated IRNSS system information and the corresponding single-system epfd results. The U.S. calculated results are in close agreement with India’s results (see Section 4.1).

# 3 Maximum Aggregate EPFD Results

## 3.1 RNSS Systems Taken into Account in the Aggregate EPFD Calculations

The list of RNSS systems considered for the maximum aggregate epfd calculation is shown in Table 1.

Table 1: RNSS Systems and aggregate epfd Calculation Sets

| System # | RNSS System Name | Administration | Input Data Source |
| --- | --- | --- | --- |
| 1 | MSATNAV-2 | F/GLS | 8th CM |
| 2 | NAVSTAR GPS | USA | 8th CM |
| 3 | GLONASS-M (FDMA) | Russia | 8th CM |
| 4 | GLONASS-M (CDMA) | Russia | 11th CM |
| 5 (Note 1) | COMPASS-MEO (Regional) | China | 11th CM |
| 6 (Note 2) | COMPASS MEO (Global) | China | 11th CM |
| 7 (Note 1) | COMPASS IGSO (Regional) | China | 11th CM |
| 8 (Note 2) | COMPASS IGSO (Global) | China | 11th CM |
| 9 | QZSS | Japan | 10th CM |
| 10 | QZSS GS3 (123E) | Japan | 10th CM |
| 11 | QZSS GS4 (127E) | Japan | 10th CM |
| 12 | LM-RPS-133W | USA | 8th CM |
| 13 | LM-RPS-107.3W | USA | 8th CM |
| 14 | INMARSAT GSO-2N (64E) | Great Britain | 11th CM |
| 15 | INMARSAT-4 143.5E/-4A 143.5E | Great Britain | 8th CM |
| 16 | INMARSAT-4 98W/-4A 98W | Great Britain | 8th CM |
| 17-23 (Note 1) | COMPASS GSO (Regional) | China | 11th CM |
| 24-28 (Note 2) | COMPASS GSO (Global) | China | 11th CM |
| 29 | INSAT-NAV(55) | India | 10th CM |
| 30 | INSAT-NAVR(83) | India | 10th CM |
| 31 | INSAT-NAVR(32.5) | India | 10th CM |
| 32 | INSAT-NAVR(83) | India | 10th CM |
| 33 | INSAT-NAV(131.5) | India | 10th CM |
| 34 | INSAT-NAVR-GS + INSAT-NAV-GS17 | India | 15th CM |
| 35 | NIGCOMSAT-1R | Nigeria | 8th CM |
| 36 | LUX-G6-2-E | Luxembourg | 8th CM |
| 37 | LUX-G7-9-E2 | Luxembourg | 9th CM |
| 38 | ALCOMSAT-24.8W | Algeria | 11th CM |
| 39 | RAGGIANA-18 | PNG | 11th CM |
| 40 | LUX-G9-38 | Luxembourg | 13th CM |

Note 1: The space constellation of the COMPASS regional system includes at most 18 satellites: 7 IGSOs, 4 MEOs and 7 GSOs (COMPASS-58.75E/80E/110.5E/140E/160E/B-84E/B-144.5E).

Note 2: The space constellation of the COMPASS global system consists of 35 satellites: 3 IGSOs, 27 MEOs and 5 GSOs ((COMPASS-58.75E/80E/110.5E/140E/160E).

## 3.2 Maximum Aggregate EPFD results using U.S. Tool

The maximum aggregate epfd results are summarized in Table 2 and Figure 1. The maximum aggregate epfd is -121.90 dB(W/(m2∙MHz)), which occurs at 1 176 MHz. The remaining margin before exceeding the Resolution 609 limit of -121.5 dB(W/(m2∙MHz)) is 0.40 dB.

Table 2: Maximum aggregate epfd calculation results

Figure 1: Maximum aggregate epfd results. The maximum aggregate epfd value calculated using the U.S. tool is -121.90 dB(W/(m2∙MHz)) and occurs at 1 176 MHz.

# 4 Fifteenth CM Single-System EPFD Results

## 4.1 INSAT-NAVR-GS + INSAT-NAV-GS17

India provided updated information on their IRNSS non-GSO system for the 15th CM. India indicated that the INSAT-NAV-NGSA filing expired on 12 August 2017. This filing had covered the four IRNSS geo-synchronous satellites in four orbital planes with an inclination of 42⁰ and equatorial crossings at 46.5⁰E, 65.5⁰E, 94.5⁰E and 119.75⁰E, respectively. India indicated that a new filing, INSAT-NAV-GS17 (API/C/224, IFIC2856, CR/C/4544 IFIC2866), covers four IRNSS geo-synchronous satellites in two orbital planes with an inclination of 29⁰ and equatorial crossings at 32.5⁰ E and 129.5⁰ E.

India indicated no changes to the four IRNSS geo-synchronous satellites covered under the  INSAT-NAVR-GS filing. These four satellites are in two orbital planes with an inclination of 29⁰. Two of the four satellites are in one orbital plane with equatorial crossings at 55⁰E, while the other two satellites are in a second orbital plane with 111.75⁰ E equatorial crossings. Together, these two filings cover eight geo-synchronous satellites relevant for the work of the CM.

The epfd results from India and the U.S. for these eight satellites are shown in Figure 2 and Figure 3, respectively. The maximum epfd values as a function of latitude using both tools are shown in Figure 4 which illustrates that the IRNSS results obtained by India and the U.S. are in very good agreement.

Figure 2: INSAT-NAVR-GS + INSAT-NAV-GS17 epfd data provided by India

Figure 3: INSAT-NAVR-GS + INSAT-NAV-GS17 epfd data from 12th CM

Figure 4: INSAT-NAVR-GS + INSAT-NAV-GS17 maximum epfd data along latitudes

# 5 Discussion Topics

5.1 Providing the Epoch Time for Non-GSO systems

Specifying the longitude of the equatorial crossings allows proper calculation of the orbit, but each administration would have to do additional calculations to determine the epoch times that result in the corresponding longitudinal crossings.  For the IRNSS epfd calculations, the U.S. tool required shifting the RAAN by 176° east so that the equatorial crossings of these satellites are correct. The values of RAAN and argument of latitude are specified at some time “t” (orbit epoch time).  The orbit epoch time is needed to allow proper modeling of the orbit. The importance of providing this information was discussed at the 10th CM. The CM should discuss this and other possible options to ensure consistency in simulation results.

5.2 Providing EPFD Data in Correct MS Excel Format

The U.S. notes that the MS Excel spreadsheet submitted by India containing IRNSS epfd data was not in the correct format. The proper MS Excel format is defined in Attachment 2 of the Resolution 609 Record of Decisions. The U.S. encourages administrations to review this attachment and provide future epfd data in the correct MS Excel format.

# Attachments

U.S. calculation of INSAT-NAVR-GS + INSAT-NAV-GS17