### ATTACHMENT

List of RNSS system characteristics to be provided to the Consultation Meeting per Item 11 c) of the Terms of Reference

# I	RNSS systems characteristics

## I-1	RNSS ITU publication reference

| RNSS network name | BR network ID | ITU publication reference | IFIC |
| --- | --- | --- | --- |
| MEASAT-1C | 119520055 | CR/C 4974 | 2900 |

## I-2	Non-GSO satellite system constellation parameters

N/A

## I-3	GSO satellites

## I-3A	GSO satellite system longitude

| longitude (deg) | Network name |
| --- | --- |
| 91.5E | MEASAT-1C |

## I-3B	GSO satellite system brought into use status

It is anticipated to bring into use the satellite frequency assignments in the 1164-1215 MHz band for the GSO network within 18 months after the start of the tentatively-scheduled Consultation Meeting.

## I-4	Maximum circular orbit non-GSO space station (with non-steerable beams) pfd versus the elevation angle at 40,000 feet above the Earth’s surface (worst 1 MHz)

N/A

## I-5	GSO space station maximum pfd versus latitude and longitude at 40,000 feet above the Earth’s surface (worst 1 MHz)

Values for 1º lat/long spacing are included in the spreadsheet below.

## I-6	Maximum off-axis EIRP density for non-GSO space stations with steerable beams, and antenna pointing information (worst 1MHz)

N/A

## I-7	Frequency/Satellite Matrix

N/A

## I-8	GSO/non-GSO satellite network spectrum

The level of spectrum emission in each 1 MHz relative to the spectrum value at the worst 1 MHz of the whole band (1 164-1 215 MHz) is provided below as the Spectral Adjustment Factor (SAF) in dB.

Normalized Signal Power Spectrum

| Center Frequency (MHz) | SAF (dB) | Center Frequency (MHz) | SAF (dB) | Center Frequency (MHz) | SAF (dB) | Center Frequency (MHz) | SAF (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -17 | 1177 | -0,1 | 1190 | -13 | 1203 | -27,8 |
| 1165 | -21,6 | 1178 | -0,5 | 1191 | -15 | 1204 | -28,8 |
| 1166 | -28,68 | 1179 | -1,1 | 1192 | -16,9 | 1205 | -33,2 |
| 1167 | -17 | 1180 | -2 | 1193 | -19,4 | 1206 | -37,8 |
| 1168 | -12,8 | 1181 | -3,5 | 1194 | -20,4 | 1207 | -49,4 |
| 1169 | -8,8 | 1182 | -5,35 | 1195 | -25 | 1208 | -41,8 |
| 1170 | -6,1 | 1183 | -7,8 | 1196 | -28,83 | 1209 | -37,8 |
| 1171 | -4 | 1184 | -11,3 | 1197 | -33,5 | 1210 | -35 |
| 1172 | -2 | 1185 | -16,1 | 1198 | -29,05 | 1211 | -35,2 |
| 1173 | -1,4 | 1186 | -23,3 | 1199 | -26,67 | 1212 | -35 |
| 1174 | -0,7 | 1187 | -23 | 1200 | -25,28 | 1213 | -37 |
| 1175 | -0,16 | 1188 | -17 | 1201 | -23,02 | 1214 | -37,6 |
| 1176 | 0 | 1189 | -15 | 1202 | -25,6 | 1215 | -41,6 |

## I-9  Results of the epfd calculation for an individual system in the worst 1 MHz of the 1 164-1 215 MHz band

Values for 1º lat/long spacing are included in the spreadsheet below.

- Technical Contact Person

Name	:		Siti Kamaliah

Tel	:		+60 3 8213 2224

Fax	:		+60 3 8213 2255

E-mail Adresse :	kamaliah@measat.com

Name	:		Ghislain Sombret

E-mail Adresse :	ghislain.sombret@airbus.com