Aggregate epfd Calculations by COMPASS for the

Seventeenth Resolution 609 (Rev. WRC-07) Consultation Meeting

1. Introduction

According to the ToR of the Resolution 609 (Rev.WRC-07) Consultation Meeting (CM), administrations that have provided data to one Resolution 609 Consultation Meeting should exchange with each other, with a copy to the BR for information, the details of their calculations of the Aggregate epfd (Aepfd) produced by all such systems, with a view to finalizing the results at the Consultation Meeting.

This document presents the calculations of the aggregate epfd performed by COMPASS in preparation to the Seventeenth Resolution 609 Consultation Meeting to be held online on 6-8 October 2020. The maximum aggregate epfd level assessed by COMPASS remains below the Resolution 609 limit of -121.5 dBW/m2/MHz.

2. RNSS Systems Taken into Account in the Calculations

Through the analysis of the input contributions to the 17th Resolution 609 Consultation Meeting, we noted the following new or updated materials:

- China has introduced a new NGSO system CENTISPACE-1, with all required technical data.

- India requested that the GAGAN GSO network INSAT-NAV(93.5) at 93.5°E should henceforth be taken into account in the aggregate epfd calculation. The pfd and epfd data for INSAT-NAVR(93.5) submitted on the 10th Consultation Meeting should be used in the Aepfd calculation.

- Malaysia has updated its GSO network MEASAT-1C(91.5E), and provided the updated technical data.

- The U.S. stated that the GSO network LM-RPS 133W should be no longer taken into account in the Aepfd calculation and introduced a new GSO network USASAT-80C-1 at 125° W with all required technical data.

Table 1 below contains all the systems that are considered by COMPASS in the calculations of the aggregate epfd for the 17th Resolution 609 Consultation Meeting.

Table 1  Systems Taken into Account in the Aepfd Calculations

| No. | RNSS system name | Administration | Orbital Position | Note |
| --- | --- | --- | --- | --- |
| 1 | MSATNAV-2 | F/GLS | NGSO | NOC |
| 2 | NAVSTAR GPS | USA | NGSO | NOC |
| 3 | GLONASS-M FDMA | RUS | NGSO | NOC |
| 4 | GLONASS-M CDMA | RUS | NGSO | NOC |
| 5 | COMPASS- MEO Regional signal | CHN | NGSO | NOC |
| 6 | COMPASS-MEO Global signal | CHN | NGSO | NOC |
| 7 | COMPASS-IGSO Regional signal | CHN | NGSO | NOC |
| 8 | COMPASS-IGSO Global signal | CHN | NGSO | NOC |
| 9 | COMPASS-58.75E Regional signal | CHN | 58.75 | NOC |
| 10 | COMPASS-58.75E Global signal | CHN | 58.75 | NOC |
| 11 | COMPASS-80E Regional signal | CHN | 80 | NOC |
| 12 | COMPASS-80E Global signal | CHN | 80 | NOC |
| 13 | COMPASS-110.5E Regional signal | CHN | 110.5 | NOC |
| 14 | COMPASS-110.5E Global signal | CHN | 110.5 | NOC |
| 15 | COMPASS-140E Regional signal | CHN | 140 | NOC |
| 16 | COMPASS-140E Global signal | CHN | 140 | NOC |
| 17 | COMPASS-160E Regional signal | CHN | 160 | NOC |
| 18 | COMPASS-160E Global signal | CHN | 160 | NOC |
| 19 | COMPASS-B-84E Regional signal | CHN | 84 | NOC |
| 20 | COMPASS-B-144.5E Regional signal | CHN | 144.5 | NOC |
| 21 | N-SAT-HEO2/QZSS-1 | J | NGSO | NOC |
| 22 | LM-RPS-107.3W | USA | -107.3 | NOC |
|  | LM-RPS-133W | USA | -133 | Deleted |
| 23 | USASAT-80C-1 | USA | -125 | New |
| 24 | INMARSAT-4 143.5E/-4A 143.5E | G | 143.5 | NOC |
| 25 | INMARSAT-4 98W/-4A 98W | G | -98 | NOC |
| 26 | INMARSAT-GSO-2N/-4B 64E | G | 64 | NOC |
| 27 | INSAT-NAVR(32.5) (IRNSS) | IND | 32.5 | NOC |
| 28 | INSAT-NAVR(83) (IRNSS) | IND | 83 | NOC |
| 29 | INSAT-NAVR(129.5) (IRNSS) | IND | 129.5 | NOC |
| 30 | INSAT-NAV(55) (GAGAN) | IND | 55 | NOC |
| 31 | INSAT-NAV(83) (GAGAN) | IND | 83 | NOC |
| 32 | INSAT-NAVR-GS/INSAT-NAV-GS17 | IND | NGSO | NOC |
| 33 | INSAT-NAV(93.5) (GAGAN) | IND | 93.5 | New |
| 34 | LUX-G6-2-E | LUX | 5 | NOC |
| 35 | LUX-G7-9-E2 | LUX | 31.5 | NOC |
| 36 | LUX-G9-38-A | LUX | -129 | NOC |
| 37 | QZSS-GS4 | J | 127 | NOC |
| 38 | QZSS-GS-A1 | J | 90.5 | NOC |
| 39 | RAGGIANA-18 | PNG | -117 | NOC |
| 40 | ALCOMSAT-24.8W | ALG | -24.8 | NOC |
| 41 | NIGCOMSAT-1R | NIG | 42.5 | NOC |
| 42 | MEASAT-1C | MLA | 91.5 | Updated |
| 43 | F-SAT-RN-5W | F | -5 | NOC |
| 44 | CENTISPACE-1 | CHN | NGSO | New |

3. Maximum Aggregate epfd Calculation

For the maximum aggregate epfd calculation, the 1° epfd matrix as well as the Spectral Adjustment Factor provided by the corresponding operator or administration were used.

Figure 1 and Table 2 give the maximum aggregate epfd calculation results by COMPASS tool. The maximum aggregate epfd is -121.82 dBW/m2/MHz, i.e. 0.32 dB below the Resolution 609 limit of –121.5 dBW/m2/MHz. The maximum is reached at the 1176 MHz frequency.

Figure 1  Maximum Aepfd Results for CM17

Table 2  Maximum Aepfd Calculation Results for CM17

| Center Frequency (MHz) | Max Aepfd (dB(W/(㎡·MHz))) | Center Frequency (MHz) | Max Aepfd (dB(W/(㎡·MHz))) | Center Frequency (MHz) | Max Aepfd (dB(W/(㎡·MHz))) | Center Frequency (MHz) | Max Aepfd (dB(W/(㎡·MHz))) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -138.88 | 1177 | -123.11 | 1190 | -135.65 | 1203 | -125.68 |
| 1165 | -140.39 | 1178 | -123.49 | 1191 | -134.79 | 1204 | -124.79 |
| 1166 | -140.07 | 1179 | -124.32 | 1192 | -134.82 | 1205 | -124.48 |
| 1167 | -139.13 | 1180 | -124.75 | 1193 | -135.58 | 1206 | -123.68 |
| 1168 | -135.88 | 1181 | -125.78 | 1194 | -136.48 | 1207 | -122.54 |
| 1169 | -132.45 | 1182 | -127.61 | 1195 | -138.02 | 1208 | -123.59 |
| 1170 | -129.28 | 1183 | -129.94 | 1196 | -138.57 | 1209 | -125.3 |
| 1171 | -126.8 | 1184 | -132.5 | 1197 | -137.68 | 1210 | -125.9 |
| 1172 | -126.1 | 1185 | -135.94 | 1198 | -135.36 | 1211 | -127.03 |
| 1173 | -124.83 | 1186 | -139 | 1199 | -132.93 | 1212 | -128.3 |
| 1174 | -124.18 | 1187 | -141.29 | 1200 | -130.4 | 1213 | -130.16 |
| 1175 | -123.38 | 1188 | -139.68 | 1201 | -128.41 | 1214 | -132.44 |
| 1176 | -121.82 | 1189 | -137.45 | 1202 | -126.77 | 1215 | -135.7 |