| Seventeenth Resolution 609 (Rev. WRC-07) Consultation Meeting | Document |
| --- | --- |
|  | 27 July 2020 |
|  | English only |

FRANCE

Galileo maximum aggregate epfd calculations for 
Seventeenth Resolution 609 (Rev. WRC-07) Consultation Meeting

This contribution presents the results of single entry and aggregate epfd calculations performed by Galileo in preparation to the Seventeenth Resolution 609 Consultation Meeting.

I Systems taken into account in the aggregate epfd calculation

From the analysis of input contributions to the Seventeenth Consultation Meeting posted on the Res609 Forum web site, Galileo notes the following new or updated data:

- Malaysia has updated the data corresponding to its GSO network MEASAT-1C at 91.5°E,

- India has indicated that the GAGAN GSO network INSAT-NAV(93.5) at 93.5°E should henceforth be taken into account in the aggregate epfd calculation.

- China has introduced the new NGSO system CENTISPACE-1, composed of two satellites which transmit signals at different central frequencies, and provided the SAF and single-entry matrix for each of these satellites

- USA has indicated that

  - the GSO network LM-RPS-133W should no longer be taken into account in the aggregate epfd calculation,

  - the new GSO network USASAT-80C-1 at 125°W should be introduced in the aggregate epfd calculation.

Table 1 below contains the list of systems that are proposed to be considered in the framework of the aggregate epfd calculations performed in preparation to the Seventeenth Resolution 609 Consultation Meeting. This table includes the F-SAT-RN-5W network for which data had been considered during last Consultation Meeting, and have now been confirmed by France.

It is to be noted that the confirmation by Nigeria of parameters for NIGCOMSAT-1R was received after the deadline of 21 May 2020, as well as certain data for MEASAT-1C, CENTISPACE-1 and USASAT-80C-1. This may require discussions at the Seventeenth Consultation Meeting.

| N° | RNSS system name | Administration | Orbital Position | Note |
| --- | --- | --- | --- | --- |
| 1 | MSATNAV-2 | F/GLS | NGSO | Confirmed |
| 2 | NAVSTAR GPS | USA | NGSO | Confirmed |
| 3 | GLONASS-M FDMA | RUS | NGSO | Confirmed |
| 4 | GLONASS-M CDMA | RUS | NGSO | Confirmed |
| 5 | COMPASS- MEO Regional signal | CHN | NGSO | Confirmed |
| 6 | COMPASS-MEO Global signal | CHN | NGSO | Confirmed |
| 7 | COMPASS-IGSO Regional signal | CHN | NGSO | Confirmed |
| 8 | COMPASS-IGSO Global signal | CHN | NGSO | Confirmed |
| 9 | N-SAT-HEO2/QZSS-1 | J | NGSO | Confirmed |
| 10 | INSAT-NAVR-GS/INSAT-NAV-NGSA | IND | NGSO | Confirmed |
| 11 | LM-RPS-107.3W | USA | -107.3 | Confirmed |
|  | LM-RPS-133W | USA | -133 | Removed |
| 12 | USASAT-80C-1 | USA | -125 | New |
| 13 | INMARSAT-4 143.5E/-4A 143.5E | G | 143.5 | Confirmed |
| 14 | INMARSAT-4 98W/-4A 98W | G | -98 | Confirmed |
| 15 | INMARSAT-GSO-2N/-4B 64E | G | 64 | Confirmed |
| 16 | COMPASS-58.75E Regional signal | CHN | 58.75 | Confirmed |
| 17 | COMPASS-58.75E Global signal | CHN | 58.75 | Confirmed |
| 18 | COMPASS-80E Regional signal | CHN | 80 | Confirmed |
| 19 | COMPASS-80E Global signal | CHN | 80 | Confirmed |
| 20 | COMPASS-110.5E Regional signal | CHN | 110.5 | Confirmed |
| 21 | COMPASS-110.5E Global signal | CHN | 110.5 | Confirmed |
| 22 | COMPASS-140E Regional signal | CHN | 140 | Confirmed |
| 23 | COMPASS-140E Global signal | CHN | 140 | Confirmed |
| 24 | COMPASS-160E Regional signal | CHN | 160 | Confirmed |
| 25 | COMPASS-160E Global signal | CHN | 160 | Confirmed |
| 26 | COMPASS-B-84E Regional signal | CHN | 84 | Confirmed |
| 27 | COMPASS-B-144.5E Regiona signal l | CHN | 144.5 | Confirmed |
| 28 | INSAT-NAVR(32.5) (IRNSS) | IND | 32.5 | Confirmed |
| 29 | INSAT-NAVR(83) (IRNSS) | IND | 83 | Confirmed |
| 30 | INSAT-NAVR(129.5) (IRNSS) | IND | 129.5 | Confirmed |
| 31 | INSAT-NAV(55) (GAGAN) | IND | 55 | Confirmed |
| 32 | INSAT-NAVR(83) (GAGAN) | IND | 83 | Confirmed |
| 33 | LUX-G6-2-E | LUX | 5 | Confirmed |
| 34 | LUX-G7-9-E2 | LUX | 31.5E | Confirmed |
| 35 | QZSS-GS4 | J | 127 | Confirmed |
| 36 | QZSS-GS-A1 | J | 90.5 | Confirmed |
| 37 | RAGGIANA-18 | PNG | -117 | Confirmed |
| 38 | ALCOMSAT-24.8W | ALG | -24.8 | Confirmed |
| 39 | NIGCOMSAT-1R | NIG | 42.5 | Confirmed |
| 40 | LUX-G9-38-A | LUX | -129 | Confirmed |
| 41 | MEASAT-1C | MLA | 91.5 | Updated |
| 42 | F-SAT-RN-5W | F | -5 | Confirmed |
| 43 | INSAT-NAV(93.5) (GAGAN) | IND | 93.E | New |
| 44 | CENTISPACE-1 | CHN | NGSO | New |

Table 1 - RNSS systems considered at the 17th Consultation Meeting

II Comparison of the single system epfd results obtained through different software tools

Updated or new data have been provided as inputs for the Seventeenth Consultation Meeting. A comparison between the single system epfd matrices calculated with the Galileo tool and those obtained by corresponding administrations has been performed for USASAT-80C-1, MEASAT-1C, INSAT-NAV(93.5), and CENTISPACE-1.

Based on the pfd matrices provided by USA, Malaysia and India in their contributions to the Consultation Meeting, the single system epfd matrix has been calculated for these GSO networks through the Galileo software tool.

Similarly, the single entry epfd of each of the two satellites of the CENTISPACE-1 system has been calculated. This single entry calculation has to be performed separately for each satellite because they transmit at different central frequencies, and their signals correspond each to a specific Spectrum Adjustment Factor (SAF). It is to be noted that adding the single entry epfd of each of these satellites to the aggregate epfd is an overestimation of the contribution of the CENTISPACE-1 system to this aggregate, as it does not take into account the different in their orbit phase. However this overestimation is considered marginal for the derivation of the maximum aggregate epfd, because the single entry epfd level of CENTISPACE-1 is very low compared to that of other systems.

The attached excel file provides the results of the single epfd matrices calculated with the Galileo tool for USASAT-80C-1, MEASAT-1C, INSAT-NAV(93.5), and CENTISPACE-1.

The maximum difference between the single entry epfd provided by each administration and the one calculated by Galileo is reflected in the table below:

| Network name | Maximum difference in the single entry epfd |
| --- | --- |
| USASAT-80C-1 | 0.007 dB |
| MEASAT-1C | 0.010 dB |
| INSAT-NAV(91.5) | 0.011 dB |
| CENTISPACE-1 | 0.005 dB |

Table 2 – Maximum difference in the calculated single entry epfds

III Results of maximum aggregate epfd calculations

Galileo has carried out the calculation for the maximum aggregate epfd considering all networks listed in Table 1.

- First calculation is performed using the single entry epfd provided by each administration for its own network.

Results are given in Table 3. The maximum aggregate epfd in the band 1164-1215 MHz is -121.82 dB(W/m2·MHz), i.e. 0.32 dB below the Resolution 609 limit of –121.5 dB(W/m2·MHz).

| Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -138.88 | 1177 | -123.11 | 1190 | -135.65 | 1203 | -125.68 |
| 1165 | -140.39 | 1178 | -123.49 | 1191 | -134.79 | 1204 | -124.79 |
| 1166 | -140.07 | 1179 | -124.32 | 1192 | -134.82 | 1205 | -124.48 |
| 1167 | -139.13 | 1180 | -124.75 | 1193 | -135.58 | 1206 | -123.68 |
| 1168 | -135.88 | 1181 | -125.78 | 1194 | -136.48 | 1207 | -122.54 |
| 1169 | -132.45 | 1182 | -127.61 | 1195 | -138.02 | 1208 | -123.59 |
| 1170 | -129.28 | 1183 | -129.94 | 1196 | -138.57 | 1209 | -125.30 |
| 1171 | -126.80 | 1184 | -132.50 | 1197 | -137.68 | 1210 | -125.90 |
| 1172 | -126.10 | 1185 | -135.94 | 1198 | -135.36 | 1211 | -127.03 |
| 1173 | -124.83 | 1186 | -139.00 | 1199 | -132.93 | 1212 | -128.30 |
| 1174 | -124.18 | 1187 | -141.29 | 1200 | -130.40 | 1213 | -130.16 |
| 1175 | -123.38 | 1188 | -139.68 | 1201 | -128.41 | 1214 | -132.44 |
| 1176 | -121.82 | 1189 | -137.45 | 1202 | -126.77 | 1215 | -135.70 |

Table 3 - Aggregate epfd results with the single entry epfd provided 
by each administration for its own network

- Second calculation is performed using the single entry epfd calculated with the Galileo tool for each updated of new network.

Results are given in Table 4 and are slightly more favourable than in the first calculation. The maximum aggregate epfd in the band 1164-1215 MHz is -121.84 dB(W/m2·MHz), i.e. 0.34 dB below the Resolution 609 limit of –121.5 dB(W/m2·MHz).

| Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) | Center Frequency (MHz) | Max RNSS Agg epfd (dB(W/m2/MHz)) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -138.89 | 1177 | -123.16 | 1190 | -135.73 | 1203 | -125.68 |
| 1165 | -140.39 | 1178 | -123.58 | 1191 | -134.80 | 1204 | -124.79 |
| 1166 | -140.07 | 1179 | -124.39 | 1192 | -134.82 | 1205 | -124.48 |
| 1167 | -139.13 | 1180 | -124.77 | 1193 | -135.58 | 1206 | -123.68 |
| 1168 | -135.89 | 1181 | -125.80 | 1194 | -136.48 | 1207 | -122.54 |
| 1169 | -132.47 | 1182 | -127.63 | 1195 | -138.02 | 1208 | -123.59 |
| 1170 | -129.30 | 1183 | -129.95 | 1196 | -138.57 | 1209 | -125.30 |
| 1171 | -126.82 | 1184 | -132.51 | 1197 | -137.68 | 1210 | -125.90 |
| 1172 | -126.13 | 1185 | -135.95 | 1198 | -135.36 | 1211 | -127.03 |
| 1173 | -124.93 | 1186 | -139.00 | 1199 | -132.93 | 1212 | -128.30 |
| 1174 | -124.26 | 1187 | -141.29 | 1200 | -130.40 | 1213 | -130.16 |
| 1175 | -123.41 | 1188 | -139.70 | 1201 | -128.41 | 1214 | -132.44 |
| 1176 | -121.84 | 1189 | -137.48 | 1202 | -126.77 | 1215 | -135.70 |

Table 4 - Aggregate epfd results with the single entry epfd calculated 
with the Galileo tool for each updated of new network

A plot of these results is given in Figure 1. The difference between Table 1 and Table 2 results is not noticeable on this plot, since it does not exceed 0.02 dB

Figure 1 - Plot of aggregate epfd results

Galileo looks forward to further discussing these results during the meeting.

______________________________

France acts on behalf of all Galileo Administrations (GLS Administrations).