# Calculation Results for Aggregate System EPFD for the Seventeenth Resolution 609 (Rev. WRC-07) Consultation Meeting

# 1 Introduction

The purpose of this contribution is to provide updated results of equivalent power flux density (epfd) calculations performed by the U.S. in preparation for the Seventeenth Resolution 609 Consultation Meeting (17th CM) to be held via correspondence.

The following relevant materials are presented in this document:

- Section 2 presents an Executive Summary.

- Section 3 provides the U.S. calculation of the maximum RNSS aggregate epfd, according to § 11 d) of the CM Terms of Reference (MOD Cyberjaya, September 2019) (ToR).

- Section 4 compares the single-system epfd results obtained using the U.S. Resolution 609 calculation tool to other operator/administration results.

# 2 Executive Summary

## 2.1 Maximum Aggregate EPFD Results

The maximum aggregate epfd results provided in this report used the most recent single-system epfd results provided by the notifying administrations. Based on these contributions, the maximum aggregate epfd value occurs at 1 176 MHz and is -121.82 dB(W/(m2∙MHz)) which is 0.32 dB below the Resolution 609 limit of -121.5 dB(W/(m2∙MHz)). The U.S. believes this value is the correct maximum aggregate epfd value to report out of the 17th CM. (See Section 3.1 below for more details.)

## 2.2 Seventeenth Consultation Meeting Single-System EPFD Results

The following new or updated information was provided for the 17th CM:

- India requested that INSAT-NAVR(93.5) at 93.5E, which serves as a backup for INSAT-NAVR(55) and INSAT-NAVR(83), be included in the 17th CM calculation. Furthermore, India requested use of the epfd data from the 10th CM for INSAT-NAVR(93.5).

- China provided new system information on CENTISPACE-1, a non-GSO system consisting of two satellites in Low Earth Orbit (LEO). As listed in Note 4 of Table 1, China provided two separate epfd data sets and spectral adjustment factors corresponding to each satellite index. The U.S. recommends that only one epfd data set corresponding to  one non-GSO system is used for the the aggregate epfd calculations. Treating CENTISPACE-1 as two separate non-GSO systems may lead to overestimation of the aggregate epfd. For the purpose of concluding the work of the 17th CM, however, the U.S. treated CENTISPACE-1 as two separate non-GSO systems. The U.S. recommends that CENTISPACE-1 be treated as a single non-GSO system for the 18th CM.

- Malaysia provided updated information on MEASAT-1C at 91.5° E and the corresponding single-system epfd data. The U.S. calculated MEASAT-1C single-system epfd results based on the pfd data provided by Malaysia and the results show good agreement.

- The U.S. provided new information on USASAT-80C-1 at 125° W. The U.S. also stated that the frequency assignments in the 1164-1215 MHz band for the LM-RPS 133W network are no longer in operation and should not be taken into account by future CMs. Hence, this network was not included in the maximum aggregate epfd calculation results.

# 3 Maximum Aggregate EPFD Results

## 3.1 RNSS Systems Taken into Account in the Aggregate EPFD Calculations

The list of RNSS systems considered for the maximum aggregate epfd calculation is shown in Table 1. The systems with updated information are shown in gray.

Table 1: RNSS Systems Included in the Aggregate epfd Calculation Sets

| System/ Network# | RNSS System Name | Administration | Input Data Source |
| --- | --- | --- | --- |
| 1 | MSATNAV-2 | F/GLS | 8th CM |
| 2 | NAVSTAR GPS | USA | 8th CM |
| 3 | GLONASS-M (FDMA) | Russia | 8th CM |
| 4 | GLONASS-M (CDMA) | Russia | 11th CM |
| 5 (Note 1) | COMPASS-MEO (Regional) | China | 11th CM |
| 6 (Note 2) | COMPASS MEO (Global) | China | 11th CM |
| 7 (Note 1) | COMPASS IGSO (Regional) | China | 11th CM |
| 8 (Note 2) | COMPASS IGSO (Global) | China | 11th CM |
| 9 | QZSS | Japan | 16th CM |
| 10 | QZSS-GS-A1 (90.5E) | Japan | 16th CM |
| 11 | QZSS GS4 (127E) | Japan | 10th CM |
| 12 | LM-RPS-107.3W | USA | 8th CM |
| 13 | INMARSAT GSO-2N (64E) | Great Britain | 11th CM |
| 14 | INMARSAT-4 143.5E/-4A 143.5E | Great Britain | 8th CM |
| 15 | INMARSAT-4 98W/-4A 98W | Great Britain | 8th CM |
| 16-22 (Note 1) | COMPASS GSO (Regional) | China | 11th CM |
| 23-27 (Note 2) | COMPASS GSO (Global) | China | 11th CM |
| 28 | INSAT-NAV(55) | India | 10th CM |
| 29 | INSAT-NAVR(83) | India | 10th CM |
| 30 | INSAT-NAVR(32.5) | India | 10th CM |
| 31 | INSAT-NAVR(83) | India | 10th CM |
| 32 | INSAT-NAV(131.5) | India | 10th CM |
| 33 | INSAT-NAVR-GS + INSAT-NAV-GS17 | India | 15th CM |
| 34 | NIGCOMSAT-1R | Nigeria | 8th CM |
| 35 | LUX-G6-2-E | Luxembourg | 8th CM |
| 36 | LUX-G7-9-E2 | Luxembourg | 9th CM |
| 37 | ALCOMSAT-24.8W | Algeria | 11th CM |
| 38 | RAGGIANA-18 | PNG | 11th CM |
| 39 | LUX-G9-38 | Luxembourg | 13th CM |
| 40 | F-SAT-RN-5W | France | 16th CM |
| 41 (Note 3) | INSAT-NAVR(93.5) | India | 10th CM |
| 42 | MEASAT-1C (91.5E) | Malaysia | 17th CM |
| 43 | USASAT-80C-1 | USA | 17th CM |
| 44 & 45 (Note 4) | CENTISPACE-1 | China | 17th CM |

Note 1: The space constellation of the COMPASS regional system includes at most 18 satellites: 7 IGSOs, 4 MEOs and 7 GSOs (COMPASS-58.75E/80E/110.5E/140E/160E/B-84E/B-144.5E).

Note 2: The space constellation of the COMPASS global system consists of 35 satellites: 3 IGSOs, 27 MEOs and 5 GSOs ((COMPASS-58.75E/80E/110.5E/140E/160E).

Note 3: India requested that INSAT-NAVR(93.5) at 93.5E be included in the 17th CM calculation even though it is a backup to INSAT-NAVR(55) and INSAT-NAVR(83). Furthermore India asked to use the epfd data from 10th CM. Hence the US calculation includes this network.

Note 4: CENTISPACE-1 consists of two satellites in low earth orbit. China provided epfd data for each satellite rather than the aggregate epfd due to the system as a whole. See discussion in section 4.1.

## 3.2 Maximum Aggregate EPFD results using U.S. Tool

The maximum aggregate epfd results are summarized in Table 2 and Figure 1. The maximum aggregate epfd is -121.82 dB(W/(m2∙MHz)), which occurs at 1 176 MHz. The remaining margin before exceeding the Resolution 609 limit of -121.5 dB(W/(m2∙MHz)) is 0.32 dB.

Table 2: Maximum aggregate epfd calculation results

| Freq (MHz) | Max. aggregate epfd dB(W/(m2·MHz)) | Freq (MHz) | Max. aggregate epfd dB(W/(m2·MHz)) | Freq (MHz) | Max. aggregate epfd dB(W/(m2·MHz)) | Freq (MHz) | Max. aggregate epfd dB(W/(m2·MHz)) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1164 | -138.88 | 1177 | -123.11 | 1190 | -135.65 | 1203 | -125.68 |
| 1165 | -140.39 | 1178 | -123.49 | 1191 | -134.79 | 1204 | -124.80 |
| 1166 | -140.07 | 1179 | -124.32 | 1192 | -134.82 | 1205 | -124.48 |
| 1167 | -139.13 | 1180 | -124.75 | 1193 | -135.58 | 1206 | -123.68 |
| 1168 | -135.88 | 1181 | -125.78 | 1194 | -136.48 | 1207 | -122.54 |
| 1169 | -132.45 | 1182 | -127.61 | 1195 | -138.02 | 1208 | -123.59 |
| 1170 | -129.28 | 1183 | -129.94 | 1196 | -138.57 | 1209 | -125.30 |
| 1171 | -126.80 | 1184 | -132.50 | 1197 | -137.68 | 1210 | -125.90 |
| 1172 | -126.10 | 1185 | -135.94 | 1198 | -135.36 | 1211 | -127.03 |
| 1173 | -124.83 | 1186 | -139.00 | 1199 | -132.93 | 1212 | -128.30 |
| 1174 | -124.18 | 1187 | -141.29 | 1200 | -130.40 | 1213 | -130.16 |
| 1175 | -123.38 | 1188 | -139.68 | 1201 | -128.41 | 1214 | -132.44 |
| 1176 | -121.82 | 1189 | -137.45 | 1202 | -126.77 | 1215 | -135.70 |

Figure 1: Maximum aggregate epfd results

The maximum aggregate epfd value calculated using the U.S. tool is -121.82 dB(W/(m2∙MHz)) and occurs at 1 176 MHz.

# 4 Seventeenth Consultation Meeting Single-System EPFD Results

## 4.1 CENTISPACE-1

China provided new information on their CENTISPACE-1 system for the 17th CM. This experimental system consists of two satellites in LEO with the following orbital characterstics:

N 	(number of space stations of the non-GSO system) 	: 2

K 	(number of orbital planes)	: 1

h	 (satellite altitude above the Earth at apogee)	: 700 km

I (inclination angle of the orbital plane above the Equator)	: 97.5 degrees

| Satellite index I | RAAN i,0(degrees) | Argument of latitude ui,0(degrees) |
| --- | --- | --- |
| 1 | 0 | 0 |
| 2 | 0 | 36 |
| Is the system operational? (Yes or No) |  | Yes To date, one NGSO satellite has been launched. |  |

The two satellites transmit signals at different center frequencies as described in the following table :

| Satellite index I | Transmitted Signal Center Frequency (MHz) | Attenuation of transmitted signal in worst 1 MHz (dB) |
| --- | --- | --- |
| 1 | 1174.404 | 0.00 |
| 2 | 1178.496 | 0.00 |

As listed in Note 4 of Table 1 and in Section 2.2, China provided two separate epfd data sets and spectral adjustment factors corresponding to each satellite index. The U.S. recommends that only one epfd data corresponding to one non-GSO system be used for the work of CM as treating CENTISPACE-1 system as two separate nonGSO systems may lead to overestimation of the aggregate epfd. For the purpose of concluding the work of the 17th CM, however, the U.S. treated CENTISPACE-1 as two separate non-GSO systems. The U.S. recommends that CENTISPACE-1 be treated as a single non-GSO system for the 18th CM.

### 4.1.1 CENTISPACE-1 Satellite Index 1

A comparison of the epfd results from China and the U.S. for CENTISPACE-1 Satellite Inex 1 are shown in Figure 2 and Figure 3, respectively. The maximum epfd values as a function of latitudes for both tools are shown in Figure 4. The results are in very close agreement.

Figure 2: CENTISPACE-1 satellite index 1 epfd data provided by China

Figure 3: CENTISPACE-1 satellite index 1 epfd data calculated using the U.S. tool.

Figure 4: Comparison of CENTISPACE-1 satellite index 1 maximum epfd by latitude

### 4.1.2 CENTISPACE-1 Satellite Index 2

A comparison of the epfd results from China and the U.S. for CENTISPACE-1 Satellite Index 2 are shown in Figure 5 and Figure 6, respectively. The maximum epfd values as a function of latitude for both tools are shown in Figure 7. The results are in very close agreement.

Figure 5. CENTISPACE-1 satellite index 2 epfd data provided by China

Figure 6. CENTISPACE-1 satellite index 2 epfd data calculated using the U.S. tool.

Figure 7. Comparison of CENTISPACE-1 satellite index 2 maximum epfd by latitude

## 4.2 Malaysia MEASAT

For the 17th CM, Malaysia submitted updated information on MEASAT-1C at 91.5° E. A comparison of the epfd results for MEASAT-1C using Malaysia’s submission and the U.S. tool is shown in Figures 8 and 9, respectively. The maximum epfd values as a function of latitude for both tools are shown in Figure 10. The results are in very good agreement.

Figure 8. MESAT-1C (91.5° E) epfd data provided by Malaysia

Figure 9. MEASAT-1C (91.5° E) epfd data calculated using the U.S. tool

Figure 10. Comparison of Malaysia and U.S. calculations of MEASAT-1C (91.5° E) epfd data