# EXERCISE TO CAPTURE/VALIDATE EARTH STATION SUBMISSION FOR COORDINATION (ARTICLE 9) /NOTIFICATION (ARTICLE 11)

Procedure : No.9.17 Coordination

# NOTICE

A1f1  Notifying adm. 		SUI				Type of Earth Station		Specific

# STATION

A1e2  Station name	GENEVA_ITU              		 	A7b1  Min. elev. Angle	  27.1

A4c1  Assoc. space station	EUTELSAT 3-33E		A7c1  Start azimuth		  141.8

A4c2  Orbital longitude	33E				A7c2  End azimuth		  142

A1e3a  Ctry			SUI				A7d  Altitude			  400

| Azimuth | 0 | 90 | 180 | 270 |
| --- | --- | --- | --- | --- |
| A7a1 Hor. elev. angle | 0 | 0 | 0 | 0 |

A1e3b  Geo. coord.	        Longitude    6 E 8 44		A7a

Latitude     46 N 12 7

# BEAM

B1a/BR17  Beam designation	        BSR			B5b  Beamwidth		     0.4

B2  Emi-Rcp				E			B5c  Co-polar antenna pattern    29-25*LOG(FI)

(pattern id: 58)

B5a  Isotropic gain			52.5

# GROUP

A3a  Op. agency        012					C4b  Nature of service			CP

General characteristics

A3b  Adm. resp.	A					C3a  Assigned freq. band		20000

C4a  Class of station	TC					C6a  Polarization type			L / 3.5 degrees

C8g1 Maximum Aggregate Power     10  	                          C8g2 Aggregate Bandwidth 		20000

# EMISSIONS

C7a  Design. of emission 		20M0MXX-		C8c1 Minimum Peak Power (dBW) 10.0

C8a1/C8b1 Total Peak Power (dBW)  10.0			C8C3 Minimum Power Density (dBW) -60.0

C8a2/C8b2 Maximum Power Density -60.0

# FREQUENCIES

C2a1  Assigned frequency (MHz)	14.4 GHz

# SPECIAL SECTION

A13  Ref. to Special Sections		AR11/A/1165       CR/C/2578

Procedure : No. 11.2 Notification

Notice :  Change Provision Reference from 						No.9.17 to No.11.2

Beam :   A10a Coordination diagram. See Attachment : 						1

Group :                 General characteristics : A2a Date of bringing into use			31/01/2021

Group :                 Coordination :  A5/A6  Coordinations/Agreements			9.17  |  O  | F  I