| STEP 1: NOTICE |  |  |  |
| --- | --- | --- | --- |
| A1f1 Notifying adm. | SUI | Notice intended for | ADD, MOD, Sup |
| A1a Identity of the Network | TEST NGSO API | A1g. Short Duration Mission | Y/N |
| A4b2. Reference body | T --> Earth D --> Deep space S --> Sun … | If T, need A4b4 Orbital Information Orbital Plane id: 1 A.4.b.4.m sun-synchronous orbit or not : no A4b4a Inclination angle: 45 degrees , A4b4b Satellites in the plane: 1 A4b4c Period: 0d 1h 33m, A4b4d Apogee 400 x exp0 A4b4e Perigee: 400x exp0 , A4b4f Minimum Altitude: 400x exp0 |  |
| A4b1a. Constellation | Y | S (Single) |  |
|  |  | M (Multiple) | A4b1c. No of sub-sets |
|  |  |  | A4b1d. Attach_num |
|  | N |  |  |

EXERCISE TO CAPTURE/VALIDATE  API satellite network

(key parameters)

| STEP 2: BEAM (receiving) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| B1a Beam designation |  | UPLINK |  |  |  |  |
| B2 Transmission/reception indicator |  | Receiving Beam |  |  |  |  |
| B3a1 Max Isotropic Gain +/- dBi |  | 0 |  |  |  |  |
| B3c1 Co-polar |  | Pattern ID empty (diagram captured in GIMS mdb) |  |  |  |  |
| B4a Orbit link |  | The current beam operates with all sat in all orbits: Y |  |  |  |  |
| Beam has Sensors |  | If yes, no service area… |  |  |  |  |
| STEP 2-1 GROUP |  |  |  |  |  |  |
| A3a Operating Agency |  | 001 |  |  | General Characteristics |  |
| A3b Responsible Administration |  | A |  |  |  |  |
| A2b Period of Validity |  | 50 years |  |  |  |  |
| C4a Class of station |  | EA, ED |  |  | Characteristics Common to a Group of Frequencies |  |
| C4b Nature of service |  | CR |  |  |  |  |
| C11a Service area |  | XAA |  |  |  |  |
| C6a Polarization type |  | L , If linear, provide angle: 180 ° |  |  |  |  |
| C5a Receiving System Noise temp. |  | 500 Kelvins |  |  |  |  |
| STEP 2-2 EMISSIONS |  |  |  |  |  |  |
| C7a Design. of emission |  | 30K0V8WWF ; |  |  |  | 20K0F1D-- |
| C7b Carrier Frequency: |  | 145.87MHz; |  |  |  | 145 MHz |
| C8a1 Max Peak Power: |  | 19 dBW |  |  |  |  |
| C8a2 Max Power Density: |  | -25 |  |  |  |  |
| C8c1 Min Peak Power: |  | 3dBW |  |  |  |  |
| C8c3 Min Power Density: |  | -41 |  |  |  |  |
| C8e1 C/N Objective: |  | 12 dB |  |  |  |  |
| STEP 2-3 FREQUENCIES |  |  |  |  |  |  |
| C1 Frequency Range |  | 144-146 MHz |  |  |  |  |
| STEP 2-4 Associate Earth Station |  |  |  |  |  |  |
| C10b2 Type of Station |  | Specific |  |  |  |  |
| C10b1 Associated Earth Station Name |  | BA1DU |  |  |  |  |
| C10d1 Class of Station |  | TA, TD |  |  |  |  |
| C10d2 Nature of service |  | CR |  |  |  |  |
| C10d3 Max Isotropic Gain |  | 10 |  |  |  |  |
| C10d4 Beamwidth |  | 50 |  |  |  |  |
| C10d5a1 Radiation Pattern Id |  | 608 (Non-directional earth station antenna pattern) |  |  |  |  |
| C10c2 Country |  | SUI |  |  |  |  |
| C10c1 Geographical Coordinates |  | Longitude (6E8M10S), Latitude (46N23M26S) |  |  |  |  |
| STEP 3: BEAM (transmitting) |  |  |  |  |  |  |  |  |
| B1a Beam designation |  | DOWNLINK |  |  |  |  |  |  |
| B2 Transmission/reception indicator |  | Transmitting Beam |  | B2a1 Transmit Only when Visible |  |  |  |  |
|  |  |  |  | B2a2 the minimum elevation angle |  |  |  |  |
| B3a1 Max Isotropic Gain +/- dBi |  | 0 |  |  |  |  |  |  |
| B3c1 Co-polar |  | 609 (Non-directional space station antenna pattern.) |  |  |  |  |  |  |
| B4a Orbit link |  | The current beam operates with all sat in all orbits: Y |  |  |  |  |  |  |
| Beam has Sensors |  | If yes, no service area… |  |  |  |  |  |  |
| B1b Steerable Beam |  | If yes, B3b1b Apply ROP 21.16 PFD limits to steerable beams |  |  | applying the method in Annex 1 of ROP No.21.16 |  |  |  |
|  |  |  |  |  | applying other method in attachment no. ( ) |  |  |  |
| STEP 3-1 GROUP |  |  |  |  |  |  |  |  |
| Except no “C5a Receiving System Noise temp.”, all the other parameters are same as receiving beam |  |  |  |  |  |  |  |  |
| STEP 3-2 EMISSIONS |  |  |  |  |  |  |  |  |
| C7a Design. of emission |  | 20K0F1D-- |  |  |  |  |  |  |
| C7b Carrier Frequency: |  | 436.5 MHz |  |  |  |  |  |  |
| C8a1 Max Peak Power: |  | -3 dBW |  |  |  |  |  |  |
| C8a2 Max Power Density: |  | -47 |  |  |  |  |  |  |
| C8c1 Min Peak Power: |  | -3dBW |  |  |  |  |  |  |
| C8c3 Min Power Density: |  | -47 |  |  |  |  |  |  |
| C8e1 C/N Objective: |  | 12 dB |  |  |  |  |  |  |
| STEP 3-3 FREQUENCIES |  |  |  |  |  |  |  |  |
| C1 Frequency Range |  | 435-438 MHz |  |  |  |  |  |  |
| STEP 3-4 Associate Earth Station |  |  |  |  |  |  |  |  |
| C10b2 Type of Station |  | Typical |  |  |  |  |  |  |
| C10b1 Associated Earth Station Name |  | BA1DW |  |  |  |  |  |  |
| C10d1 Class of Station |  | TA |  |  |  |  |  |  |
| C10d2 Nature of service |  | CR |  |  |  |  |  |  |
| C10d3 Max Isotropic Gain |  | 10 |  |  |  |  |  |  |
| C10d4 Beamwidth |  | 50 |  |  |  |  |  |  |
| C10d5a1 Radiation Pattern Id |  | “diag_no=1” (diagram captured in GIMS mdb) |  |  |  |  |  |  |
| C10d6 Rev. system Noise Temperature |  | 500 |  |  |  |  |  |  |