REC. ITU-R P.528-5 EXECUTABLE SOFTWARE AND DIGITAL PRODUCTS

Author: William Kozma Jr

wkozma@ntia.gov

US Dept of Commerce, NTIA/ITS

May/June 2022 Geneva Study Group 3 Meetings

Copyright Notice and Copyright Waiver

This software was developed by employees of the National Telecommunications and Information Administration (NTIA), an agency of the Federal Government and is provided to you as a public service. Pursuant to Title 15 United States Code Section 105, works of NTIA employees are not subject to copyright protection within the United States.

The software is provided by NTIA “AS IS.” NTIA MAKES NO WARRANTY OF ANY KIND, EXPRESS, IMPLIED OR STATUTORY, INCLUDING, WITHOUT LIMITATION, THE IMPLIED WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, NON-INFRINGEMENT AND DATA ACCURACY. NTIA does not warrant or make any representations regarding the use of the software or the results thereof, including but not limited to the correctness, accuracy, reliability or usefulness of the software.

To the extent that NTIA holds rights in countries other than the United States, you are hereby granted the non-exclusive irrevocable and unconditional right to print, publish, prepare derivative works and distribute the NTIA software, in any medium, or authorize others to do so on your behalf, on a royalty-free basis throughout the World.

You may improve, modify, and create derivative works of the software or any portion of the software, and you may copy and distribute such modifications or works. Modified works should carry a notice stating that you changed the software and should note the date and nature of any such change.

You are solely responsible for determining the appropriateness of using and distributing the software and you assume all risks associated with its use, including but not limited to the risks and costs of program errors, compliance with applicable laws, damage to or loss of data, programs or equipment, and the unavailability or interruption of operation. This software is not intended to be used in any situation where a failure could cause risk of injury or damage to property.

Please provide appropriate acknowledgments of NTIA’s creation of the software in any copies or derivative works of this software.

Description

This package contains executable software for Rec. ITU-R P.528-5, "Propagation curves for aeronautical mobile and radionavigation services using the VHF, UHF and SHF bands."

The package consists of the following:

- p528_x86.dll and p528_x64.dll: 32- and 64-bit compiled binary libraries of the ITU-R P.528 model

- P528Drvr_x86.exe: 32-bit command-line driver that allows a user to call the p528_x86.dll DLL and export the output to a file

- Figure.m: Matlab/Octave .m file that allows the user to generate one of the figures that existed in                  Annex 3 of Rec. ITU-R P.528-3

The DLLs and executable were compiled using Microsoft Visual Studio 2019 with the platform toolset v142, the __stdcall calling convention for Excel compatibility, and the /MT runtime library so the software does not require the Microsoft C++ Redistributable library.

Compiled Binary Libraries

- 32-bit Windows DLL, p528_x86.dll

- 64-bit Windows DLL, p528_x64.dll

Each library contains a single function:

int P528(double d__km, double h_1__meter, double h_2__meter, double f__mhz, int T_pol, double p, Result *result)

Inputs

- d__km: Path distance (km); 0 < d__km

- h_1__meter: Height of the low terminal (m); 1.5 < h_1__meter < 20 000

- h_2__meter: Height of the high terminal (m); 1.5 < h_2__meter < 20 000  (h_2__meter > h_1__meter)

- f__mhz: Frequency (MHz); 100 < f__mhz  < 30 000

- T_pol: Polarization

  - 0 = Horizontal polarization

  - 1 = Vertical polarization

- p: Time percentage; 1 < p < 99

Output Structure

- int propagation_mode: Propagation mode

  - 0 = Not Set

  - 1 = LOS

  - 2 = Diffraction

  - 3 = Scattering

- double d__km: Path distance (km)

- double A__db: Total loss (dB)

- double A_fs__db: Free space path loss (dB)

- double A_a__db: Median atmospheric absorption loss (dB)

- double theta_h1__rad: Elevation angle of ray from low terminal (rad)

Propagation Mode

The DLL identifies the following propagation mode:

| Value | Interpretation |
| --- | --- |
| 1 | Line of sight |
| 2 | Diffraction |
| 3 | Troposcatter |

Return Codes

The DLL returns the following integer return codes:

| Value | Interpretation |
| --- | --- |
| 0 | SUCCESS |
| 1 | VALIDATION ERROR: D_KM < 0 |
| 2 | VALIDATION ERROR: H_1 < 1.5 OR H_1 > 20 000 |
| 3 | VALIDATION ERROR: H_2 < 1.5 OR H_2 > 20 000 |
| 4 | VALIDATION ERROR: H_1 > H_2 |
| 5 | VALIDATION ERROR: F_MHZ < 100 |
| 6 | VALIDATION ERROR: F_MHZ > 30 000 |
| 7 | VALIDATION ERROR: P < 1 |
| 8 | VALIDATION ERROR: P > 99 |
| 9 | VALIDATION ERROR: Invalid value for T_pol |
| 10 | VALIDATION ERROR: Terminals at same location (D_KM = 0 and H_1 = H_2) |
| 20 | WARNING: Possible discontinuity at the transition between diffraction and troposcatter propagation |

Note that the Propagation Mode and Return Code vary with distance along the propagation path.

P528Drvr_x86.exe

The command-line driver, P528Drvr_x86.exe, a Windows-only executable that calls the p528_x86.dll DLL from the command terminal and save the output to a file. The executable and DLL will run in both 32- and 64 bit Windows 10. The output file is saved in Command Separated Value (CSV) format. The executable file verifies that the version of p528_x86.dll and P528Drvr_x86.exe are identical. If they are not identical, there is no output from the executable file.

The following input arguments are not case sensitive, and the order does not matter.

Input Arguments:

-mode: The mode to operate the driver.  Possible values are:

POINT: Generate losses for a specific h1, h2, frequency, and q

CURVE: Generate losses vs. distance (0 < d < 1800 km) for specific h1, h2, frequency, and q

TABLE: Generate a table of losses vs. d for various h1, h2 and specific frequency and q

-h1: Height of the low terminal, in meters (Required for modes: POINT and CURVE)

-h2: Height of the high terminal, in meters (Required for modes: POINT and CURVE)

-d: Path distance, in km (Required for mode: POINT)

-f: Frequency, in MHz (Required for all modes)

-tpol: Polarization code (Required for all modes)

-p: Time percentage (Required for all modes)

-o: Output file (Required for modes: CURVE and TABLE; Optional for mode: POINT)

(if omitted for POINT, the output will print to screen)

-v: Version information

-h: Help information

Validation Examples

P528Drvr_x86.exe -mode POINT -h1 10 -h2 1000 -d 800 -f 500 -tpol 0 -p 50 [-o point.csv]

P528Drvr_x86.exe -mode CURVE -h1 25 -h2 20000 -f 1200 -tpol 1 -p 1 -o curve.csv

P528Drvr_x86.exe -mode TABLE -f 1200 -tpol 0 -p 95 -o table.csv

Refer to the attached point.csv, curve.csv, and table.csv files for validation.

Figure.m

Figure.m is an example Matlab/Octave-compatible .m file that allows the user to generate a figure similar to those present in Annex 3 of Rec. ITU-R P.528-3 that were removed from Rec. ITU-R P.528-4. The file contains information in a commented header block explaining how to call it.  P528Drvr_x86.exe and p528_x86.dll must be located in the same directory as Figure.m.

The included file fig_4-4b.png is an example figure corresponding to Figure(1200, 10000, 50, 0) which is Figure 4-4(b) in Rec. ITU-R P.528-3.

Excel Compatibility

The 32- and 64-bit DLLs were compiled to be 32- and 64-bit Excel compatible, respectively. The following VBA code can be used to interface to the 32-bit and 64-bit DLLs.  The Excel function is pdot528(). The input parameters are d_km, h1_m, h2_m, f_MHz, T_pol, and pct and the Excel function returns the array: propagation_mode, result.d__km, A__db, A_fs__db, and i.

- Ensure the Excel workbook and the appropriate DLL are in the same folder.

- Add the following VBA code to the ThisWorkbook module in the Excel workbook:

Private Sub Workbook_Open()

ChDir ActiveWorkbook.Path

Application.CalculateFull

End Sub

- Add the following VBA code to a new module in the Excel workbook:

#If Win64 Then

Private Declare PtrSafe Function P528 Lib "p528_x64.dll" (ByVal d_ As Double, ByVal h1 As Double, ByVal h2 As Double, ByVal f As Double, ByVal pol As Double, ByVal pct As Double, ByRef result As Result_Type) As Long

#Else

Private Declare PtrSafe Function P528 Lib "p528_x86.dll" (ByVal d_ As Double, ByVal h1 As Double, ByVal h2 As Double, ByVal f As Double, ByVal pol As Double, ByVal pct As Double, ByRef result As Result_Type) As Long

#End If

Type Result_Type

propagation_mode As Long

dummy As Long

d__km As Double

A__db As Double

A_fs__db As Double

A_a__db As Double

theta_h1__rad As Double

End Type

Function pdot528(d_km As Double, h1_m As Double, h2_m As Double, f_MHz As Double, T_pol As Double, pct As Double) As Variant

Dim result As Result_Type, i As Long

i = P528(d_km, h1_m, h2_m, f_MHz, T_pol, pct, result)

pdot528 = Array(result.propagation_mode, result.d__km, result.A__db, result.A_fs__db, result.A_a__db, i)

End Function

______________________