| Radiocommunication Study Groups |  |
| --- | --- |
|  | 5 July 2012 |
|  | English only |
| Working Party 3M |  |
| validation examples for prediction methods in recommendation itu-r p.530-14 |  |

The example values presented in the following tables can be used to verify the correct software implementation of the prediction methods of Recommendation ITU-R P.530-14 concerning the statistics of:

•	fading due to multipath;

•	rain attenuation;

•	XPD outage due to clear sky;

•	XPD outage due precipitation attenuation;

•	distortion due to selective fading;

•	space diversity outage;

•	total outage.

For each case shown, the intermediate and final results listed should be obtained if the given input values are used.

It should be noted that the examples do not represent any particular measurement programme, system, terrestrial microwave link, or frequency allocation, but are provided to demonstrate the range of computational capability of the prediction method.

It should be also noted that the intermediate and final values are shown with more precision than required for the appropriate implementation of the prediction method to assist users in tracing possible problems with the implementation.

General information

These new validation examples are the update of the validation examples submitted by Canada in 2011 (Document 3M/213). Only, Table 2 and Table 4 are affected. See notes and instructions below for details.

Table 1

Validation examples of the fading due to multipath model – all percentages of time (§ 2.3.1 and § 2.3.2 of Recommendation ITU-R P.530-14 using
Recommendations ITU-R P.453-9 for the point refractivity gradient)

|  | Input values |  |  |  |  |  | Intermediate values |  |  |  | Final results |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Latitude (°N) | Longitude (°E) | Path length d (km) | Antenna height he (m) | Antenna height hr (m) | Frequency f (GHz) | dN1 (N-unit/km) | Geoclimatic factor K | Multipath occurrence factor p0 (%) | Transition fade depth At (dB) | pW for A = 2 dB (%) | pW for A = 5 dB (%) | pW for A = 10 dB (%) | pW for A = 30 dB (%) |
| 1 | 39.55 | 116.25 | 80 | 100 | 55 | 2 | –333.54 | 1.998E-4 | 138.7 | 27.5705 | 25.86 | 10.71 | 4.842 | 0.1387 |
| 2 | 38 | 23.4 | 60 | 45 | 30 | 6 | –594.75 | 0.001013 | 974.3 | 28.5864 | 37.24 | 25.06 | 19.18 | 0.9743 |
| 3 | 29.46 | 264.78 | 45 | 500 | 610 | 8 | –324.14 | 1.884E-4 | 9.652 | 26.1815 | 15.15 | 3.141 | 0.6869 | 9.652E-3 |

Table 2

Validation examples of the long-term rain attenuation model (§ 2.4.1 of Recommendation ITU-R P.530-14
using Recommendations ITU-R P.837-5 for rain rate and ITU-R P.838-3 for specific attenuation)

| Input values |  |  |  |  |  |  | Intermediate values |  |  |  | Final results |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Latitude (°N) | Longitude (°E) | Frequency f (GHz) | Path length d (km) | Pol | Polarizationtilt angle τ (°) | R001 (mm/h) | γr (dB/km) | r | deff (km) | A0.001 (dB) | A0.01 (dB) | A0.1 (dB) | A1 (dB) |
| 1 | –22.5 | 317 | 13 | 20 | L | 90 | 53.7662 | 2.5140 | 0.51333 | 10.2650 | 51.8956 | 25.8058 | 9.7859 | 2.8463 |
| 2 | 1.17 | 103.51 | 18 | 10 | L | 0 | 105.5145 | 10.9347 | 0.5085 | 5.0851 | 109.8449 | 55.6038 | 21.0394 | 5.9851 |
| 3 | 48.52 | 2.2 | 30 | 8 | L | 90 | 33.7179 | 5.6863 | 0.6373 | 5.0981 | 55.6883 | 28.9890 | 10.9312 | 3.0032 |

Table 3

Validation examples of XPD outage due to clear-air effects (§ 4.1 of Recommendation ITU-R P.530-14)

|  | Input values |  |  |  |  |  |  | Intermediate values |  |  |  |  |  | Final results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Multipath occurrence factor p0 (%) | P0 = p0/100 | XPDg (dB) | Vertical antenna separation st (m) | Wavelength λ (m) | C0/I (dB) | XPIF (dB) | XPD0 (dB) | Multipath activity η | kXP | Q | C | XPD margin MXPD (dB) | Outage probability PXP |
| 1 | 138.7 | 1.387 | 40 | 0 | 0.15 | 15 | 0 | 40 | 0.2256 | 0.700 | 9.4372 | 49.4372 | 34.4372 | 4.9930E-4 |
| 2 | 974.3 | 9.743 | 30 | 1 | 0.05 | 20 | 0 | 35 | 0.6681 | 0.7005 | 13.1845 | 48.1845 | 28.1845 | 1.4799E-2 |
| 3 | 9.652 | 0.09652 | 42 | 2 | 0.0375 | 25 | 20 | 40 | 0.0340 | 0.7034 | 6.0542 | 46.0542 | 41.0542 | 7.5717E-6 |

Table 4

Validation examples for predicting XPD outage due to precipitation effects (§ 4.2.2 of Recommendation ITU-R P.530-14)

| Input values |  |  |  |  |  |  |  |  | Intermediate values |  |  |  |  |  | Final results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Latitude (°N) | Longitude (°E) | Frequency f (GHz) | Path length d (km) | Pol τ (°) | C0/I (dB) | U0 | XPIF (dB) | A0.01 (dB) | U | V | Ap (dB) | m | n | Outage probability PXPR |
| 1 | –22.5 | 317 | 13 | 20 | 90 | 15 | 15 | 12 | 25.8058 | 48.42 20,8381522399266 151,207828111670 39,2786006533252 | 20.84 | 151.21 | 39.2786 | -5.3357 | 4.6169e-08 |
| 2 | 1.17 | 103.51 | 18 | 10 | 0 | 20 | 15 | 0 | 55.6038 | 52.66 22,1672462424059 29,7346351781868 15,0952115323682 | 22.17 | 29.73 | 15.0952 | -1.3288 | 4.6901 e-04 |
| 3 | 48.52 | 2.2 | 30 | 8 | 90 | 25 | 15 | 0 | 28.9890 | 59.31 22,6000000000000 32,9839414224825 22,7224045883904 | 22.60 | 32.98 | 22.7224 | -2.1565 | 6.9736e-05 |

Table 5

Validation examples of the distortion due to selective fading model (§ 5.1 of Recommendation ITU-R P.530-14 using
Recommendations ITU-R F.1093-2 for the signature parameters)

|  | Input values |  |  |  |  |  |  |  |  |  | Intermediate | Final results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Latitude (°N) | Longitude (°E) | Path length d (km) | Frequency f (GHz) | Multipath occurrence factor p0 (%) | Multipath activity η | Baud period T (ns) | Modulation | Normalized system parameter Kn,M | Normalized system parameter Kn,NM | Mean delay τm (ns) | Selective outage probability Ps |
| 1 | 39.55 | 116.25 | 80 | 2 | 138.7 | 0.2256 | 105 | 8-PSK | 7.0 | 7.0 | 1.2896 | 1.024E-3 |
| 2 | 38 | 23.4 | 60 | 6 | 974.3 | 0.6681 | 77 | 16-QAM | 5.5 | 5.5 | 0.8872 | 2.098-3 |
| 3 | 29.46 | 264.78 | 45 | 8 | 9.652 | 0.0340 | 35.5 | 64-QAM | 15.4 | 15.4 | 0.6104 | 6.664E-4 |

Table 6

Validation examples of the non-selective space diversity improvement model (§ 6.2.4 of Recommendation ITU-R P.530-14)

|  | Input values |  |  |  |  |  | Final results |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No | Path length d (km) | Frequency f (GHz) | Multipath occurrence factor p0 (%) | Antenna vertical separation S (m) | Absolute difference in antenna gains V (dB) | Fade depth A (dB) | Diversity improvement Ins |
| 1 | 80 | 2 | 138.7 | 15 | 4 | 30 | 7.4264 |
| 2 | 60 | 6 | 974.3 | 10 | 1 | 30 | 1.0560 |
| 3 | 45 | 8 | 9.652 | 5 | 6 | 30 | 18.0006 |

Table 7

Validation examples of the space diversity outage probability model (§ 6.2.5.1 of Recommendation ITU-R P.530-14)

|  | Input values |  |  |  | Intermediate values |  |  |  |  | Final results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Non-selective outage probability Pns = pw/100 (A = 30 dB) | Selective outage probability Ps | Diversity improvement Ins | Multipath activity η | Non-selective correlation coef. kns | Correlation coefficient rw | Selective correlation coef. ks | Non-selective diversity outage probability Pdns | Selective diversity outage probability Pds | Total diversity outage probability Pd |
| 1 | 0.001387 | 1.024E-3 | 7.4264 | 0.2256 | 0.9769 | 0.9715 | 0.9677 | 1.8677E-4 | 7.3097E-05 | 3.192-04 |
| 2 | 0.009743 | 2.098-3 | 1.0560 | 0.6681 | 0.9923 | 0.9908 | 0.9820 | 9.226E-3 | 1.8452E-04 | 9.886E-03 |
| 3 | 0.00009652 | 6.664E-4 | 18.0006 | 0.0340 | 0.9741 | 0.9681 | 0.9657 | 5.3621E-6 | 1.9337E-04 | 2.111-04 |

Table 8

Prediction of total outage due to clear-air effects (§ 7 of Recommendation ITU-R P.530-14)

|  | Input values |  |  |  | Final results |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Non-selective outage probability Pns = pw/100 (A = 30 dB) | Selective outage probability Ps | XPD outage probability PXP | Diversity outage probability Pd | Total outage probability without diversity Pt | Total outage probability with diversity Pt |
| 1 | 0.001387 | 1.024E-3 | 4.9930E-4 | 3.192-04 | 2.910E-03 | 8.185E-04 |
| 2 | 0.009743 | 2.098-3 | 1.4799E-2 | 9.886E-03 | 2.664E-02 | 2.469E-02 |
| 3 | 0.00009652 | 6.664E-4 | 7.5717E-6 | 2.111-04 | 7.705E-04 | 2.187E-04 |

NOTE 1  In Table 1, the point refractivity gradient in the lowest 65 m of the atmosphere not exceeded for 1% of an average year, dN1, is calculated using Recommendation ITU-R P.453-9. For the calculation of the geoclimatic factor K and the percentage of time p0 that the fade depth A = 0 dB is exceeded, the corresponding quick calculation equations (5) and (8) – which is the same as equation. (11) – are employed, respectively.

NOTE 2  In Table 2, the  and k parameters of the specific rain attenuation are computed using the sum-formula recommended in Recommendation ITU-R P.838-3, while for rain rate statistics the maps in Recommendation ITU-R P.837-5 are used.

NOTE 3 – The example in the first row of Table 4 (where an XPIC device is used) yields a value for n less than –3. In this case, the outage BER < .

NOTE 4  In Table 5, equation (68), which is a more compact version of equation. (67) is used. The normalized system parameter for each modulation is obtained from Recommendation ITU-R F.1093-2.

NOTE 5 – The total outage probability due to rain is calculated from taking the largest value between Prain and PXPR.

_______________

____________________