LFMF-SmoothEarth (P.368) EXECUTABLE SOFTWARE AND DIGITAL PRODUCTS

Author: William Kozma Jr

wkozma@ntia.gov

US Dept of Commerce, NTIA/ITS

June 2021 Geneva Study Group 3 Meetings

Copyright Notice and Copyright Waiver

This software was developed by employees of the National Telecommunications and Information Administration (NTIA), an agency of the Federal Government and is provided to you as a public service. Pursuant to Title 15 United States Code Section 105, works of NTIA employees are not subject to copyright protection within the United States.

The software is provided by NTIA “AS IS.” NTIA MAKES NO WARRANTY OF ANY KIND, EXPRESS, IMPLIED OR STATUTORY, INCLUDING, WITHOUT LIMITATION, THE IMPLIED WARRANTY OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE, NON-INFRINGEMENT AND DATA ACCURACY. NTIA does not warrant or make any representations regarding the use of the software or the results thereof, including but not limited to the correctness, accuracy, reliability or usefulness of the software.

To the extent that NTIA holds rights in countries other than the United States, you are hereby granted the non-exclusive irrevocable and unconditional right to print, publish, prepare derivative works and distribute the NTIA software, in any medium, or authorize others to do so on your behalf, on a royalty-free basis throughout the World.

You may improve, modify, and create derivative works of the software or any portion of the software, and you may copy and distribute such modifications or works. Modified works should carry a notice stating that you changed the software and should note the date and nature of any such change.

You are solely responsible for determining the appropriateness of using and distributing the software and you assume all risks associated with its use, including but not limited to the risks and costs of program errors, compliance with applicable laws, damage to or loss of data, programs or equipment, and the unavailability or interruption of operation. This software is not intended to be used in any situation where a failure could cause risk of injury or damage to property.

Please provide appropriate acknowledgments of NTIA’s creation of the software in any copies or derivative works of this software.

Description

This package contains supplemental executable software for Rec ITU-R P.368, “Ground-wave propagation curves for frequencies between 10 kHz and 30 MHz”.  The supplemental executable software is the Low Frequency/Medium Frequency, Smooth Earth Model (LFMF, v1.1).

The package consists of the following:

- LFMF.dll: 32- and 64-bit compiled binary libraries of LFMF, v1.1

- LFMFDrvr.exe: 32-bit command-line driver that allows a user to call the lfmf.dll 32-bit DLL and export the output to a file

- LFMFDrvr.exe Source Code: The C++ source code for the LFMFDrvr.exe executable

- Ground-wave curve figures: Re-generation of the set of curve figures from Rec. ITU-R P.368-9 using LFMF-SmoothEarth.

The DLLs and executable were compiled using Microsoft Visual Studio 2019 with the platform toolset v142, the __stdcall calling convention for Excel compatibility, and the /MT runtime library so the software does not require the Microsoft C++ Redistributable library.

Each library contains a single function:

int LFMF(double h_tx__meter, double h_rx__meter, double f__mhz, double P_tx__watt,

double N_s, double d__km, double epsilon, double sigma, int pol, Result *result)

Inputs

- h_tx__meter: Height of the transmitter (m); 0 ≤ h_tx__meter ≤ 50

- h_rx__meter: Height of the receiver (m); 0 ≤ h_rx__meter ≤ 50

- f__mhz: Frequency (MHz); 0.01 ≤ f__mhz  ≤ 30

- P_tx__watt: Transmit power (W); 0 < P_tx__watt

- N_s: Surface refractivity (N-Units); 250 ≤ N_s ≤ 400

- d__km: Distance (km); d__km ≤ 10 000

- epsilon: Relative permittivity on the surface of the Earth; 1 ≤ epsilon

- sigma: Conductivity (S/m) on the surface of the Earth; 0 < sigma

- pol: Polarization

Assumptions

The LFMF DLL implements the same assumptions that are defined in Rec P.368, specifically, that the transmit and receive antennas are short vertical monopoles on a perfectly conducting ground plane.  These antenna’s each have a gain of 4.77 dBi.

Polarization

The DLL identifies the following polarization options:

| Value | Interpretation |
| --- | --- |
| 0 | Horizontal polarization |
| 1 | Vertical polarization |

Output Structure

- double A_btl__db: Basic transmission loss (dB)

- double E_dbuVm: Electric field strength (dB(uV/m))

- double P_rx__dbm: Received Power (dBm)

- int method: Solution method

Solution Method

The DLL identifies the following method used in the computation of the solution:

| Value | Interpretation |
| --- | --- |
| 0 | Flat Earth with curve correction |
| 1 | Residue series |

Return Codes

The DLL returns the following integer return codes:

| Value | Interpretation |
| --- | --- |
| 0 | SUCCESS |
| 1000 | VALIDATION ERROR: h_tx__meter out of range |
| 1001 | VALIDATION ERROR: h_rx__meter out of range |
| 1002 | VALIDATION ERROR: f__mhz out of range |
| 1003 | VALIDATION ERROR: P_tx__watt out of range |
| 1004 | VALIDATION ERROR: N_s out of range |
| 1005 | VALIDATION ERROR: d__km out of range |
| 1006 | VALIDATION ERROR: epsilon out of range |
| 1007 | VALIDATION ERROR: sigma out of range |
| 1008 | VALIDATION ERROR: invalid value for pol |

LFMFDrvr_x86.exe

The command-line driver, LFMFDrvr.exe, a Windows-only executable that calls the LFMF.dll DLL from the command terminal and save the output to a file. The executable and DLL will run in both 32- and 64 bit Windows 10. The output file is saved in Command Separated Value (CSV) format. The executable file verifies that the version of LFMF.dll and LFMFDrvr.exe are identical. If they are not identical, there is no output from the executable file.

The following input arguments are not case sensitive, and the order does not matter.

Input Command-Line Driver Arguments:

-mode: The mode to operate the driver.  Possible values are:

POINT: Generate losses for a specific path distance (d__km)

LINE: Generate losses vs. distance (1 < d < 9900 km)

-i: Input file

-o: Output file

-method: Include the solution method in the output file (optional)

-v: Version information

-h: Help information

Input File Format

Inputs are specified in a text file in KEY,VALUE pairs.  The order of the input parameters does not matter.  The KEY is the DLL input variable name; the VALUE is the value that should be assigned.

Validation Examples

LFMFDrvr.exe -i point_input.txt -o point_output.txt -mode POINT -method

LFMFDrvr.exe -i line_input.txt -o line_output.txt -mode LINE -method

Refer to the attached point_output.txt, line_output.txt files for validation.

Excel Compatibility

The 32- and 64-bit DLLs were compiled to be 32- and 64-bit Excel compatible, respectively. The 32-bit or 64-bit LFMF.dll must correspond to the installed version of Excel. The following VBA code can be used to interface to the 32-bit or 64-bit DLLs.  The Excel function is Pdot368(). The input parameters are h_tx__meter, h_rx__meter, f__mhz, P_tx__watt, N_s, d__km, epsilon, sigma, and pol; and the Excel function returns the array: A_btl__db, E_dBuVm, P_rx__dbm, and i.

- Ensure the Excel workbook and the appropriate DLL are in the same folder.

- Add the following VBA code to the ThisWorkbook module in the Excel workbook:

Private Sub Workbook_Open()

ChDir ActiveWorkbook.Path

Application.CalculateFull

End Sub

- Add the following VBA code to a new module in the Excel workbook:

Option Explicit

Declare PtrSafe Function LFMF Lib "LFMF.dll" (ByVal h_tx__meter As Double, ByVal h_rx__meter As Double, ByVal f__mhz As Double, ByVal P_tx__watt As Double, ByVal N_s As Double, ByVal d__km As Double, ByVal epsilon As Double, ByVal sigma As Double, ByVal pol As Long, ByRef Result As Result_Type) As Long

Type Result_Type

A_btl__db As Double

E_dBuVm As Double

P_rx__dbm As Double

Method As Long

End Type

Function Pdot368(h_tx__meter As Double, h_rx__meter As Double, f__mhz As Double, P_tx__watt As Double, N_s As Double, d__km As Double, epsilon As Double, sigma As Double, pol As Long) As Variant

Dim Result As Result_Type, i As Long

i = LFMF(h_tx__meter, h_rx__meter, f__mhz, P_tx__watt, N_s, d__km, epsilon, sigma, pol, Result)

Pdot368 = Application.Transpose(Array(Result.A_btl__db, Result.E_dBuVm, Result.P_rx__dbm, Result.Method, i))

End Function

______________________