| QAT – Qatar (Space Service) |  |  |  |
| --- | --- | --- | --- |
| Centralizing office | Postal address | Telephone, Telefax, Electronic-mail | Remarks |
| Communications Regulatory Authority (CRA) | Al Nasr Tower-B Corniche Road P.O. Box 23404 Doha | PHONE: +974 44995374 TELEFAX: +974 44830630 EMAIL: msadeq@cra.gov.qa | Contact: Mr. Mohammad Sadeq Spectrum Management Department Director |

Stations in the Space radiocommunication services

| Name of the station | Postal address | Telephone, Telefax, Electronic-mail |  |
| --- | --- | --- | --- |
| Space Radio Monitoring Center (SRMC) | Communications Regulatory Authority (CRA) Al Nasr Tower-B Corniche Road P.O. Box 23404 Doha Qatar | PHONE: +974 449 95374 TELEFAX: +974 448 30630 EMAIL: srmc@cra.gov.qa |  |

1. Geographical coordinates

25°05'03"N. 051°10'29"E.

2. Hours of service

Sunday - Thursday 04:00 AM - 11:00 AM UTC.

3. Information on antennas in use

(A) Two 7.3 m Turning-Head Antennas for frequency ranges: 3.4 - 4.8 GHz (C band) and 10.7 - 12.75 GHz (Ku band) (B) Two 6.2 m Turning-Head Antennas for frequency ranges: 7.25 - 7.75 GHz (X band) and 17.6 - 22.0 GHz (Ka band) (C) One 3.5 m X/Y full motion antenna for frequency ranges: 10.7 - 12.75 GHz (Ku band) and 17.7 - 21.2 GHz (Ka band).

4. Range of azimuth and elevation angles

(A) Azimuth = 90° - 270° Elevation = 10° – 85° (B) Azimuth = 90° - 270° Elevation = 10° – 85° (C) Azimuth = 360° Elevation = 5° – 175°.

5. Maximum attainable accuracy in determining orbital positions of space stations

(A) 0.016° (B) 0.016° (C) 0.010°.

6. Information on system polarization

(A) 3.4 - 4.8 GHz: Linear (Horizontal and Vertical), Circular (RHC, LHC) 10.7 - 12.75 GHz: Linear (Horizontal and Vertical) (B) 7.25 - 7.75 GHz: Circular (RHC, LHC) 17.6 - 22 GHz: Linear (Horizontal and Vertical), Circular (RHC, LHC) (C) 10.7 - 12.75 GHz: Linear (Horizontal and Vertical) 17.7 – 21.2 GHz: Linear (Horizontal and Vertical).

7. System noise temperature

(A) 3.4 - 4.8 GHz: 110K 10.7-12.75 GHz: 150K (B) 7.25 - 7.75 GHz: 120K 17.6 - 22 GHz: 350K (C) 10.7-12.75GHz: 130K 17.7-21.2 GHz: 310K.

8. Ranges of frequencies with the maximum attainable accuracy of frequency measurement for each frequency range

(A) 3.4 - 4.8 GHz: Accuracy: 5 x 10^-8 10.7 - 12.75 GHz: Accuracy: 5 x 10^-8 (B) 7.25 - 7.75 GHz: Accuracy: 5 x 10^-8 17.6 - 22 GHz: Accuracy: 1 x 10^-8 (C) 10.7-12.75 GHz: Accuracy: 1 x 10^-10 17.7-21.2 GHz: Accuracy: 1 x 10^-10.

9. Ranges of frequencies in which field strength or power flux-density measurements can be performed

All frequency ranges as mentioned in (A), (B), (C).

10. Minimum value of measurable field strength or power flux-density with indication of attainable accuracy of measurement

(A) 3.4 - 4.8 GHz: -210 dBW/m² ± 2 dB 10.7-12.75 GHz: -220 dBW/m² ± 2 dB (B) 7.25 - 7.75 GHz: -210 dBW/m² ± 2 dB 17.6 - 22 GHz: -220 dBW/m² ± 2 dB (C) 10.7-12.75GHz: -185 dBW/m² ± 2 dB 17.7-21.2 GHz: -185 dBW/m² ± 2 dB.

11. Information available for bandwidth measurements

Bandwidth Measurement in accordance with the methods described in the Spectrum Monitoring Handbook..

12. Information available for spectrum occupancy measurements

Automatic and Manual spectrum occupancy measurements are carried out in all frequency ranges..

13. Information available for orbit occupancy measurements

Automatic and Manual orbit occupancy measurements are carried out in all frequency ranges..

______________