BS 01: VHF analog sound broadcasting assignment

Prepare an electronic notice file for the recording in the Master Register of a sound broadcasting station transmitting at 101.1 MHz.

- Purpose of exercise: To create a frequency assignment notice using TerRaNotices “File/New file” functionality, for addition to the MIFR

Solution

- The notice type to use for the notification of an analog sound broadcasting station in the VHF band is: T01

- The identifying elements for broadcasting station are :

  - Assigned Frequency

  - Geographical coordinates

Electronic notice file:

- Open TerRaNotices and select the “New file” option from the File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- Select Administration TRD, VHF/UHF broadcasting notices category and T01 notice type as requested.

- Then select OK to create the T01 notice.

- Fill in the required fields as given in the exercise.

- Finally validate and save the file in order to submit to BR via WISFAT.