BS 02: UHF digital television broadcasting assignment

Prepare an electronic notice file for the recording in the Master Register of a TV broadcasting station transmitting at 503 MHz.

- Purpose of exercise: To create a frequency assignment notice using the “File/Wizard” functionality in TerRaNotices, for recording in the Master Register

Solution

- The notice type to use for the notification of a digital television broadcasting station in the UHF band is:  T02

- The purpose of the exercise is to record in the Master Register, the relevant fragment is: Article 11

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not later than 3 months from the date of receipt of the notice by the BR

  - Operating hours

- The Administrations are requested to submit in remarks field the additional information as:

  - Modulation code (modul_code)

  - Code rate (code_rate)

  - Reception mode (rx_mode)

  - Class of Emission (emi_cls)

Electronic notice file:

- Open TerRaNotices and select “Wizard” option from File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- Select Administration USA and select update of Master Register as Fragment.

- Select ADD, Assignment, Class of station BT. Note the selected Notice type at the bottom of the screen. Then select finish to generate the required “T02” notice.

- Confirm the Summary.

- Fill the required parameters as given in the exercise.

- Validate and save the file in order to submit to BR via WISFAT.