BS03 Solution: Modification of an assignment which is recorded in the Master register

Prepare an electronic notice file proposing to modify the assigned Frequency to 102.9 MHz of a frequency assignment which is recorded in the Master Register having the unique identification code HC-AR5.

Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “File/Open a notice from the database” functionality to modify an existing assignment.

Solution

- TerRaNotices provides a tool “File/Open a notice from the DB”. The frequency assignment to modify can be identified by its unique identification code.

- The frequency assignment notice belongs to the FMTV domain and it is a notice proposing a modification to a frequency assignment notice in the Master Register, therefore the relevant fragment is Article 11.

- The purpose of the exercise is to modify the site name of a recorded assignment; therefore the intent shall be Modify.

- When submitting changes to a recorded assignment, the identifying elements of a BS assignment shall be either:

  - Frequency and geographical coordinates, or

  - Unique identification code of the assignment given by the administration, if this data item has been notified previously and recorded with the Frequency assignment to be modified.

For this exercise we use the Unique identification code to identify the Frequency assignment to be modified.

Electronic notice file:

- Open TerRaNotices and select “Open a notice from the database” option from File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- Select Domain FMTV, Administration EQA, Fragment Article 11, Recorded Assignment, then enter Unique identification code “HC-AR5” as requested in the exercise. (You must click on “Add >” to “confirm” the unique identification code)

- By executing the above selection, the required assignment is retrieved from the database.

- Select the assignment and click Ok to open the selected T01 notice.

- Change the intention to Modification as requested in the exercise.

- Enter the Unique identification code of the target assignment to be modified by selecting the “ID” button at the bottom left of the dialog box. This is a TerRaNotices functionality to copy the identifying elements automatically.

- Enter the required value of the new Frequency as requested.

- Validate and save the Modification notice file in order to submit to the BR through WISFAT.