BS05 Solution: AM sound broadcasting station (in the tropical zone).

Prepare an electronic notice file for the recording in the Master Register of an AM broadcasting station transmitting at 5527 kHz, having a circular receiving area of 200 km.

Purpose of exercise:  To adequately select the notice type, you may use the “File/Wizard” functionality available in TerRaNotices.

Solution

- TerRaNotices provides a notice type selection Wizard.

- The frequency assignment notice belongs to the FXM domain and it is a notice proposing the addition of a frequency assignment notice to the Master register, therefore the relevant fragment is Article 11.

- Class of station is BC

- The notice type to use for requesting Addition of AM Sound Broadcasting Station: T12

Electronic notice file:

- Open File/Wizard and select “TRD” and  “Update of MIFR”

- Select ADD

- Fill the required fields as requested.

Class of station is BC   Frequency 5527 kHz

- Click on Finish and you should have the following dialog

- Then Generate the T12 notice.

- Fill all required data

- To finish the task, Validate and save the notice in order to notify to the BR through WISFAT.