SOLUTION: FXM 02: Fixed service (Point-to-Multipoint) in shared bands

Prepare an electronic notice of frequency 7.05000 GHz, which falls within the bands shared on equal basis with the space services, used for the operation of three fixed links for its recording in the Master Register.

Solution

- The purpose of the exercise is to notify three fixed links for its recording in the Master Register.

- The notice type for fixed station is T11. The identifying elements for fixed station are:  Assign Frequency; Geographical coordinates; Designation of emission (bandwidth code and class of emission); Class of station Operating hours

Therefore, if an Administration submits a notification for each link, the identifying elements of those two links are identical, only the antenna characteristics will be different.

The processing system in the BR, allow administrations to change the submitted data at any time, for this, the system needs to identify the concerned notice (with the identifying elements) therefore, the first notice will be replaced by the second notice. 
Administrations need to submit a single notice and the three links are notified within the antenna characteristics (or Operations)

- The assigned frequency falls within the bands shared on equal basis with space services, the following fields are mandatory

  - Altitude of site above sea level

  - Height of Antenna above ground level

  - Elevation angle

- The value of the Maximum antenna gain and radiated power shall be notified in isotropical values.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list of “Nature of services” in Chapter IV, Section 7.

- The class of station is a mandatory field and in this case it is FX; the preface to the BR IFIC provides a complete list of “class of station” in Chapter IV, Section 6

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 years

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

The completed notice in text format.

<HEAD>

t_adm=CLM

</HEAD>

<NOTICE>

t_notice_type=T11

t_fragment=NTFD_RR

t_action=ADD

t_freq_assgn=7050.000000

t_long=-0761249

t_lat=+033023

t_site_name=BUITRERA

t_addr_code=A

t_op_hh_to=24:00

t_prov=RR11.2

t_emi_cls=G7E--

t_d_inuse=2013-10-15

t_site_alt=1224

t_op_hh_fr=00:00

t_bdwdth_cde=28M0

t_is_resub=FALSE

t_stn_cls=FX

t_ctry=CLM

t_nat_srv=CP

<ANTENNA>

t_pwr_xyz=Y

t_ant_dir=D

t_elev=-0.3

t_pwr_dbw=31.2

t_azm_max_e=295

t_pwr_eiv=I

t_hgt_agl=25

t_polar=V

t_gain_max=36

t_pwr_ant=-4.8

t_bmwdth=1.6

t_gain_type=I

<RX_STATION>

t_long=-0764218

t_lat=+034422

t_site_name=ATUNZUELA

t_geo_type=POINT

t_ctry=CLM

</RX_STATION>

</ANTENNA>

<ANTENNA>

t_pwr_xyz=Y

t_ant_dir=D

t_elev=-0.5

t_pwr_dbw=27

t_azm_max_e=263

t_pwr_eiv=I

t_hgt_agl=21

t_polar=V

t_gain_max=34

t_pwr_ant=-7

t_bmwdth=1.8

t_gain_type=I

<RX_STATION>

t_long=-0763244

t_lat=+032802

t_site_name=CALI

t_geo_type=POINT

t_ctry=CLM

</RX_STATION>

</ANTENNA>

<ANTENNA>

t_pwr_xyz=Y

t_ant_dir=D

t_elev=-0.7

t_pwr_dbw=27

t_azm_max_e=283

t_pwr_eiv=I

t_hgt_agl=17

t_polar=H

t_gain_max=34

t_pwr_ant=-7

t_bmwdth=1.8

t_gain_type=I

<RX_STATION>

t_long=-0762935

t_lat=+033417

t_site_name=YUMBO

t_geo_type=POINT

t_ctry=CLM

</RX_STATION>

</ANTENNA>

</NOTICE>

<TAIL>

t_num_notices=1

</TAIL>