SOLUTION: FXM 04: Maritime mobile Service (point-to-area)

Prepare an electronic notice for notifying to the BR, for the recording in the Master Register of frequency 217.9375 MHz assigned to a coast station open to public correspondence situated in Trinidad having a circular receiving area of a radius of 100 km.

Solution

- The purpose of the exercise is to notify a coast station for its recording in the Master Register. The notice type to use is for Maritime mobile land station is T12.

- The class of station for a coast station is FC; the preface to the BR IFIC provides a complete list of Class of stations in Chapter IV, Section 6.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list in Chapter IV, Section 7.

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

- The Frequency does not fall within the bands shared on equal basis with space services, the values for Antenna gain and Radiated power shall be notified in dipole reference.

- For coast station, call sign or station identification is mandatory. There is no validation on station identification. However, if call sign is notified then it shall be in conformity with the provisions of Article 19 and Appendix 42.

The completed notice in text format.