SOLUTION FXM07: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of the electronic notice file “FXM07__T11_with error.txt”. 

Purpose of exercise:  To validate and identify errors in Frequency assignment notice using TerRaNotices “Open file” and “Validate Notice” functionalities.

Solution

- TerRaNotices can be used to validate and identify errors in a Frequency assignment notice.

- Note that TerRaNotices will not do a full validation. However, errors such as missing or incorrect mandatory values are indicated.

- Open TerRaNotices and select “Open file” from the File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- Choose the file to be validated. For this exercise, we choose a file “FXM07__T11_with error.txt “

- Validate the file.

- We can see that the file contains 2 notices. And Notice 1 and Notice 2 have the same identifying elements.

- We can see that the 2 notices correspond to the same identifying elements but with different Antenna sections and receiving stations.

- In order to correct this, we note that;

- One notice can have multiple Antenna sections. Therefore, we can add the Antenna section of the 2nd notice to the 1st notice in order to have 1 notice instead of 2 notices.

- The Antenna section and Receiving station can be added in the same notice as many as needed. And the 2nd notice can be removed.

- After adding multiple Antenna (Operations), fill in the required items.

- After the required correction is done, Validate the complete notice.