BS 02 Solution: VHF digital sound broadcasting assignment T-DAB

Prepare an electronic notice file of frequency 106.5 MHz assigned to a digital sound broadcasting station T-DAB, for its recording in the Master Register.

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices  “New Notice”  functionality for Addition to the Master Register,

Solution

- The notice type to use for the notification of a digital sound broadcasting station in the VHF band is: T01

- Transmission system for T-DAB station is S1 and the bandwidth is in the range: 1536 – 1750 kHz

- The purpose of the exercise is to record in the Master Register, the relevant fragment is: Article 11

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours

Electronic notice file:

- Open TerRaNotices and select new file ( TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise, select the following from the New notice dialog box;  - Administration CLN from Administration dropdown button;

VHF/UHF broadcasting notices category T01 type of notice for VHF sound broadcasting station.

- You will get the T01 notice form shown below; then select Fragment  Article 11 and Action Add as requested by the exercise and start to fill the required data items:

Note that at the bottom left, the list of items to be filled are indicated. The red color boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list and the color of the box also becomes normal. For example, see the following slide, when the antenna site name is filled.

- Fill the remaining missing fields to complete the notice.

- Validate and save the text file containing the complete T01 notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.