SOLUTION: FXM 02: Fixed service (Point-to-Multipoint) in shared bands

Prepare an electronic notice of frequency 14.669 GHz, which falls within the bands shared on equal basis with the space services, used for the operation of two fixed links for its recording in the Master Register.

Solution

- The purpose of the exercise is to notify two fixed links for its recording in the Master Register.

- The notice type for fixed station is T11. The identifying elements for fixed station are:  Assign Frequency; Geographical coordinates; Designation of emission (bandwidth code and class of emission); Class of station Operating hours

Therefore, if an Administration submits a notification for each link, the identifying elements of those two links are identical, only the antenna characteristics will be different.

The processing system in the BR, allow administrations to change the submitted data at any time, for this, the system needs to identify the concerned notice (with the identifying elements) therefore, the first notice will be replaced by the second notice. 
Administrations need to submit a single notice and the two links are notified within the antenna characteristics (or Operations)

- The assigned frequency falls within the bands shared on equal basis with space services, the following fields are mandatory

  - Altitude of site above sea level

  - Height of Antenna above ground level

  - Elevation angle

  - Polarization

- The value of the Maximum antenna gain and radiated power shall be notified in isotropical values.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list of “Nature of services” in Chapter IV, Section 7.

- The class of station is a mandatory field and in this case it is FX; the preface to the BR IFIC provides a complete list of “class of station” in Chapter IV, Section 6

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : maximum 3 years in advance as the frequency falls within the bands shared with Space Services

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

- Electronic notice file:

- Open TerRaNotices and select new file ( TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise, select the following from the New notice dialog box;  - Administration MLA;  FXM notices category - T11  type of notice for Transmitting station in the fixed service .

- You will get the T11 notice form shown below; then select Provision RR11.2 and Notification intended for Addition as requested by the exercise and start to fill the required fields.  Note that the missing fields are listed at the bottom left of the screen. The red color of the boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list and the color of the box also becomes normal.  Fill the remaining missing fields to complete the notice.

- The complete Notice form is shown below with the following screens.

- Validate and save the text file containing the complete T11 notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.