BS05 Solution: AM Sound broadcasting station (in the tropical zone).

- Prepare an electronic notice file of frequency 5050 kHz assigned to an AM broadcasting station having a circular receiving area of 200 km, for its recording in the Master Register.

Purpose of exercise:  To adequately select the notice type, you may use the facility available in TerRaNotices under the tab “File/Wizard”.

Solution

- TerRaNotices provides a tool Wizard.

- The frequency assignment notice belongs to the FXM domain and it is a notice proposing addition of a frequency assignment notice to the Master register, therefore the relevant fragment is Article 11.

- Class of station is BC

- The notice type to use for requesting Addition of AM Sound Broadcasting Station: T12

Electronic notice file:

- Open Wizard and select “CTR” and  Update of MIFR

- Select ADD the assignment

- Fill the required fields as requested.

Class of station is BC   Frequency 1150 kHz

Note that the Notice type is determined as T12 by the Wizard. See the bottom of the screen.

- Then confirm the summary and generate the notice.

- Fill in all required data

- To finish the task, Validate and save the notice file in order to notify to ITU through WISFAT.