FXM 03 Solution: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

- Purpose of exercise:  To validate and identify errors in Frequency assignment notice using the online validation.

Solution

- To Validate and identify errors of a notice file, we will use the online validation tool which can be accessed from http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

Electronic notice file:

- Log on to the online validation tool.

http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

- Browse the file to be validated and upload to process. For this exercise, we choose a file “FXM 03_OnlineVal.txt “

- Once the process completed, we can see the following validation error messages as output;

Note that we also receive an email announcing the completion of validation.

- We can see the following error messages;

DeepVal Error - Unique Parameters would duplicate those in Assignment ID 118038076.

- Please check in TerRaQ database the BR assign IDs: 118038076.

- 118038076 is recorded in MIFR

- It is not possible t_action=ADD.

- The only possible action is MODIFY

DeepVal Error - Operation 2: Type of Radiated Power (I) should be (E) for non-shared bands.

- The notified frequency falls within a band that is not shared on equal basis with Space services.

- t_pwr_eiv = E (Effective radiated power)

4.  Correction:

After corrections and the validation message will be only warnings:

Warning(s)

DeepVal Warning - Attempt to modify unique ID of target. Current value is "GRIVITA_RADAR"; proposed value is "RADAR 1".