SOLUTION: FXM 03:  Land mobile service (point-to-area/area-to-point)

1/ Prepare an electronic notice file of frequency 959 MHz assigned to a base station having circular receiving area of a radius of 10 km, for its recording in the Master Register.

2/ Prepare an electronic notice file of frequency 914 MHz assigned to the associated receiving land mobile station (handset) of the above base station, for its recording in the Master Register.

Solution

- The purpose of the exercise is to create two notices in the land mobile service:

  - The first notice is to notify a base station at a given location and the receiving area is within a radius of 10 km of the base station. The notice type to use for this notification is a T12

  - The second notice is to notify the frequency assignment of the base station for reception from associated mobile stations. In this case the notice type is T13.

- The class of station is a mandatory field.

  - For the first notice the class of station is FB.

  - For the second it is ML.

  - The preface to the BR IFIC provides a complete list of Class of stations in Chapter IV, Section 6.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list in Chapter IV, Section 7.

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

- The Frequency does not fall within the bands shared on equal basis with space services, the values for Antenna gain and Radiated power shall be notified in dipole reference.

- TerRaNotices provides the possibility to create more than one notice within a notice file and it is highly recommended that administrations use this facility when notifying more than one notice per submission.

Electronic notice file

- Open TerRaNotices and select ‘New file’ from File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise,  select the following from the New notice dialog box;  Administration MDA;

FXM notices category T12 notice type for Transmitting Base station.

- You will get the T12 notice form shown below; then select provision RR11.2 and Action Add as requested by the exercise and start to fill the required data items.

- Additional notice can be added by choosing “Insert New Notice” from the File Menu. And select T13 notice type for Receiving Land station

- Now there are two notices in the file.

- Fill in the required fields as requested to complete the Notices.

- Validate and save the text file containing the complete T12 and T13 notices. The complete notices without error or warning are ready to be submitted to ITU via WISFAT.