SOLUTION: FXM 03:  Land mobile service (point-to-area/area-to-point)

1/ Prepare an electronic notice file of frequency 1 867.5 MHz assigned to a base station having circular receiving area of a radius of 25 km, for its recording in the Master Register.

2/ Prepare an electronic notice file of frequency 1 772.5 MHz assigned to the associated receiving land mobile station (handset) of the above base station, for its recording in the Master Register.

Solution

- The purpose of the exercise is to create two notices in the land mobile service:

  - The first notice is to notify a base station at a given location and the receiving area is within a radius of 25 km of the base station. The notice type to use for this notification is a T12

  - The second notice is to notify the frequency assignment of the base station for reception from associated mobile stations. In this case the notice type is T13.

- The class of station is a mandatory field.

  - For the first notice the class of station is FB.

  - For the second it is ML.

  - The preface to the BR IFIC provides a complete list of Class of stations in Chapter IV, Section 6.

- Nature of service is mandatory; the preface to the BR IFIC provides a complete list in Chapter IV, Section 7.

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours : starting time 00:00 – 23:59 (valid range for this data item)

  - Operating hours : end time 00:00 – 24:00 (valid range for this data item)

- The Frequency does not fall within the bands shared on equal basis with space services, the values for Antenna gain and Radiated power shall be notified in dipole reference.

- TerRaNotices provides the possibility to create more than one notice within a notice file and it is highly recommended that administrations use this facility when notifying more than one notice per submission.

Electronic notice file

- Open TerRaNotices and select ‘New file’ from File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise,  select the following from the New notice dialog box;  Administration EGY;

FXM notices category T12 notice type for Transmitting Base station.

- You will get the T12 notice form shown below; then select provision RR11.2 and Action Add as requested by the exercise and start to fill the required data items.

- Additional notice can be added by choosing “Insert New Notice” from the File Menu. And select T13 notice type for Receiving Land station

- Now there are two notices in the file.

- Fill in the required fields as requested to complete the Notices.

- Validate and save the text file containing the complete T12 and T13 notices. The complete notices without error or warning are ready to be submitted to ITU via WISFAT.