SOLUTION: FXM08: Validating the file with frequency assignment notices

Validate the electronic notice file “FXM 08_OnlineVal.txt” using the web online validation tool. This validation tool is accessible to all TIES users from the ITU web site at http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx.

Solution

- The purpose of the exercise is to validate the electronic notice file before submitting via WISFAT

- The notice file contains 4 T11 notices.

- The identifying elements for fixed station are:  Assign Frequency; Geographical coordinates; Designation of emission (bandwidth code and class of emission); Class of station Operating hours

Therefore, if an Administration submits a notification for each link, the identifying elements of those two links are identical, only the antenna characteristics will be different.

The processing system in the BR, allow administrations to change the submitted data at any time, for this, the system needs to identify the concerned notice (with the identifying elements) therefore, the first notice will be replaced by the second notice. 
Administrations need to submit a single notice and the two links are notified within the antenna characteristics (or Operations)

- Electronic notice file:

- Open Online Validation Tool and filled your details (ties account and password)

- The new session will opened for your user account.

- You have to select “New validation”

- Choose the file and Upload it

- The status of your task is shown below with the following screen.

- You will receive an email when your “job id” is completed

“Dear user 
Your online validation request for FXM 08_OnlineVal.txt (jobID = 30940) is completed.
The results of the validation is available on the online validation portal
For any clarification please send an email to the above address, giving your username, the jobID and the filename that requires clarification or you may send the file as an attachment to the above address. “

- When you back to the Validation history you can view results of validation

- In this test file, there are 4 T11 notices where 1st and 2nd notices have identical identifying elements. The differences are in the antenna section. For that reason only one T11 notice shall be submitted and this T11 notice must have with two antenna sections.

The same apply for 3rd and 4th notices.

Please note that it is highly recommended to submit your notice file/s without error.