BS 05: FM sound broadcasting assignment

Prepare an electronic notice file of frequency 96.1 MHz assigned to a sound broadcasting station, for modification of the GE84 plan..

Solution

To prepare this notice we will use the “Wizard” functionality of TerRaNotices and we will select Morocco (MRC) as the notifying administration.

- The ”Wizard” will identify the notice type to use, once the minimum required fields (Administration, Fragment, Station class, Action and Frequency) are entered.

- The completed notice in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Wizard “

- Then select  the notifying administration and choose option Update of Regional Plan , in this case GE84.

- Then specify the action as “ADD”.  as requested.

Note: the Wizard identifies the appropriate Notice type at the bottom of the screen.

- Confirm to open the identified notice type.

- Fill in the required fields as requested.

- To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

- Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)