EXAMPLE

UHF digital television broadcasting assignment

Prepare an electronic notice file of frequency 755.0 MHz assigned to a digital television broadcasting station, for its recording in the Master Register.

To prepare this notice we will use the “New File” functionality of TerRaNotices and we will select the administration of Japan (J) as the notifying administration.

| Item No. | Parameter | Required value |
| --- | --- | --- |
| 4A | Transmitting antenna site name | YOKOSUKA |
| 4C | Coordinates of the transmitting antenna site | 139°38'57"E - 35°13'22"N |
| 9E | Height of the Antenna above ground level | 24 m |
| 9D | Polarization | Vertical |
| 8B | Effective radiated power | 30.0 dBW |
| 7C1 | Transmission system | T9 |
| 1EO | Offset (kHz) | 142.857 |
| 9 | Antenna Directivity | Non Directional |
| 9EC | Maximum Effective Antenna Height and Effective antenna heights (m) at 36 different azimuths in 10 degrees interval | To be calculated using TerRaNotices facility |
| 2C | Date of bringing the frequency assignment into use | 23 January 2009 |
| 12B | Address code | Preface to the BR IFIC |
| 10B | Operating Hours | 24 Hours |