FXM 03 Solution: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

- Purpose of exercise:  To validate and identify errors in Frequency assignment notice using the online validation.

Solution

- To Validate and identify errors of a notice file, we will use the online validation tool which can be accessed from http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

Electronic notice file:

- Log on to the online validation tool.

http://www.itu.int/ITU-R/terrestrial/OnlineValidation/Login.aspx

- Browse the file to be validated and upload to process. For this exercise, we choose a file “FXM 03_OnlineVal.txt “

- Once the process completed, we can see the following validation error messages as output;

Note that we also receive an email announcing the completion of validation.

- We can see the following error messages;

DeepVal Warning - This notice seems to be an amendment to the notice 118097300 from the TIP (due to Identifying Elements).

DeepVal Error - Unique Parameters would duplicate those in Assignment ID 113266560.

- Please check in TerRaQ database the BR assign IDs: 118097300 and 113266560.

- 118097300 is a MODIFY notice with target 113266560 (recorded in MIFR)

- It is not possible t_action=ADD.

- The only possible action is MODIFY

DeepVal Warning - Operation 1: Type of Antenna Gain (I) should be D.

DeepVal Warning - Operation 2: Type of Antenna Gain (I) should be D.

DeepVal Error - Operation 1: Type of Radiated Power (I) should be (E) for non-shared bands.

DeepVal Error - Operation 2: Type of Radiated Power (I) should be (E) for non-shared bands.

- The notified frequency falls within a band that is not shared on equal basis with Space services.

- t_gain_type = D (Gain relative to a half wave dipole)

- t_pwr_eiv = E (Effective radiated power)

4.  Correction:

After corrections and the validation message will be only warnings:

Warning(s)

DeepVal Warning - This notice seems to be an amendment to the notice 118097300 from the TIP (due to Identifying Elements).

DeepVal Warning - Attempt to modify unique ID of target. Current value is "936165"; proposed value is ""

That means that:

The new notice will replace the previous submitted Modification request (118097300)

In the new notice, there is not notified Unique Identification code of Administration. Therefore, the previous submitted “936165” will be removed from database.