GE06 01: 2L assignments

Digital television broadcasting for SFN composed Linked assignments

Prepare an electronic notice file of frequency 570 MHz assigned to two TV broadcasting stations for their recording in GE06 Plan based on the information below:

To prepare these notices we will use “New File” functionality of TerRaNotices, select the Administration of Switzerland (SUI) as the notifying administration and the relevant notice type.

Solution

The notice type to use for the notification of a television broadcasting station assignment in the GE06 band is: GT1

The identifying elements for broadcasting station are:

  - Unique Identification Code given by the administration

  - Assign Frequency

  - Geographical coordinates

- Open TerRaNotices , This software can be found in the BRIFIC-DVD select “New Notice “; then GT1 notice type for VHF/UHF broadcasting

- Open the GT1 notice form shown below. Then Select Fragment “GE06D”  and intention “Addition” as requested;

Note that at the bottom left, the list of items to be filled are indicated. The red color boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list and the color of the box also becomes normal.

- Fill the remaining missing fields to complete the notice.

- Altitude of site above sea level, Effective Antenna Heights and Maximum effective antenna height can be calculated using the SRTM3 Terrain Database from Tools menu.

- Additional notice can be added by choosing “Insert New Notice” from the File Menu. And select GT1

- Now there are two notices in the file.

- Fill in the required fields as requested to complete the Notices.

- Again calculate the Altitude of site above sea level, Effective Antenna Heights and Maximum effective antenna height can be calculated using the SRTM3 Terrain Database from Tools menu.

- Once the required fields are completed, validate the notice. Then save file with no error/warning which is ready to be submitted via Wisfat.