GE06 02: 3C converted assignment

Assignment stemming (converted) from an allotment

Prepare an electronic notice of frequency 634 MHz assigned to a digital television broadcasting station stemming from an allotment based on the information below, for the modification of the GE06D Plan.

To prepare this notice we will use “Wizard” functionality of TerRaNotices for the Administration of Russia (RUS).

Solution

- The notice type to use for the notification of a television broadcasting station in the UHF band in the GE06 band is:  GT1

- To create a frequency assignment notice using TerRaNotices “Wizard” facility

- Open TerRaNotices and select “ Wizard “  from File Menu

- Specify the Fragment and continue to choose the required action.

- Choose the class of station as “BT” for Television Broadcasting. Enter the Frequency, and specify the Geographical area, and the system as Normal digital TV.

Note that the wizard has selected the appropriate Notice type, at the bottom of the screen.

Then press Finish and confirm the summary to open the GT1 notice.

- Fill in the required fields as requested.

- Altitude of site above sea level, Effective Antenna Heights and Maximum effective antenna height can be calculated using the SRTM3 Terrain Database from Tools menu.

- Once the required fields are completed, validate the notice. Then save file with no error/warning which is ready to be submitted via Wisfat. (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)