GE06 04:  Notification of assignment for recording in the Master Register (MIFR)

Prepare an electronic notice to notify a digital sound broadcasting assignment linked to an allotment. The assigned frequency is 229.072 MHz with the following characteristics for recording in the Master Register (MIFR).

Solution

To prepare this notice we will use “”New File” functionality of TerRaNotices and select the Administration of United Kingdom (G) as notifying administration.

- The completed notice in text format.

The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “New File “

then select SUI as notifying administration. - Type of notice GT1 “ Digital sound broadcasting (T-DAB) assignment in the frequency and geographical scope of GE06”

- Proceed to open the GS1 notice type.

- Fill in the required fields as requested.

- To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

- Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat (Web Interface for Submission of Frequency Assignments/allotments for Terrestrial services)