BS 01: FM sound broadcasting assignment

Prepare an electronic notice of frequency 98.50 MHz assigned to a sound broadcasting station based on the information below, for its recording in the Plan GE84.

Solution

To prepare this notice we will use the “Wizard” functionality of TerRaNotices and we will select Seychelles (SEY) as the notifying administration.

- The ”Wizard” will identify the notice type to use, once the minimum required fields (Administration, Fragment, and Action) are entered.

- The purpose of the exercise is to modify the GE84 plan: the relevant fragment is: GE84.

- The tool available in TerRaNotices “Calculate effective antenna heights using SRTM3 Terrain Database” will provide the “Altitude of site above sea level” the “Effective antenna heights” and the “Maximum effective antenna height” based on the coordinates of the transmitting site and the height of the antenna above the ground level”

- For information : the table here below describes the analog sound transmission systems (Reference: Chapter 3: “Technical standards and transmission characteristics for sound broadcasting service” of GE84 Agreement)

| #### Analogue transmission systems | Necessary bandwidth | System description (GE84) |
| --- | --- | --- |
| 1 | 180 - 200 kHz | Monophonic (max. frequency deviation ± 75kHz) |
| 2 | 130 – 180 kHz | Monophonic (max. frequency deviation ± 50kHz) |
| 3 | 180 - 300 kHz | Stereophonic, polar modulation system (max. frequency deviation ± 50kHz) |
| 4 | 200 - 310 kHz | Stereophonic, pilot-tone system (max. frequency deviation ± 75kHz) |
| 5 | 200 - 300 kHz | Stereophonic, pilot-tone system (max. frequency deviation ± 50kHz) |

- The completed notice in text format.

- The next pages provide screen shots from TerRaNotices

- Open TerRaNotices and select “Wizard “

Then select SEY as notifying administration, and “Update to one of the world regional plans” then select “GE84” from Plans option.

- Then select Action ADD.

Note that a Notice type T01 is identified by the wizard from the candidate notice types at the bottom of the screen.

And Finish to generate the identified T01 Notice type.

- Open the T01 notice type. Note that the required fields are marked in red. And are indicated at the bottom of the screen.

- Fill in the required fields as requested.

- To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Once the required fields are completed, validate the notice and save file which is ready to be submitted via Wisfat.