BS 02 Solution: Modification of existing assignment

Prepare an electronic notice of to modify existing frequency 103.0 MHz to 99.1 MHz assigned to a sound broadcasting station.

Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “Open a notice from the database” functionality to modify an assignment.

Solution

Electronic notice file:

- Open TerRaNotices and select “Open a notice from the DB” option from File menu ( TerRaNotices software can be installed from BRIFIC DVD)

- According to the requirement of the exercise, we choose the following items:

Notifying administration: SEY Domain: FMTV   Fragment: GE84  Recorded assignment and Put the target Frequency to be modified.

- After executing, the required Frequency assignment from the Database will be displayed.

- Change the Notification intention to “Modification”. You will have a dialog box requesting the Target information. You have a choice to enter Assigned Frequency and Geographical coordinates or a Unique identifier. For this exercise, we can enter the given Frequency and Geographical coordinates “

Please note by pressing the button “ID” at the bottom left side of the dialog box, the required target identifier fields will be filled automatically.

- Then modify the required item. For this exercise the Assigned frequency is to be modified to “99.1”

- Then Validate and save the Modification notice in order to notify to ITU through WISFAT.