BS 04 Solution: UHF digital television broadcasting assignment

Prepare an electronic notice file of frequency 642 MHz assigned to a TV broadcasting station, for its recording in the GE06D plan.

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices “Wizard” functionality for recording in the GE06D plan

Solution

- The notice type to use for the notification of a digital television broadcasting station in the UHF band is:  GT1

Electronic notice file:

- Open TerRaNotices and Select “Wizard” from File menu.

The Wizard functionality of TerRaNotices, guides you to select the notice type to use according to the given criteria.

For this exercise, we select SEY as notifying Administration, and “Update to one of the world or regional plans” and “GE06D” as requested by the exercise.

- Next we specify the required action as “ADD”.  Note the candidate notice types at the bottom for the specified criteria at each stage.

- The Class of station shall be selected from the “Notice’s Parameter” menu as “BT” for the Broadcasting television station. (Please refer to the PREFACE to BRFIC  which can be found within the BRIFIC DVD)

And Enter the frequency as requested by the exercise.

- Note that the Notice type selected by the wizard for this requirement is “GT1”

- Then confirm the summary to generate the required Notice form.

- Fill all the required fields to complete the notice.

- To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Validate and save the text file containing the complete notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.