BS 05 Solution: VHF digital sound broadcasting assignment

Prepare an electronic notice file of frequency 222.064 MHz assigned to a sound broadcasting station, for its recording in the Master Register.

- Purpose of exercise:  To create a frequency assignment notice using TerRaNotices  “New Notice”  functionality for Addition to the Master Register,

Solution

- The notice type to use for the notification of an analog sound broadcasting station in the VHF band is: GS1

- The purpose of the exercise is to record in the Master Register, the relevant fragment is: Article 11

- The following fields are mandatory for all notifications under Article 11.

  - Address code : Preface to the BR IFIC, Chapter IV, Section 3

  - Date of bringing into use : not earlier than 3 months

  - Operating hours

Electronic notice file:

- Open TerRaNotices and select new file ( TerRaNotices software can be installed from BRIFIC DVD)

- For this exercise,  select the following from the New notice dialog box;  - Administration SEY from Administration dropdown button;

VHF/UHF broadcasting notices category, GS1 type of notice for VHF sound broadcasting station.

- You will get the GS1 notice form shown below; then select Fragment  Article 11 and Action Add as requested by the exercise and start to fill the required data items:

Note that at the bottom left, the list of items to be filled are indicated. The red color boxes also indicate the missing fields. As the field is correctly filled the item disappears from the list and the color of the box also becomes normal.

- Fill the remaining missing fields to complete the notice.

- To calculate the effective antenna height (m) at 36 different azimuths, open Tools and select the option “Calculate Effective antenna heights (eff_hgt) using the SRMT3 Terrain database.

Then enter the Geographical coordinates and the height of antenna above ground level (m) and submit. The values of effective antenna height (m) at 36 different azimuths in 10 degrees interval will be filled automatically.

- Validate and save the text file containing the complete notice.

- The complete notice without error or warning is ready to be submitted to ITU via WISFAT.