BS 07 Solution: Request of a recorded plan assignment which has been put into operation, to be recorded in the Master Register with all technical characteristics exactly the same as in the plan.

Prepare an electronic notice for an assignment recorded in the plan which has been put into operation, of a frequency assignment from the GE84 Plan, in order to record to the Master Register (MIFR) with all technical characteristics as in the plan.

Solution

- To prepare this notice we will use the ““Generate TB notices”” functionality of TerRaNotices and we will select Seychelles (SEY) as the notifying administration.

- The frequency assignment notice belongs to the FMTV domain, and the fragment is GE84.

- The identifying elements of a BS assignment is either:

  - Frequency and geographical coordinates, or

  - Unique identification code of the assignment given by the administration, if this item is notified previously and recorded with the Frequency assignment to be modified.

- For this exercise we have selected the notice having the given identifying elements (Frequency and Geographical coordinates)

Electronic notice file:

- Open TerRaNotices and select “Generate TB notices “  ( TerRaNotices software can be installed from BRIFIC DVD)

- Then select the Notice type “ TB2 “ for the notification under article 11 of an assignment with all technical characteristics as in the plan.

- According to the requirement of the exercise, we choose the following items:

Notifying administration: SEY Domain: FMTV   Fragment: GE84 Recorded assignment

- Then enter: the given frequency as requested by the exercise.

- After executing, the required Frequency assignment from the Database will be displayed. Make sure the Geographical coordinates are as requested.

- Then generate the required TB2 notice. By entering the mandatory items for notification of Article 11 notices.

Note that a link is provided for the list of Address codes, and operating agency codes on record concerning all Administrations. Please check the list concerning your Administration and for all possible addition or modification contact the Bureau.

Note that for this exercise, values are assumed;

- Once the required data items are entered execute to generate the TB2 notice

- Then Validate and save the TB2 notice in order to notify to ITU through WISFAT.