BS 06 Solution: Validating and identifying errors of a Frequency assignment notice.

Validate and identify the errors of an electronic notice file which was previously prepared and saved.

- Purpose of exercise:  To validate and identify errors in Frequency assignment notice using TerRaNotices “Open file” and “Validate Notice” functionalities.

Solution

- TerRaNotices can be used to validate and identify errors in a Frequency assignment notice.

- Note that TerRaNotices will not do a full validation. However, errors such as missing or incorrect mandatory values are indicated.

Electronic notice file:

- Open TerRaNotices and select “Open file” from the Tools menu ( TerRaNotices software can be installed from BRIFIC DVD)

- Choose the file to be validated. For this exercise, we choose a file “BS 06_NoticeWithError.txt  “

- We can see the following errors messages;

  - The “Necessary Bandwidth” is missing.

4.2 The Effective radiated power of the vertically polarized component is missing. While the Effective radiated power is given for the horizontal component which is indicated as a warning.

Correction:

4.1 We can put 200 kHz for the necessary Bandwidth,

4.2 We have a choice to correct the polarization as H to align with the effective radiated power in the horizontal direction, or to provide the effective radiated power of the vertically polarized component to align with Polarization given as Vertical.

For this exercise, the effective radiated power of vertically polarized component is considered as Polarization is V. And the value 23 dbW is entered.