# Update on Wireless Power Transfer (Transmission) Interference Issues to T-Coil systems

## Bus Systems

Further to my last report practical testing with an electric bus at Milton Keynes has been carried out thanks to Ofcom UK.

The tests where extensive and carried out early on a Sunday morning in good weather conditions. The spectrum used by this bus system is only one of those which can be used but can be considered as representative.

Ofcom is producing a detailed report and I will distribute when available to JCA-AHF and T-Coil manufacturers.

The bus is required to drive into a space only centimetre’s wider than the bus with the bus being guided by kerbs on either side of the wheels. Under these physical restraints it is almost impossible for the charging source and the charging plate on the bus to be misaligned.

Three manufacture’s supplied  equipment containing T-Coil  receivers where tested, whilst there where  clicks in the hearing aids  when the bus charging plate was lowered onto the fixed power supply plate in the road, no major interference was experienced. An additional safeguard is that no members of the public (including during tests) are allowed to stay on the bus during the charging sequence.

## Other WPT Equipment

To date I haven unable to arrange any other testing, however CEPT is discussing the issue within the next month and is likely to generate a new work item to generate a comprehensive report on the subject. I would hope to be involved in this work

http://www.cept.org/