| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 2 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 574 Rev.1 |
|  |  | English only Original: English |
| Question(s): | 7/2 | Geneva, 18-27 March 2015 |
| TD |  |  |
| Source: | Editors |  |
| Title: | Draft Recommendation M.3170.4: Multi-technology network management - Conformance testing specification (for consent) |  |

Summary

ITU-T M.3170 series of Recommendations provide the Multi-technology network management interface. This Recommendation specifies the conformance testing specifications associated with it,  and provides the principals for the conformance testing of MTNM interface, the testing methodology that will be used for the conformance testing of the MTNM interface, the scope and contents that will be tested for the ITU-T M.3170 series of Recommendations, and the ICS proforma definitions for the MTNM IDL interface.

Keywords

MTNM interface, Conformance testing, ICS.

Table of Contents

1	Scope	3

2	References	3

3	Definitions	3

3.1	Terms defined elsewhere	3

3.2	Terms defined in this Recommendation	4

4	Abbreviations and acronyms	4

5	Conventions	4

6	Overview	4

7	Principals for MTNM interface conformance testing	4

8	Testing Methodology for MTNM interface conformance testing	5

9	The scope for content for conformance testing	5

Annex A ICS Proforma definitions for Notification Service Interface	6

Annex B ICS Proforma definitions for Trading Service Interface	7

Annex C ICS Proforma definitions for MTNM specific Interface	8

1	Scope

The purpose of this Recommendation is to provide the conformance testing specification for the MTNM interface specified in the Recommendations ITU-T M.3170 series.

It is within the scope of this Recommendation to specify: the principals for the conformance testing of MTNM interface, the testing methodology that will be used for the conformance testing of the MTNM interface, the scope and the contents that will be tested for the conformance of  ITU-T M.3170 series of Recommendations, and the ICS proforma definitions for the MTNM IDL interface.

2	References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

[ITU-T X.296 ]	Recommendation X.296 (1995), OSI conformance testing methodology and framework for protocol Recommendations for ITU-T applications – Implementation conformance statements.

[ITU-T X.781]		Recommendation ITU-T X.781 (2001) Requirements and guidelines for Implementation Conformance Statements proformas associated with CORBA-based systems.

[ITU-T M.3170.0]	Recommendation ITU-T M.3170.0 (2007), Multi-technology network management – Introduction and supporting documentation.

[ITU-T M.3170.0 Amd1]	Recommendation ITU-T M.3170.0  Amendment1 (2015), Upgrade to MTNM Release 3.5.

[ITU-T M.3170.1]	Recommendation ITU-T M.3170.0 (2007), Multi-technology network management –Business agreement (TMF513).

[ITU-T M.3170.1 Amd1]	Recommendation ITU-T M.3170.1  Amendment1 (2015), Upgrade to MTNM Release 3.5.

[ITU-T M.3170.2]	Recommendation ITU-T M.3170.0 (2007), Multi-technology network management – Information agreement (TMF608).

[ITU-T M.3170.2 Amd1]	Recommendation ITU-T M.3170.2  Amendment1 (2015), Upgrade to MTNM Release 3.5.

[ITU-T M.3170.3]	Recommendation ITU-T M.3170.3 (2007), Multi-technology network management: CORBA IDL solution set (TMF814) with implementation statement templates and guidelines (TMF814A).

[ITU-T M.3170.3 Amd1]	Recommendation ITU-T M.3170.3  Amendment1 (2015), Upgrade to MTNM Release 3.5.

[OMG NSS]		OMG Specification (2002), Notification Services Specificaiton (Version 1.0.1).

[OMG COSS]		OMG Specification (1998), CORBA Services : Common Object Services Specificaiton.

3	Definitions

3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	Implementation Confrormance Statements (ICS) [ITU-T X.296]

3.1.2	ICS Proforma [ITU-T X.296]

3.1.3	Multi-Technology Network Management (MTNM) [ITU-T M.3170.1]

3.2	Terms defined in this Recommendation

This Recommendation has no new terms or definitions.

4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

CORBA	Common Object Request Broker Architecture

ICS	Implementation Conformance Statement

IDL	Interface Definitions Language

MTNM	Multi-Technology Network Management

5	Conventions

This Recommendation follows the conventions defined in [ITU-T M.781].

The following common notations, defined in [ITU-T X.296] are used for the "status" value column in this Recommendation:

m	Mandatory

o	Optional

-	Not applicable or out of scope

6	Overview

MTNM interface is the multi-technology network management interface used to manage transport networks, and it is defined in ITU-T M.3170 series of Recommendations. This Recommendation provides the specification for MTNM interface conformance testing.

In order to test the conformance of an implementation, it is useful to specify the principals and the testing methodology and the contents for interface conformance testing. The implementation conformance statement is also a very important part of the testing specification.

Clause 7 specifies the principals for the conformance testing of MTNM interface; clause 8 specifies the testing methodology that will be used for the conformance testing for the MTNM interface; clause 9 specifies the scope and the contents that will be tested for M.3170 series; and Annex A, B and C provides the ICS proforma definitions for the MTNM IDL interfaces.

7	Principals for MTNM interface conformance testing

The principals for the MTNM interface conformance testing is according the approach defined in X.290 series of Recommendations.

An implementation that claims to be conformant to the ITU-T M.3170 series of Recommendation should be tested for conformance. M.3170 series of Recommendation provides a network management interface that are used to manage transport networks. [ITU-T M.3170.0] prvovides the introduction and support documentation for the whole M.3170 series of Recommendations. [ITU-T M.3170.1] provides the interface requirements, [ITU-T M.3170.2] provides the technology-neutral information model, which can meet the requirements specified in [ITU-T M.3170.1], and [ITU-T M.3170.3] provides the CORBA IDL interface design (solution set), which is protocol specific, and it conveys all the information specified in [ITU-T M.3170.2].

The conformance testing should test both the interface syntax in IDL implementation, as well as the semantic information that are carried by CORBA interactions. Within this Recommendation, the ICS proformas that will be used for conformance testing is mainly against [ITU-T M.3170.3], as it is the only implementable interface specification in M.3170 series of Recommendations.

8	Testing methodology for MTNM interface conformance testing

Clause 5.2 in Recommendation [ITU-T X.781] provides a general testing methodology of conformance testing for CORBA-based interface implementations. This methodology is also applicable for the MTNM interface specified in [ITU-T M.3170.3]. This Recommendation follows the testing methodology specified in [ITU-T X.781], and mainly provides the ICS proforma definitions for the IDL interfaces defined in M.3170.3. All the other related information and definitions regarding the test procedures and test concepts can be found in [ITU-T X.296] and [ITU-T X.781].

9	Scope and contents for the MTMN interface conformance testing

9.1	Scope for the MTMN interface conformance testing

In Recommendation [ITU-T M.3170.3], the detailed CORBA interface definition for the MTNM solution set is provided.

The scope of the conformance testing for the MTNM interface includes all the IDL interfaces and data types defined in [ITU-T M.3170.3] (i.e., TMF814). As the MTNM interface also uses two OMG CORBA services to support its functionality, which are “Notification Service” (specified in [OMG NSS]) and “Trading Service” (specified in [OMG COSS]), these two services are also within the scope for the CORBA interface, although it might be an implementation from a third party.

9.2	Contents for the MTMN interface conformance testing

9.2.1	Contents for the conformance testing of interfaces in Notification Services

For the Notification Service specified in [OMG NSS], all the interfaces specified in Table 1 shall be tested for conformance within the scope of this Recommendation.

All the operations specified in these interfaces shall be tested.

Table 1:  List of interfaces to be tested for CORBA Notification Service

| Number | Interface Name |
| --- | --- |
|  | CosEventChannelAdmin::ConsumerAdmin |
|  | CosEventChannelAdmin::EventChannel |
|  | CosEventChannelAdmin::ProxyPullConsumer |
|  | CosEventChannelAdmin::ProxyPullSupplier |
|  | CosEventChannelAdmin::ProxyPushConsumer |
|  | CosEventChannelAdmin::ProxyPushSupplier |
|  | CosEventChannelAdmin::SupplierAdmin |
|  | CosEventComm::PullConsumer |
|  | CosEventComm::PullSupplier |
|  | CosEventComm::PushConsumer |
|  | CosEventComm::PushSupplier |
|  | CosNotification::AdminPropertiesAdmin |
|  | CosNotification::QoSAdmin |
|  | CosNotifyChannelAdmin::ConsumerAdmin |
|  | CosNotifyChannelAdmin::EventChannel |
|  | CosNotifyChannelAdmin::EventChannelFactory |
|  | CosNotifyChannelAdmin::ProxyConsumer |
|  | CosNotifyChannelAdmin::ProxyPullConsumer |
|  | CosNotifyChannelAdmin::ProxyPullSupplier |
|  | CosNotifyChannelAdmin::ProxyPushConsumer |
|  | CosNotifyChannelAdmin::ProxyPushSupplier |
|  | CosNotifyChannelAdmin::ProxySupplier |
|  | CosNotifyChannelAdmin::SequenceProxyPullConsumer |
|  | CosNotifyChannelAdmin::SequenceProxyPullSupplier |
|  | CosNotifyChannelAdmin::SequenceProxyPushConsumer |
|  | CosNotifyChannelAdmin::SequenceProxyPushSupplier |
|  | CosNotifyChannelAdmin::StructuredProxyPullConsumer |
|  | CosNotifyChannelAdmin::StructuredProxyPullSupplier |
|  | CosNotifyChannelAdmin::StructuredProxyPushConsumer |
|  | CosNotifyChannelAdmin::StructuredProxyPushSupplier |
|  | CosNotifyChannelAdmin::SupplierAdmin |
|  | CosNotifyComm::NotifyPublish |
|  | CosNotifyComm::NotifySubscribe |
|  | CosNotifyComm::PullConsumer |
|  | CosNotifyComm::PullSupplier |
|  | CosNotifyComm::PushConsumer |
|  | CosNotifyComm::PushSupplier |
|  | CosNotifyComm::SequencePullConsumer |
|  | CosNotifyComm::SequencePullSupplier |
|  | CosNotifyComm::SequencePushConsumer |
|  | CosNotifyComm::SequencePushSupplier |
|  | CosNotifyComm::StructuredPullConsumer |
|  | CosNotifyComm::StructuredPullSupplier |
|  | CosNotifyComm::StructuredPushConsumer |
|  | CosNotifyComm::StructuredPushSupplier |
|  | CosNotifyFilter::Filter |
|  | CosNotifyFilter::FilterAdmin |
|  | CosNotifyFilter::FilterFactory |
|  | CosNotifyFilter::MappingFilter |

9.2.2	Contents for the conformance testing of interfaces in Trading Services

For the Trading Service specified in [OMG COSS], all the interfaces specified in Table 2 shall be tested for conformance within the scope of this Recommendation.

All the operations specified in these interfaces shall be tested.

Table 2:  List of interfaces to be tested for CORBA Trading Service

| Number | Interface Name |
| --- | --- |
|  | CosTrading::Admin |
|  | CosTrading::ImportAttributes |
|  | CosTrading::Link |
|  | CosTrading::LinkAttributes |
|  | CosTrading::Lookup |
|  | CosTrading::OfferIdIterator |
|  | CosTrading::OfferIterator |
|  | CosTrading::Proxy |
|  | CosTrading::Register |
|  | CosTrading::SupportAttributes |
|  | CosTrading::TraderComponents |

Note: The IDL interfaces listed in Table 2 are only a subset of all the IDL interfaces defined in  [OMG COSS] for Trading Service, as only the above interfaces (in Appendix A.1 of Trading Service Specification) will be used in the MTNM interface.

9.2.3	Contents for the conformance testing of MTNM specific interfaces

For the MTNM specific interfaces defined [ITU-T M.3170.3], all the interfaces specified in Table 3 shall be tested for conformance within the scope of this Recommendation.

All the operations specified in these interfaces shall be tested.

Table 3:  List of interfaces to be tested for MTNM specific Service

| Number | Interface Name |
| --- | --- |
|  | ELLink::ELLinkMgr_I |
|  | ELLink::FDIterator_I |
|  | EthernetServiceMgr::EthernetServiceMgr_I |
|  | LCAS::LCAS_I |
|  | PathBinding::PathBindingIterator_I |
|  | PathBinding::PathBinding_I |
|  | aSAP::ASAPIterator_I |
|  | alarmMgr::AlarmMgr_I |
|  | callSNC::CallAndTopLevelConnectionsAndSNCsIterator_I |
|  | callSNC::CallAndTopLevelConnectionsIterator_I |
|  | common::Common_I |
|  | emsMgr::EMSMgr_I |
|  | emsSession::EmsSession_I |
|  | emsSessionFactory::EmsSessionFactory_I |
|  | equipment::EquipmentInventoryMgr_I |
|  | equipment::EquipmentOrHolderIterator_I |
|  | ethernetLayer2Manager::VlanMgr_I |
|  | flowDomain::FDIterator_I |
|  | flowDomain::FlowDomainMgr_I |
|  | flowDomain::MFDIterator_I |
|  | flowDomainFragment::FDFrIterator_I |
|  | flowDomainFragment::MFDFrIterator_I |
|  | globaldefs::NamingAttributesIterator_I |
|  | guiCutThrough::GuiCutThroughMgr_I |
|  | mLSNPP::MLSNPPIterator_I |
|  | mLSNPP::MLSNPPMgr_I |
|  | mLSNPPLink::MLSNPPLinkIterator_I |
|  | mLSNPPLink::MLSNPPLinkMgr_I |
|  | maintenanceOps::CurrentMaintenanceOperationIterator_I |
|  | maintenanceOps::MaintenanceMgr_I |
|  | managedElement::ManagedElementIterator_I |
|  | managedElementManager::ManagedElementMgr_I |
|  | mtnmVersion::Version_I |
|  | multiLayerSubnetwork::MultiLayerSubnetworkMgr_I |
|  | multiLayerSubnetwork::SubnetworkIterator_I |
|  | nmsSession::NmsSession_I |
|  | notifications::EventIterator_I |
|  | performance::JobIterator_I |
|  | performance::PMDataIterator_I |
|  | performance::PMPIterator_I |
|  | performance::PerformanceManagementExtMgr_I |
|  | performance::PerformanceManagementMgr_I |
|  | performance::TCAParameterProfileIterator_I |
|  | protection::EProtectionGroupIterator_I |
|  | protection::OSNCPProtectMgr_I |
|  | protection::ProtectionGroupIterator_I |
|  | protection::ProtectionMgr_I |
|  | session::Session_I |
|  | softwareAndDataManager::BackupIdIterator_I |
|  | softwareAndDataManager::SoftwareAndDataMgr_I |
|  | subnetworkConnection::CCIterator_I |
|  | subnetworkConnection::SNCIterator_I |
|  | terminationPoint::GTPiterator_I |
|  | terminationPoint::TerminationPointIterator_I |
|  | topologicalLink::TopologicalLinkIterator_I |
|  | trafficConditioningProfile::TCProfileIterator_I |
|  | trafficConditioningProfile::TCProfileMgr_I |
|  | trafficDescriptor::TrafficDescriptorIterator_I |
|  | trafficDescriptor::TrafficDescriptorMgr_I |
|  | transmissionDescriptor::TransmissionDescriptorIterator_I |
|  | transmissionDescriptor::TransmissionDescriptorMgr_I |

10	Summary of the ICS proformas for MTNM interface conformance testing

ICS is an implementation conformance statement completed by an implementer who claims conformance to a standard. An ICS proforma is a document containing all the questions for the conformance claims to be answered by an implementer, and the ICS proforma covers all the items to be claimed for conformance. When an ICS proforma is filled in by an implementer, it becomes an ICS. The ICS can also be used for conformance testing, as it contains all the items for conformance testing, and the test cases should be derived from the ICS for conformance testing. ICS proformas can be used as the original testing record during the process of the whole interface conformance testing.

Usually an ICS can be generated by tools for a specific interface. The ICS proformas in Annex A, B and C are all generated by a tool according to the template specified in [ITU-T X.781].

In Annex A, the ICS proforma definitions for the CORBA Notification Service interface are provided; in Annex B, the ICS proforma definitions for the CORBA Trading Service interface are provided; in Annex C, the ICS proforma definitions for the MTNM specific interface are provided.

_________________