| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 11 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 857 (GEN/11) |
|  |  | English only Original: English |
| Question(s): | 15/11 | Geneva, 2-11 December 2015 |
| TD |  |  |
| Source: | Editor |  |
| Title: | Output – Draft new Recommendation ITU-T Q.TM_Int_sp_test “Testing methodologies of internet speed measurement system to be used on the fixed and mobile networks” (Vienna, 8-10 September 2015) |  |

Notes

This document is an output document of the work item ITU-T Q.TM_Int_sp_test “Testing methodologies of internet speed measurement system to be used on the fixed and mobile networks”. It’s based on the text of the working document WD4  and WD5 which have been submitted to the SG11 for the joint Rapporteur’s group meeting of ITU-T Q15/11 and ETSI TC INT meeting of  9 September (https://www.itu.int/ifa/t/2013/sg11/exchange/wp4/q15/2015-09-Vienna/) to follow up on the decision to split the previous work item Q.Int_speed_test “Unified methodology of Internet speed quality measurement usable by end-users on the fixed and mobile networks” (TD739 GEN/11, SG11 meeting of 22-29 April 2015) into two work items:  Q.FW_Int_sp_test for the framework definition and Q.TM_Int_sp_test for the testing methodology (as referred in WD2 submitted to the joint Rapporteur’s group meeting of ITU-T Q15/11 and ETSI TC INT meeting of  9 September).

|  | INTERNATIONAL TELECOMMUNICATION UNION |
| --- | --- |

ITU-Т	Series Q

STANDARDIZATION SECTOR	(mm/yyyy)

SERIES Q: SWITCHING AND SIGNALLING

Recommendation ITU Q.TM_Int_sp_test

Testing methodologies of internet speed measurement system to be used on the fixed and mobile networks

Content

1.	Scope	4

2.	References	4

3.	Definitions	4

4.	Abbreviations	4

5.	Conventions	4

6.	Test methodology	5

6.1.	General requirements	5

6.2.	System components	5

6.3.	Initial conditions	6

6.4.	Measurement procedure	6

6.4.1.	Phase 1: Initialization	7

6.4.2.	Phase 2: Downlink pre-test	7

6.4.3.	Phase 3: Latency test	8

6.4.4.	Phase 4: Downlink subtest	9

6.4.5.	Phase 5: Uplink pre-test	10

6.4.6.	Phase 6: Uplink subtest	10

6.4.7.	Phase 7: Finalization	11

6.4.8.	Measurement of Data	11

7.	Additional considerations	11

8.	Bibliography	12

Annex A		13

Keywords

Internet, testing, measurement of QoS, network performance, QoE

# Scope

This Recommendation describes the requirements for parameters and procedures to be used for Internet speed quality measurement. The proposed methodology is based on the concept of the ITU-T Q.FW_Int_sp_test “Framework of Internet speed measurements for the fixed and mobile networks”.

# References

[ETSI EG 202 057-4]:	ETSI Guide 202 057-4 (2008), Speech processing, transmission and Quality aspects (STQ); User related QoS parameter definitions and measurements; Part 4. V1.2.1.

[ETSI TS 103 222-1]:	ETSI Technical Specification 103 222-1 (2015), Speech and multimedia Transmission Quality (STQ); Reference benchmarking background traffic profiles and KPIs. Part 1: Reference benchmarking, background traffic profiles and KPIs for VoIP

# Definitions

Data Transmission Speed — the data transmission rate that is achieved separately for downloading and uploading specified test files between a remote web site and a user's computer. (As defined in [ETSI EG 202 057-4], Section 5.2.1).

Internet Service Provider (ISP):  organization that provides users with an Internet access.

The definition of ISP for the purpose of this document can be drawn from the definition of IAP (ETSI EG 202 057-4, Section 3.1) by analogy

Telecom operator: an organization responsible for identification and management of telecommunication network used wholly or partly for the provision of publicly available telecommunication services.

Combination of the following definitions: ITU-T, M.1400 (04), 3.11 and ETSI EG 202 057-4, Section 3.1, PTN

# Abbreviations

BDP	Bandwidth-Delay Product

CDN	Content Data Network

IR	Internet Resource

ISP	Internet Service Providers

RTT	Round-Trip Time

# Conventions

None

# Test methodology

This section defines the basic framework of the test methodology, including initial conditions, the phases of the test, measurement algorithms and stop conditions. Also, some guidelines in results processing are introduced.

# General requirements

The test aims at delivering an accurate measurement of the maximum bandwidth available over a given internet connection. This is achieved by transferring multiple parallel data streams over separate TCP connections within a predefined amount of time. The transferred data consist of randomly generated data with high entropy. It is not expected that the (pseudo) random number generator meets cryptographic requirements. However it should effectively avoid data compression during the transmission. In order to increase the probability that the test can be performed even within networks protected by firewalls and proxy servers, the data should be transferred over HTTP or HTTPS (i.e. TLS or SSL connection).

# System components

The measurement system is composed as follows:

RESULT

DATA

CONTROLLER

RESULTS

MEASUREMENT PEERS

DB

PARAMETERS

MEASUREMENT

COLLECTOR

RESULT

DATA

MEASUREMENT

AGENT

CONTROLLING

& MEASUREMENT

RESULTS

DATA

ACQUISITION

Figure 1. Measurement System Components

The requirements for the system components are described in related ITU-T Q.FW_Int_sp_test Recommendation.

By default, all data transmissions between Measurement Agent and Measurement Peer (test data) and between Measurement Agent and Collector (result data) should be carried over TCP in order to avoid interference with firewalls as much as possible. The ports for communication and data transfers between the different servers themselves are configurable.

Only the controller may use a different protocol to keep the communication simple and therefore the workload of the server at a minimum.

# Initial conditions

The test methodology defines a set of variables and constants that need to be assigned prior to the execution of the measurement algorithm or during the test execution. Some of these values are open to selection within a range, according to the specific considerations of each implementation or the current network conditions at the time the test is being executed.

|  | Parameter | Unit | Range | Default value |
| --- | --- | --- | --- | --- |
| n | Number of parallel connections | # | 1 ≤ n ≤ 10 | n = 3 |
| Tp | Duration of pre-test | s | 0 ≤ Tp ≤ 5 | Tp = 2 s |
| Td | Duration of the downlink subtest | s | 5 ≤ Td ≤ 15 | Td = 7 s |
| Tu | Duration of the uplink subtest | s | 5 ≤ Tu ≤ 15 | Td = 7 s |
| To | Timeout value | s | 5 ≤ To ≤ 10 | To = 5 s |
| p | Number of ‘pings’ during delay subtest | # | 5≤ p ≤ 20 | p =10 |
| Tl | Maximum elapsed time before the first ping starts | ms | 200 ≤ Tl ≤ 1000 | Tl = 500 ms |
| z | Reference size of data block (chunk size) | KB | TBD | z = 4 KB |
| c | Number of received chunks after which the MP answers back to the MA | # | 5 ≤ c ≤ 20 | c = 10 |

Table 1. Indicative Measurement Parameters*

*All the values in the table are indicative. It must be discussed the suitability of each of the variables/constants and the most adequate default values, as well as the acceptable ranges.

Some of these parameters (specially z and c) might be related to the specific methodology to be applied. Suitability must be discussed.

The meaning of these parameters and the selection of the most adequate values will be further detailed in next sections.

More information about this issue can be found in Annex A: “List of issues under study”

# Measurement procedure

This section defines the main phases of the measurement procedure, once the Measurement Agent has obtained the test code from the Controller and the location or locations of the measurement peers.

The Measurement Peer (MP) is determined directly in Controller by geo-locating the Measurement Agent (MA) and determining his closest access to the Internet. In case of executing the Internet resource speed test, the Measurement Peer can be either selected by default or letting the user chose a destination within a list of Internet resources.

The following set of graphics state the basic exchange of information between the Measurement Agent (MA) and the selected Measurement Peer (MP) for each of the cases (The same procedure is defined for both the Network Internet speed test and the Internet resource speed test).

The test follows the procedure outlined below. The test consists of seven phases which are carried out one after each other, i.e. phase m starts after phase m-1 has finished without any pause in-between. That means that the phases do not overlap.

To ensure comparable data transmission speed test conditions in mobile networks, a pre-load should be initiated. With the pre-load, the mobile networks are set in a defined initial state i.e. CELL_DCH in UMTS and connected in LTE.

If the downlink and uplink pre-tests procedures are not implemented (Phases 2 and 5), the Measurement Agent shall open a TCP uplink and downlink connection. For downlink, the Controller sends a data block of size z. For uplink, the Measurement Agent sends a data block of size z.

# Phase 1: Initialization

The Measurement Agent tries to connect to the Controller on the TCP using HTTP and/or HTTPS using port 443 over TLS or SSL. In order to pass through certain firewalls, which might block unencrypted data transmissions, HTTPS might be necessary. The data streams themselves are optionally unencrypted. In order for the computational cost of encryption not to deteriorate the measurement results, encryption and authentication parameters should be chosen as low as possible.

After establishing a proper connection, Measurement Agent and Measurement Peer exchange the information, which is required for running the test.

The Measurement Agent sends its version number, optional the geo-location data and IP address as well as other relevant information to the Controller.

The Controller determines the Measurement Peer to be used. It then generates a token consisting of the following components:

- a unique test ID;

- the time at which the measurement is allowed to start (a string representing Unix time);

- the period of time during which access to the Measurement Peer is allowed (a string representing number of seconds);

- the maximum number of permitted parallel connections from the Measurement Agent to the Measurement Peer (a string representing a positive integer);

The Controller then transmits the token as well as all the additional test parameters (IP address of the Measurement Peer, public IP address of the Measurement Agent, number of parallel TCP connections, etc.) to the Measurement Agent. The token is used for identifying the test session.

# Phase 2: Downlink pre-test

The downlink pre-test is for high speed internet connections optional.

In Phase 2, the Measurement Agent opens n TCP connections to the assigned Measurement Peer. Within each TCP connection, the Measurement Agent requests and the Measurement Peer sends a data block of size z (randomly generated data with high entropy). While the duration of the pre-test has not exceeded Tp, the Measurement Agent requests a data block of double size compared to the last iteration step. The transfer of the last data block will be finished even if the duration has already exceeded Tp. At the end of the pre-test, all TCP connections are left open for further use if more than four chunks have been transmitted in the downlink pre-test. Otherwise, n is reduced to 1.

| # Chunks | Data Block chunk in Kbyte/Mbyte | Data Blocks [Mbyte] transmitted in time tp |
| --- | --- | --- |
| 1 | 4/ 0.004 | 0.004 |
| 2 | 8/ 0.008 | 0.012 |
| 3 | 16/0.016 | 0.028 |
| 4 | 32/0.032 | 0.06 |
| 5 | 64/0.06 | 0.096 |
| 6 | 128 | 0.224 |
| 7 | 256 | 0.480 |
| 8 | 512 | 0.992 |
| 9 | 1024 | 2.016 |
| 10 | 2048 | 4.064 |

Table 2: Size of chunks and Data Blocks chunks transmitted in time Tp

MA

MP

Request Pre-DL

PHASE 2

Downlink Pre-test

n connections

Random

Incompressible

data

Evaluates initial parameters

for downlink subtest

Pre-test

duration time

Tp

Figure 2. Sample diagram of phase 2 of measurement algorithm

# Phase 3: Latency test

During this phase, the Measurement Agent sends p “pings” in short intervals to the Measurement Peer to test the latency of the connection. One “ping” consists of the transmission of short strings via one of the TCP connections to the Measurement Peer, which returns short strings as acknowledgement. The Measurement Agent measures the time between sending and receiving the return message, while the Measurement Peer additionally measures the time between sending its return message and the Measurement Agent’s reception response. The Measurement Agent stores all measurements.

MA

MP

ping # 1

ping # p

ack # 1

ack # p

PHASE 3

Latency Subtest

1 TCP connection

RTT1

RTTp

ack # 2

ping # 2

Figure 3. Sample diagram of phase 3 of measurement algorithm

# Phase 4: Downlink subtest

Within each of the n TCP connections opened during phase 2, the Measurement Agent simultaneously requests and the Measurement Peer continuously sends data streams consisting of fixed-size chunks of size s (randomly pre-generated data with high entropy).

As an option, the Collector can continuously send data streams on n TCP connections consisting data files from the data-reference system. All transmissions start at the same time, which is denoted as relative time 0. For each TCP connection k, 1 ≤ k ≤ n, the Measurement Agent records the elapsed time tk  and the amount of data zk received from time 0 to tk for successive values (usually after receiving each chunk or data files from the data-reference system). After time td, each TCP connection is reset immediately after the next chunk (or data file) has been received. After finishing all tests, the Measurement Agent sends the collected data to the Collector.

MA

MP

Request DL

PHASE 4

Downlink Test

n connections

Incompressible

data

Downlink subtest duration time

Td

Figure 4. Sample diagram of phase 4 of measurement algorithm

# Phase 5: Uplink pre-test

The uplink pre-test is for high speed internet connections optional.

In Phase 5, the Measurement Agent opens n TCP connections to the assigned Measurement Peer. Within each TCP connection, the Measurement Agent sends a data block of size z (randomly pre-generated data with high entropy). While the duration of the pre-test has not exceeded Tp, the Measurement Agent sends a data block of double size compared to the last iteration step. The transfer of the last data block will be finished even if the duration has already exceeded Tp. At the end of the pre-test, the TCP connections are left open for further use.

MA

MP

PHASE 5

Uplink Pre-test

n connections

Pre-UL

Random

Incompressible

Data

Evaluates initial parameters

for uplink subtest

Pre-test

duration time

Tp

Figure 5. Sample diagram of phase 5 of measurement algorithm

# Phase 6: Uplink subtest

Within each of the n TCP connections opened during phase 5, the Measurement Agent continuously sends data streams consisting of fixed-size chunks of size z (randomly generated data with high entropy). As an option, the Measurement Agent can continuously send data streams on n TCP connections consisting of a data file from the data-reference system. All transmissions start at the same time, which is denoted as relative time 0. For each TCP connection k, 1 ≤ k ≤ n, the Measurement Peer gives feedback to the Measurement Agent by sending the elapsed time tk and the amount of data zk  received from time 0 to tk

- after receiving each chunk whose transmission has taken more than 1 ms or

- otherwise 1 ms after the previous feedback

After time t, it is tested at intervals of 0.25 s whether all feedbacks for chunks sent from the Measurement Agent to the Measurement Peer until time t – 1 s have been received. As soon as this is the case, all TCP connections are reset. Otherwise, the test is aborted after time t + 3 s. All results and additional information on the Measurement Agent are transferred directly to the Collector.

MA

MP

PHASE 6

Uplink Test

n connections

Uplink subtest

duration time

Tu

MP sends

reference data

to the MA

Upload

Incompresible

Data

Figure 6. Sample diagram of phase 6 of measurement algorithm

# Phase 7: Finalization

After finishing all tests, the Measurement Agent sends the collected data to the Collector. All results and additional information on the Measurement Agent are transferred directly to the Collector. Both datasets are then compared by the Collector to check the quality and integrity of the result. All tests, successful or unsuccessful, are stored by the Collector.

## Measurement of Data

As far as it is available, the following information is stored for each completed test:

- Test UUID.

- Date and time.

- Latency.

- Uplink capacity.

- Downlink capacity.

- Used encryption: HTTPS over TLS, SSL, HTTP (NONE).

- Number of concurrent connections Measurement Agent public IP (Optional).

- Measurement Agent phone status and radio technology (e.g. UMTS, LTE, etc.) and quality (eg.RSSI).

Geo-location and tracking accuracy as well as movement during the test (latitude, longitude, altitude, timestamp, speed, bearing, location provider, information to assess GPS accuracy, etc.). (Optional).

# Additional considerations

TBD

# Bibliography

- http://www.alexa.com/topsites

- http://www.oecd.org/internet/speed-tests.htm

- https://blog.kissmetrics.com/loading-time/

- http://www.wrox.com/WileyCDA/WroxTitle/Professional-Website-Performance-Optimizing-the-Front-End-and-Back-End.productCd-1118487524.html

- http://sgdev-blog.blogspot.sg/2014/01/maximum-concurrent-connection-to-same.html

- https://fasterdata.es.net/assets/fasterdata/Tierney-bulk-data-transfer-tutorial-Sept09.pdf

- http://fasterdata.es.net/assets/fasterdata/JT-201010.pdf

- http://www.testvelocidad.eu

- https://support.speedtest.net/entries/20862782

- http://www.testskorosti.ru/

- http://speedmeter.de

- http://2ip.ru/speed/

- http://pr-cy.ru/speed_test_internet

- https://itunes.apple.com/ru/app/net-speed-test-lite/id471993926?mt=8

- http://www.speed.io

- http://www.voiptest.org/

- http://speedtest.south.rt.ru/#

- http://myconnect.ru/speed

- http://wi.test

__________________

____________

Annex A

List of issues under study related to Q.Int_Speed_test Draft Recommendation _____

A.1	Introduction

In the interim Rapporteur meeting of 13th November 2014 of QuestionQ15/11 it was agreed to attach to the baseline document of Q.Int_Speed_test Draft Recommendation a separate document with the issues under study. This document contains the referred information. The aim of this attachment is to encourage delegates to make their contributions on these issues and track the rationale for the agreement on selected parameters and measurement algorithms.

.

A.2	Issues under study

Location of external resources

As referred in section 8.2 of the baseline document TD-XXX, for the Internet access speed test it should be decided whether the measurement endpoint could be located within the operator network, as close to the internal interface of the exchange point, considering the limitations any operator might have when deploying the system.

In the case of the Internet resources access speed test, there are also different options for the location of the external resources: a) top 10 or 20 websites (i.e. in Alexa Rank), b) most important content distribution network (CDNs), c) most important network interconnection points (with higher traffic volume/peaks, most used...), a d) network of geographically dispersed destinations around the world, to cover, at least, tests in different ranges (same continent, long distance...), etc.

It should be decided which one can be the best option according to the aim of the future recommendation.

Figure 12 shows a sample of two way latency tests to #10 #50 and #500 most popular websites (according to Alexa Ranking) from a particular location (in Europe) which will have an impact into not only the maximum achievable speed but also the time needed for a reliables test. As can be easily seen, most of the top 10 sites are quite close to the end users (below 100ms two way latency). However, as the focus is broaden and regardless CDN technologies and exchange points suggesting “moving content as close to the user as possible” regional location still plays a role so that defining a worldwide neutral Internet Resource is not trivial. Regional (geolocalised) most important Network Interconnection Points (option d) is recommended.

Figure 12.  Two way latency example to #10, #50 and #500 most popular websites (Apr2015)

Collecting additional information from user´s hardware and software

As suggested in section 10 of the baseline document TD-XXX, collecting data on the user´s hardware and software configuration (SO, Browser) can be useful not only for statistical result evaluation but also to send warnings to the users about unsuitable configuration if detected or other configuration problems that may lead to unreliable measurements. In contrast, collecting user´s information can be a sensitive issue since it could threaten the privacy of the user if some delicate information is collected (i.e. geolocation). It should be decide a “compromise solution” for this issue.

The following minimum information will be collected

- For fixed networks:

    - Test time;

    - Time zone;

    - User IP address;

    - TCP and UDP settings;

    - Operation system version;

    - Web-browser version/User Agent (In case web technology is used);

    - Number of parallel connections (downlink and uplink);

    - Duration of pre-test;

    - Duration of the downlink subtest;

    - Duration of the uplink subtest;

    - Timeout value;

    - Number of ‘pings’ during delay subtest;

    - Maximum elapsed time between ping starts;

    - Reference size of data block (chunk size);

    - Number of received chunks after which the ME answers back to the TE

    - WS advertised during the TCP negotiation phase for the different TCP streams: Regarding the warnings, although n parallel connections will be launched in the test, the Measurement Peer will calculate whether for the estimated RTT, OS (TCP flavour + typical Tx/Rx buffer sizes) and measured WS the calculated downlink and uplink speeds are too close (>90%) of the theoretical limitations, notifying the user about the unreliability of the test and the need to optimize his configuration for achieving better speeds.

(according to https://www.netztest.at/en/ and section 12.1 of this Recommendation)

- For mobile networks (in addition to information for fixed networks):

    - Location inferred from cell tower triangulation, WiFi triangulation, or GPS

    - Active cell tower ID and signal strength (RSSI)

    - Visible neighboring cell towers, including cell IDs and RSSI

    - Active network operator ID ("MNC") and name

    - Active network country code ("MCC") and name

    - SIM's network operator ID and name

    - SIM's network country code and name

    - Bearer (CDMA, GPRS, EDGE, all of the 3G variants, LTE, etc)

    - Phone type (GSM, CDMA)

(according to http://www.fcc.gov/measuring-broadband-america/mobile/technical-summary)

Measurements methodology

As referred in sections 11 and 12 of the baseline document TD-XXX, there are still many open issues related to the tests requirements and methodology to be defined:

              - Phases in the tests:	After analysing many of the measurement tests available at the OECD website, it seems clear that different phases should be considered in the tests measurement procedure:

- Latency subset: Different alternatives can be used for latency measurement: conforming and sending a classic ICMP packet, measuring the elapsed time from a SYN message to the SYN ACK response, sending a zero byte file to the measurement endpoint and awaiting acknowledgement...). In this phase a unique TCP connection seems to be adequate but it should be defined the more accurate alternative for measurement. In order to avoid firewall traversal problems and once web technology is agreed an Application Level “ping” is recommended (i.e. ping/pong mechanism in websockets or empty file download time in traditional HTTP/HTTPS).

- Downlink/Uplink Pretest: The pre-test phases aims to widen the congestion windows size, which is quite important, specially the in uplink pre-test. Nevertheless, many aspects of this phase (some described here below) are still to be defined.

- Downlink/Uplink test: The test phases aims to measure the downlink/uplink data transmission speed but it should be clarified if the aim is to measure the average downlink/uplink speed or the bulk capacity because this would also help to define the proper measurement procedure.

Figure 13.  Samples of CWND and Goodput evolution for USA=>EUR tests in a wired connection

- Figure 13 shows the evolution of both CWND and achieved goodput for several different test along a whole day between USA=>EUR. The high variability, even for wired connections, a single maximum/average value is not representative enough. Instead, a RTT-sized bin vector instead will be collected. Later, the server will process this information to a) Remove SS phase, b) statistically remove outliers c) Calculate significative enough maximum and average speed to be reported to the Users.

              - Protocol to be used: 	HTTP or HTTPs are proposed.

              - Duration of the test: 	Test could be limited by size of the transmitted data or by time. Different alternatives can be considered:

- Transmitting a fixed volume of data within a maximum timeout limit.

- Transmitting a variable/adaptable volume of data within a timeout limit.

- Transmitting data continuously for a limited period of time.

- .

- In any case, some reference (maximum test durations/file size) should be specified in the recommendation.

              - Number of concurrent TCP connections: The usage of multithreading as well as the (optimum) number of parallel TCP connections should be also considered. It should be also decided if default values can be proposed in the recommendation (like the ones in ETSI TS 103 222 v0.0.25) or not (since they will need revision periodically due to TCP new versions implemented).

- Carried out tests, common trends towards multicore processors even in mobile devices and current TCP flavours aggressiveness suggest 4/8 threads suitable enough to achieved maximum speed in the 100s of Mbps range for RTTs around 150ms (covering worst case scenario for most popular websites).

____________