Home Network Transport

Standards Overview and Work Plan

September 2016 Q1/15 meeting

Contact persons for project updates:

| Study Group 15 Advisor: Mr. Hiroshi Ota International Telecommunication Union (ITU) Place des Nations 1211 Geneva 20 Switzerland Tel.: +41 22 730 6356 E-mail: hiroshi.ota@itu.int | Study Group 15 Chairman Dr. Stephen J. Trowbridge Alcatel-Lucent 5280 Centennial Trail Boulder, Colorado 80303-1262 USA Tel: +1 720 945 6885 E-mail: steve.trowbridge@nokia.com | Question 1/15 Rapporteur Mr. Jean-Marie Fromenteau Corning Incorporated Corning, NY 14831 USA Tel: +49 9561 42 74 20 E-mail: fromentejm@corning.com |
| --- | --- | --- |

Home Network Transport is an ITU-T Project dealing with studies and Recommendations on the Home Network.

- Home Network Transport Standards Overview and Work Plan

- Issue 5, September 2016

# Revision Status Report: Major Updates of Version 5, September 2016

1) Existing Recommendations within ITU-T SG15 – WP1/15 Table 2 – “Approved” Recommendations related to Home Network In this version 5: - ITU-T G.9903 (2014) Amd.1, G.9904 (2012), G.9960 (2015) Cor.2, G.9960 (2015) Amd.2, G.9961 (2015) Cor.2, G.9961 (2015) Amd.2, G.9962 (2014) Amd.1 (ex G.hn), G.9963 (2015) Amd.1, G.9963 (2015) Cor.1, G.9964 (2011) Amd.1, G.9964 (2011) Amd.2, G.9979 (2014) Amd.1 and G.9977 (2016) have been added. 2) Recommendations “Under Study” within ITU-T SG15 – WP1 Table 3 – Recommendations “Under Study” and related to Home Network has been renewed 3) Other groups within ITU and other Standard Developing Organizations - Refer to updates from ITU-R SG5 WP5A and from IEEE 802.3

- Home Network Transport Standards Overview and Work Plan

- Issue 5, September 2016

1. Introduction	4

2. Broadband in-premises networking – Home Network	4

3. Home Network Activities in ITU-T	5

4. Existing Recommendations within ITU-T SG15 – WP1/15	9

5. Recommendations “Under Study” within ITU-T SG15 – WP1/15	12

6. Other groups within ITU and other Standard Developing Organizations	13

7. Contacts	26

HOME NETWORK TRANSPORT STANDARDS OVERVIEW AND WORK PLAN

- Issue 5, September 2016

# 1. Introduction

- As the use of electronics and communications equipment proliferates in the home, as well as their
technologies and protocols, there have been several proposals for standardization of equipment,
transport and networking within the "home network"-

# 2. Broadband in-premises networking – Home Network

- The continuing customer demand for ever higher bit rate data services, high-speed Internet access and other innovative services, and the ongoing needs of network operators to leverage in-premises connectivity for distributing within the home IPTV and other applications, will require the development of new Recommendations and enhancements to existing Recommendations covering all aspects of in-premises Networking Transceivers. These studies will include, but are not limited to, the transport of higher layer protocols, the management and test of the in-premises systems, spectral management aspects and energy saving techniques.

- Given the interdisciplinary nature of Home Network applications, it is expected that a high degree of cooperation with other ITU Sectors (ITU-R, ITU-D), ITU-T Study Groups, Questions, Focus Groups (FGs), Joint Coordination Activities (JCAs), Global Strategic Initiatives (GSIs), as well as other international bodies will be required.

- Within the ITU-T, the study and development of Recommendations related to transport in the access networks - in premises networks - is being carried out in a number of different Study Groups (see Table 1 below), e.g.  SGs 5, 9, 13, 15, and 16. Also ITU-R and other standards bodies, forums and consortia are also active in this area. See Sect. 2 for a list of such activities.

- Recognizing that without a strong coordination effort there is the danger of duplication of work as well as the development of incompatible and non-interoperable standards, the ITU-T designated Study Group 15 as the Lead Study Group on Home Network.

# 3. Home Network Activities in ITU-T

- This section provides an overview of the existing Home Network related ITU-T activities.
Several ITU-T study groups are working on Home Network related topics.
Table 1 below gives an updated overview of such activities.

Table 1 – ITU-T study groups working on Home Network related topics

| Items | SGs and aspects |  |
| --- | --- | --- |
| (1) Environment and climate change | SG5 | TD 612 WP1 February 2016 WP1/5 “Damage prevention and safety” Q4/5 “Resistibility and safety in telecommunications” - ITU-T K.21: "Resistibility of telecommunication equipment installed in customer premises to overvoltages and overcurrents". This existing Recommendation is being updated in order to take into account the long cabling used for Ethernet ports in home networks, particularly those using PoE. Q5/5 “Lightning protection and earthing of telecommunication systems” - ITU-T K.85: "Requirements for the mitigation of lightning effects on home networks installed in customer premises". |
|  |  | TD 612 WP1 February 2016 WP2/5 “Electromagnetic fields: emission, immunity and human exposure” Q8/5 “EMC issues in home networks” - ITU-T K.74: “Electromagnetic compatibility, resistibility and safety requirements for home network devices”. This existing Recommendation was updated the description and references to harmonize the newly developed Recommendation and EMC standards. - ITU-T K.92: “Conducted and radiated electromagnetic environment in home networking”. - ITU-T K.93: “Immunity of home network devices to electromagnetic disturbances”. This existing Recommendation provides test method for home network equipment against broadband noises. - ITU-T K.104: “Method for identifying the transfer potential of the earth potential rise from high or medium voltage networks to the earthing system or neutral of low voltage networks”. This new Recommendation provides guidance to solve interference problems in home networking environments between radio devices and the cable or equipment connected to wired broadband networks and/or cable television networks. |
|  |  | WP3/5 “ICT and climate change” Q17/5 “Energy efficiency for the ICT sector and harmonization of environmental standards” TD 357 WP1 Nov.-Dec. 2014 Q17/5 Work Program: An up-to-date status of work under Q17/5 is contained in the SG 5 work program (use URL below): http://www.itu.int/ITU-T/workprog/wp_search.aspx?isn_sp=1749&isn_sg=1752&isn_wp=2196&isn_qu=1932&isn_status=-1,1,3,7&details=0&field=aebcgfkjl Note: in particular, L.1310 “Energy efficiency metrics and measurement for telecommunication equipment” contains Clause 12 dedicated to “Energy efficiency metric for small networking devices” Moreover, L.1340 “Energy efficiency reference values for telecommunication equipment and infrastructure” contains Clause 9.1 on “Informative values for small networking devices” |
| (2) Broadband cable and TV | SG9 | Q1/9 “Transmission of television and sound programme signal for contribution, primary distribution and secondary distribution” Q7/9 “Cable television delivery of digital services and applications that use Internet protocol (IP) and/or packet-based data” J.HiNoC “Physical layer specification for high speed transmission over coaxial networks” (joint work between Q1/9 and Q7/9) TD 395 WP1 Nov.-Dec. 2014 Leakage of radio frequency energy from J.HiNoC systems and coexistence of wired telecommunications with radiocommunication systems Considerations with respect to work on Recommendation ITU-T J.195.1 (J.HINOC-REQ): response to ITU-R WP 5B - 5B/TEMP/295 & ITU-R WP 1A - LS 19. TD 399 WP1 Nov.-Dec. 2014 New Recommendations ITU-T J.195.2 and J.195.3 SG 9 reached status of AAP Consent for the draft new Recommendations ITU-T J.195.2 (ex. J.HiNoC-phy) “PHY layer specification of high performance network over coax” and J.195.3 (ex. J.HiNoC-mac) “MAC layer specification of high performance network over coax”. This new Recommendations realizes bi-directional high-performance wideband access digital systems that use coaxial cable inside a building. |
|  |  | Q5/9 “Functional requirements for residential gateway and set-top box for the reception of advanced content distribution services” Q9/9 “Requirements for advanced service capabilities for broadband cable home networks” TD 397 WP1 Nov.-Dec. 2014 SG9 just started the study of HNT area taking into consideration wireless technologies, such as IEEE 802.11ac, ZigBee, Bluetooth and other low power radio communication technologies which are workable on 6LowPan protocol |
| (3) Future networks | SG13 | Q11/13 Evolution of user-centric networking services, and interworking with networks of the future including software-defined networking (Next generation home network) TD 591 WP1 February 2016 Q11/13 informs: (TD 324 WP3 Apr. 2015) SG13 just started the study of home network area taking into consideration Web of Objects technologies. (Y.WoO-hn, Service capability and architecture in web of objects enabled home network). NOTE: Based on the decision from TSAG, SG13 has decided to transfer this work item (Y.WoO-hn) to SG20. |
| (4) Access and home networking | SG15 | Q1 Coordination of Access and Home Network Transport standards Q15/15: Communications for Smart Grid (home networking related Smart Grid communications): see Smart Grid overview and work plan (use URL below) http://www.itu.int/en/ITU-T/studygroups/2013-2016/15/Pages/sg.aspx Q18/15 Broadband in-premises networking: see section 4 - table 2 and section 5 - table 3 |
| (5) Multimedia | SG16 | Q21/16 Multimedia framework, applications and services (home network services) |
| (6) Security | SG17 | Q6/17 Security guidelines for Home Area Network (HAN) devices in Smart Grid systems TD 505 WP1 Jun-Jul 2015 ITU-T SG 17, Security, in its Question 6/17, Security aspects of ubiquitous telecommunication services, informs of ongoing activity related to Home Networks from last September 2014 SG17 meeting. Q6/17 work item which is related to Home Networks is draft Recommendation X.sgsec-2, Security guidelines for home area network (HAN) devices in smart grid systems. |

# 4. Existing Recommendations within ITU-T SG15 – WP1/15

“Approved” Recommendations related to “Access networks – In premises network” are listed here for convenience in Table 2.

Table 2 – “Approved” Recommendations related to Home Network

| Rec. No. | Title | Status | Date |
| --- | --- | --- | --- |
| G.9902 (2012) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for ITU-T G.hnem networks | Approved | 10/2012 |
| G.9902 (2012) Amd.1 | Narrowband orthogonal frequency division multiplexing power line communication transceivers for ITU-T G.hnem networks Amendment | Approved | 03/2013 |
| G.9902 (2012) Amd.2 | Narrowband orthogonal frequency division multiplexing power line communication transceivers for ITU-T G.hnem networks Amendment 2: Clarifications on payload encoder and addition of a network admission procedure | Approved | 08/2013 |
| G.9903 (2014) | Narrow-band OFDM power line communication transceivers for G3-PLC networks | Approved | 02/2014 |
| G.9903 (2014) Amd.1 | Narrowband orthogonal frequency division multiplexing power line communication transceivers for G3-PLC networks: Amendment 1 | Approved | 08/2015 |
| G.9904 (2012) | Narrowband orthogonal frequency division multiplexing power line communication transceivers for PRIME networks | Approved | 10/2012 |
| G.9951 (2001) | Phoneline networking transceivers − Foundation | Approved | 02/2001 |
| G.9952 (2001) | Phoneline networking transceivers – Payload format and link layer requirements | Approved | 11/2001 |
| G.9953 (2003) | Phoneline networking transceivers – Isolation function | Approved | 03/2003 |
| G.9954 (2007) | Home networking transceivers – Enhanced physical, media access, and link layer specifications | Approved | 01/2007 |
| G.9959 (2015) | Short range narrow-band digital radiocommunication transceivers - PHY and MAC layer specifications | Approved | 01/2015 |
| G.9960 (2015) | Unified high-speed wireline-based home networking transceivers – System architecture and physical layer specification | Approved | 07/2015 |
| G.9960 (2015) Cor.1 | Unified high-speed wireline-based home networking transceivers - System architecture and physical layer specification: Corrigendum 1 | Approved | 11/2015 |
| G.9960 (2015) Cor.2 | Unified high-speed wireline-based home networking transceivers - System architecture and physical layer specification: Corrigendum 2 | Approved | 04/2016 |
| G.9960 (2015) Amd.1 | Unified high-speed wireline-based home networking transceivers - System architecture and physical layer specification: Amendment 1 | Approved | 11/2015 |
| G.9960 (2015) Amd.2 | Unified high-speed wireline-based home networking transceivers - System architecture and physical layer specification: Amendment 2 | Approved | 04/2016 |
| G.9961 (2015) | Unified high-speed wire-line based home networking transceivers - Data link layer specification | Approved | 07/2015 |
| G.9961 (2015) Cor.1 (ex G.hn) | Unified high-speed wire-line based home networking transceivers - Data link layer specification: Corrigendum 1 | Approved | 11/2015 |
| G.9961 (2015) Cor.2 | Unified high-speed wire-line based home networking transceivers - Data link layer specification: Corrigendum 2 | Approved | 04/2016 |
| G.9961 (2015) Amd.1 (ex G.hn) | Unified high-speed wire-line based home networking transceivers - Data link layer specification: Amendment 1 | Approved | 11/2015 |
| G.9961 (2015) Amd.2 | Unified high-speed wire-line based home networking transceivers - Data link layer specification: Amendment 2 | Approved | 07/2016 |
| G.9962 (2014) | Unified high-speed wire-line based home networking transceivers – management specification | Approved | 10/2014 |
| G.9962 (2014) Amd.1 (ex G.hn) | Unified high-speed wire-line based home networking transceivers - Management specification: Amendment 1 | Approved | 04/2016 |
| G.9963 (2015) | Unified high-speed wire-line-based home networking transceivers – Multiple input/multiple output specification | Approved | 07/2015 |
| G.9963 (2015) Cor.1 | Unified high-speed wire-line based home networking transceivers - Multiple input/multiple output specification: Corrigendum 1 | Approved | 04/2016 |
| G.9963 (2015) Amd.1 | Unified high-speed wire-line based home networking transceivers - Multiple input/multiple output specification: Amendment 1 | Approved | 07/2016 |
| G.9964 (2011) | Unified high-speed wireline-based home networking transceivers – Power spectral density specification | Approved | 12/2011 |
| G.9964 (2011) Amd.1 | Unified high-speed wire-line based home networking transceivers - Power spectral density specification: Amendment 1 | Approved | 02/2016 |
| G.9964 (2011) Amd.2 | Unified high-speed wire-line based home networking transceivers - Power spectral density specification: Amendment 2 | Approved | 09/2016 |
| G.9970 (2009) | Generic home network transport architecture | Approved | 01/2009 |
| G.9971 (2010) | Requirements of transport functions in IP home network | Approved | 07/2010 |
| G.9972 (2010) | Coexistence mechanism for wireline home networking transceivers | Approved | 06/2010 |
| G.9972 (2010) Cor.1 | Coexistence mechanism for wireline home networking transceivers Corrigendum 1: Revised definition of coexisting systems categories | Approved | 04/2014 |
| G.9973 (2011) | Protocol for identifying home network topology | Approved | 10/2011 |
| G.9977 (2016) | Mitigation of interference between DSL and PLC | Approved | 02/2016 |
| G.9979 (2014) | Implementation of the generic mechanism in the IEEE 1905.1a-2014 Standard to include applicable ITU-T Recommendations | Approved | 12/2014 |
| G.9979 (2014) Amd.1 | Implementation of the generic mechanism in the IEEE 1905.1a 2014 Standard to include applicable ITU-T Recommendations: Amendment 1 | Approved | 02/2016 |
| G.9980 (2012) | Remote management of customer premises equipment over broadband networks – Customer premises equipment WAN management protocol | Approved | 11/2012 |
| TPLS.G-HN | Operation of G.hn technology over access and in-premises phone line medium | Agreed | 07/2015 |

# 5. Recommendations “Under Study” within ITU-T SG15 – WP1

- Recommendations “Under Study” related to “Access networks – In premises networks” are listed here for convenience in Table 3.

Status:  September 2016

Table 3 – Recommendations “Under Study” and related to Home Network

| Work item | Status | Question | Approval process | Subject / Title |
| --- | --- | --- | --- | --- |
| G.9903 | Under Study | Q15/15 |  | Narrow-band orthogonal frequency division multiplexing power line communication transceivers for G3-PLC networks |
| G.9904.1 | Under study | Q15/15 |  | Narrow-band OFDM power line communication transceivers for PRIME networks with extended band plan |
| G.9960 (2015) Cor.3 | Consented 2016-09-30 | Q18/15 | AAP | Unified high-speed wireline-based home networking transceivers – System architecture and physical layer specification (2015) – Corrigendum 3 |
| G.9961 (2015) Cor.3 | Consented 2016-09-30 | Q18/15 | AAP | Unified high-speed wireline-based home networking transceivers – Data link layer specification (2015) – Corrigendum 3 |
| G.9961 (2015) Amd.3 | Under study | Q18/15 |  | Unified high-speed wireline-based home networking transceivers – Data link layer specification (2015) – Amendment 3 |
| G.9962 (2014) Cor.1 | Consented 2016-09-30 | Q18/15 | AAP | Unified high-speed wireline-based home networking transceivers – Management specification |
| G.9962 (2014) Amd.2 | Under study | Q18/15 |  | Unified high-speed wireline-based home networking transceivers – Management specification Amendment 2 |
| G.9963 (2015) Amd.2 | Under study | Q18/15 |  | Unified high-speed wireline-based home networking transceivers – Multiple input/multiple output specification (2015) – Amendment 2 |
| G.9978 | Under study | Q18/15 |  | Unified high-speed wireline-based home networking transceivers – Secure admission |
| G.vlc | Under study | Q18/15 |  | High speed indoor visible light communication transceiver specifications |

- 6. Other groups within ITU and other Standard Developing Organizations

- This following list is intended to improve understanding and communication of the on-going work related to Home Network in the other groups within ITU and other Standardization Developing Organizations and may help identify possible gaps or overlaps.

| Standards Org. | SGs, Committees, Aspects, Activities |  |  |
| --- | --- | --- | --- |
| ITU-R | SG1 WP1A | SG1 “Spectrum Management” TD 733 WP1 September 2016 Working Party 1A has noted two amendments to Recommendation ITU T G.9964, “Unified high-speed wireline-based home networking transceivers – Power spectral density specification” of the G.hn family of ITU-T Recommendations: - Amendment 1 – New PSD element for 200 MHz bandplan coax cable systems; - Amendment 2 – Revisions to the specification of spectral content for telephone line systems (e.g., VDSL2). Working Party 1A will keep in mind the continuing interest in home-networking and access systems operating over coax and telephone lines, as well as PLT operating over the low voltage electrical mains supply, as Working Party 1A continues to study co-existence issues involving wired telecommunication systems and radiocommunication systems. |  |
|  | SG5 WP5B | SG5 “Terrestrial Services” TD 355 WP1 Nov.-Dec. 2014 WP5B “Maritime mobile service including the Global Maritime Distress and Safety System (GMDSS); the aeronautical mobile service and the radiodetermination service” Coexistence of wired telecommunications with radiocommunication systems. WP5B would seek further clarification about coordination mechanisms between wired standards and radiocommunication services, particularly with external standards developing organizations (SDOs) who do not participate in the relevant ITU activities. |  |
|  | SG5 WP5A | SG5 “Terrestrial Services” TD 722 WP1 September 2016 Radio Local Area Networks (RLANs) and Multiple Gigabit Wireless Systems (MGWS) WP 5A is conducting work on radio local area networks (RLANs) and Multiple Gigabit Wireless Systems (MGWS) under Question ITU-R 212-4/5. RLANs/MGWS could be used to implement a home network. For a description of this work refer to: – section 8.2 of the Guide to the use of ITU-R texts relating to the land mobile service, including wireless access in the fixed service, and in particular Recommendation ITU-R M.1450-5, which recommends characteristics of broadband radio local area networks, – section 8.4 of the Guide to the use of ITU-R texts relating to the land mobile service, including wireless access in the fixed service, and in particular Recommendation ITU-R M.2003-1, which provides general characteristics and radio interface standards for MGWS in frequencies around 60 GHz. |  |
|  | SG6 | SG6 “Broadcasting Service” TD 490 WP1 Jun-Jul 2015 Radiated disturbances from PLT and wired telecommunication systems. Study Group 6 would like to invite further consideration on the following issues: - raise awareness of the adverse impact of introducing extraneous or unnecessary radio frequency energy into the environment in ways that can compromise the operation of radiocommunication services and telecommunication access networks; - consider ways in which ITU can make use of its leading role in telecommunications by deploying the expertise at its disposal for educating and advising outside entities on coexistence, noise and interference issues; and - consider how to encourage SDOs outside ITU to avoid continuing coexistence problems involving wired telecommunication systems that depend on the transmission of radio frequency energy for their operation and radiocommunication systems by adopting ITU-T standards as normative references Refer to ANT Standards Work Plan (Par. 5.1) Recommendation under study at Q4/15: G.9977 (ex G.dsl/plt G.dpm) “Mitigation of interference between DSL and PLC”. |  |
| BBF | TD 248 GEN February 2016 Broadband Forum Organizational Changes - Broadband Forum recently announced an organizational restructuring: “Broadband 20/20 vision” - Broadband 20/20 is a vision of innovative, high value services for the business and consumer stakeholders and their 700+ million broadband installations. These leverage use of NFV, SDN, Ultra-Fast access, Internet of Things, 5G in the home, in small business and in multi-user infrastructure of the Broadband network to create new business models and user experiences. - The Forum has restructured its organization into the following Work Areas which are now in operation: “Broadband User Services”, “Fiber Access Networks”, “Converged Wireless-Wireline”, “SDN and NFV”, “Fiber to the Distribution Point”, “Physical Layer Transmission”, “Routing and Transport”, “Architecture and Migration”. Oversight and coordination of the Work Areas is via the Broadband Forum’s Steering Committee guided by the Technical Committee Chair. Each Work Area is comprised of Project Streams tasked with related technical and marketing projects. - For further details see: https://www.broadband-forum.org/broadband-forum-2020.php - TD 596 WP1 February 2016 Performance Test Plan For In-premises Powerline Communication Systems (TR-208) - The Broadband Forum has published technical report TR-208 to provide industry, operators, and test labs with a well-defined test bed and an established set of tests that enable a performance comparison between powerline products and technologies. Currently, the PHYtx Work Area is working on a second issue of this Technical Report. - Additional information regarding both the Work Areas and current projects can be found at: https://www.broadband-forum.org/technical/technicalwip.php TD 494 WP1 Jun-Jul 2015 CPE WAN Management Protocol (CWMP) The Broadband Forum defines several data models for use with the CPE WAN Management Protocol. New versions of the data model can be found on the CWMP data model web page : http://www.broadband-forum.org/cwmp.php This page, as well as CWMP version 1.4 (defined in TR-069 Amendment 5), represent the current state of the CPE WAN Management Protocol. Other area of interest (as communicated in TD 494 WP1 Jun-Jul 2015): Current study document (SD-333), “Intra Customer Premises Network Measurements”: The study document explores existing mechanisms for customer premises network topology discovery and measurement systems, in addition to exploring the expansions necessary to CWMP (TR-069) and Broadband Forum residential gateway requirements (TR-124) to address these topics An overview of WG mission statements and scope can be found at: http://www.broadband-forum.org/technical/technicalworkinggroups.php Published BBF Technical Reports can be found at: http://www.broadband-forum.org/technical/trlist.php |  |  |
| ETSI PLT | …………………………………………………………………………………. |  |  |
| ETSI EE | TD 357 WP1 Nov.-Dec. 2014 ETSI EE Environmental Engineering Note: in particular refer to EN 301 575 v1.1.1. “Measurement methods for energy consumption of Customer Premises Equipment (CPE)”. |  |  |
| HomeGrid Forum | …………………………………………………………………………………. |  |  |
| HomePlug® Alliance | Power line communication |  |  |
| IEC | CISPR I |  | EMC requirements……………………………………………. |
|  | TC57 WG20 |  | Power line communications…………………………………... |
|  | TC 69 |  | Power line communication for electric vehicles……………… |
| IEEE | IEEE 802.3 |  | WG – IEEE 802.3 Ethernet Working Group TD 806 WP1 September 2016 IEEE Std 802.3-2015, Standard for Ethernet is the current version. IEEE Std 802.3-2015 has a number of approved amendments. (Not all amendments are published as there is a delay between approval of an amendment and its publication.) There also are a number of proposed amendments to the standard. The following are example applicable technologies in IEEE Std 802.3-2015:- The 10BASE-T, 100BASE-TX and 1000BASE-T specifications for operation over various grades of twisted pair cabling have long been used as a home networking technology, and they continue to be applicable. - Home gateways typically include both IEEE Std 802.11 specified capabilities and either 10/100 Mb/s or 10/100/1000 Mb/s Ethernet ports. - 10GBASE-T provides a migration path for higher bandwidth home networks. - Some Ethernet port types would be applicable to HNT needs though use is not common today. For example BASE-T port types are not appropriate for outdoor cable installations, but fiber optic port types would be acceptable. - For access technologies, the approved standard includes various speeds of operation for Ethernet Passive Optical Networks. - The standard also includes DTE Power via the MDI (more popularly called Power over Ethernet) capabilities applicable to HNT (e.g., to power security equipment). The following approved amendments and amendments pending approval are relevant to HNT: - IEEE Std 802.3br-2016, (Amendment 5), Specification and Management Parameters for Interspersing Express Traffic (IET) adds capabilities to reduce message latency for time sensitive networking, which among other things provides enhanced capabilities for multimedia, gaming and other applications becoming more common in the home. - IEEE Std 802.3bn-201x (Amendment 6), Physical Layer Specifications and Management Parameters for Ethernet Passive Optical Networks Protocol over Coax (EPoC) includes new coaxial cable network access capabilities. - IEEE Std 802.3bz-201x (Amendment 7), Media Access Control Parameters, Physical Layers, and Management Parameters for 2.5 Gb/s and 5 Gb/s Operation, Types 2.5GBASE-T and 5GBASE-T includes new speeds of operation between 1 Gb/s and 10 Gb/s speeds on twisted pair, providing additional migration options which will likely find acceptance where higher than 1 Gb/s operation is needed within the home. Other projects and study groups adding capabilities to Ethernet that are relevant to HNT: - IEEE P802.3bv (Amendment 9), Gigabit Ethernet over Plastic Optical Fiber, specifically addresses in-home networking with 1000BASE-RHA. This port type targets providing an easier to install non-conductive media option for home network needs. - IEEE P802.3bv is in the Sponsor ballot stage, with the Task Force working to allow approval by March 2017. - IEEE P802.3bt DTE Power via MDI over 4-Pair is currently in Working Group ballot, progressing toward 2018 approval. This project will support devices requiring more power (Watts) than currently supported in IEEE Std 802.3 specifications. - IEEE P802.3ca 25 Gb/s, 50 Gb/s, and 100 Gb/s Ethernet Passive Optical Networks Task Force is working to create a draft that will focus on development of symmetric and asymmetric data rate 25G-EPON, 50G-EPON, and 100G-EPON PHYs, supporting operation over point-to-multipoint fiber-based subscriber access networks. - The anticipated IEEE P802.3.2 (IEEE 802.3cf) YANG Data Model(s) Task Force will focus on development of a draft standard for YANG data models for Ethernet. |
|  | IEEE 1901 |  | WG - Broadband Over Power Lines PHY/MAC Working Group (BPLPHMAC) |
|  | IEEE 1901.2 |  | WG - IEEE Low-Frequency Narrow-Band Powerline Communications Working Group |
|  | IEEE 1905.1 |  | WG - Convergent Digital Home Network (CDHN) Working Group |
| ISO/IEC | JTC1/SC 25 |  | JTC 1/SC 25 “Interconnection of Information Technology Equipment” TD 383 WP1 Nov.-Dec. 2014 Scope of SC 25: The scope of SC 25 is to provide technologies for interconnection of information technology equipment on Customer premises. SC 25/WG 1 develops a family of standards called the Home Electronic System (HES) for communications within the home and small business environments, and between entities within the home or business and the outside world. SC 25/WG 3 develops standards for customer premises cabling that is the cabling infrastructure on customer premises that supports communication on customer premises as well as of entities on customer premises with the outside world. PTTT (Project Team Taxonomy and Terminology) develops technical reports for terminology and taxonomy. List of published ISO standards under the direct responsibility of ISO/IEC JTC 1/SC 25 and list of drafts and new work items of ISO/IEC JTC 1/SC 25 are available on ISO/IEC JTC 1/SC 25 web page using URL below www.iso.org/iso/iso_technical_committee?commid=45270 Most important specifications: See section 6.1 - table 4 The major HES standards specify the architecture and protocols of the Home Electronic System. These are found in the ISO/IEC 14543 series. Presently three sets of protocols are specified, namely the ISO/IEC 14543-3 series, the ISO/IEC 14543-4 series and the ISO/IEC 14543-5 series. They all conform to a common architecture specified in ISO/IEC 14543-2-1. Since these three protocols cannot directly communicate with each other, SC 25/WG 1 has also specified a series of interoperability and gateway standards, the ISO/IEC 18012 and ISO/IEC 15045 series. For the protection of the home and related communications, a set of standards have been specified. ISO/IEC 24767-1 specifies the security requirements. For protection of the communications between devices in the home that do not have the capability to support IP, a simpler security protocol has been specified in ISO/IEC 24767-2. In order to support energy harvesting devices, i.e. devices that do not depend on batteries or mains power, very energy-efficient wireless communication protocols have been developed (ISO/IEC 14543-60 3-10 and ISO/IEC 14543-3-11, the latter under development). The ISO/IEC 29145 series specifies an efficient wireless mesh network. ISO/IEC SC 25 also has the responsibility for maintaining the ISO/IEC 29341 series of specifications originally developed by the UPnP Forum. In addition to these communication protocol specifications, SC 25/WG 1 is also working on a set of specifications to manage energy usage and generation in the home. These are in the ISO/IEC 15067-3 series. One part has been published; another has been proposed. The cabling standards specify a cabling infrastructure supporting the transport of all kinds of information on customer premises with help of primarily balanced cable and optical fibre cable, and in some cases also of correctional cables. Up to now standards have been published for offices, homes, industrial premises and data centres. |
| MoCA® Multimedia over Coax Alliance | MoCA 1.1 MoCA 2.0 MoCA 2.5 |  | TD 617 WP1 February 2016 The Multimedia over Coax Alliance, MoCA®, currently provides two versions of its MAC/PHY specifications (MoCA 1.1 and MoCA 2.0) which describe an up to 1Gbps Ethernet frame transport protocol over coaxial cable. MoCA 2.0 includes: - Two performance modes: 400 and 800 (bonded) Mbps net throughputs in a point-to-multipoint network and 500 and 1000 (bonded) Mbps in a point-to-point network. - Deterministic (scheduled) media access with 3.5ms average latency. - Two Packet Error modes: Very low Packet Error Rate (1e-8) and Nominal Packet Error Rate (1e-6). - Single or Dual 100 MHz channels operating in the frequency range of 500…1650MHz. - Power states: Active, Low Power, Standby and Sleep power states. -Backward interoperability with previous MoCA versions. Recently concluded field tests of MoCA 2.0 in the US confirmed 400 Mbps in 90 percent of all outlets. Field tests are in progress in China, Israel and South Africa. MaxLinear recently announced availability of a MoCA 2.0 chip capable of 1 Gbps. The Alliance is preparing its next generation specification, MoCA 2.5, which will provide 2.5 Mbps net/actual data rates. It is also backward interoperable with MoCA 2.0. Enhanced management and security features will also be included in MoCA 2.5. MoCA has historically distinguished between PHY and MAC data rates. The Alliance has always emphasized true and realized data rates and thus conducts field tests that verify MAC rates. MoCA does not believe that promoting PHY data rates provides value or contributes to the promotion and adoption of any standard in particular or home networking in general. See also: www.mocalliance.org www.mocainyourhouse.com |
| TIA | TR - 41 |  | Spectral management considerations |

# 6.1. List of the most important ISO/IEC JTC1/SC 25 standards related to Home Electronic Systems (HES) and customer premises cabling

Table 4– ISO/IEC JTC1/SC 25 Standards related to HES and customer premises cabling

| ISO/IEC Specification. No. | Title | Date |
| --- | --- | --- |
| 14543-2-1 | IT - Home Electronic System (HES) Architecture - Part 2-1: Introduction and device modularity Medium: Twisted pair, Wireless | 2006 |
| 14543-3-1 | IT - Home Electronic System (HES) architecture - Part 3-1: Communication layers – Application layer for network based control of HES Class 1 Medium: Twisted pair, Wireless | 2006 |
| 14543-3-2 | IT -Home Electronic System (HES) Architecture - Part 3-2: Communication layers – Transport, network and general parts of data link layer for network based control of HES Class 1 Medium: Twisted pair, Wireless | 2006 |
| 14543-3-3 | IT - Home Electronic System (HES) architecture - Part 3-3: User process for network based control of HES Class 1 Medium: Twisted pair, Wireless | 2007 |
| 14543-3-4 | IT -Home Electronic System (HES) architecture - Part 3-4: System management - Management procedures for network based control of HES Class 1 Medium: Twisted pair, Wireless | 2007 |
| 14543-3-5 | IT - Home Electronic System (HES) architecture - Part 3-5: Media and media dependent layers – Powerline for network based control of HES Class 1 Medium: Powerline, Wireless | 2007 |
| 14543-3-6 | IT - Home Electronic System (HES) architecture - Part 3-6: Media and media dependent layers - Twisted pair for net-work based control of HES Class 1 Medium: Twisted pair | 2007 |
| 14543-3-7 | IT - Home Electronic System (HES) architecture - Part 3-7: Media and media dependent layers - Radio frequency for network based control of HES Class 1 Medium: Wireless | 2007 |
| 14543-3-10 | IT - Home Electronic System (HES) architecture - Part 3-10: Wireless Short-Packet (WSP) protocol optimised for energy harvesting – Architecture and lower layer protocols Medium: Wireless | 2012 |
| 14709-1 | IT - Configuration of Customer Premises Cabling (CPC) for applications - Part 1: Integrated Services Digital Network (ISDN) basic access Medium: Twisted pair | 1997 Amd 1: 2004 |
| 14709-2 | IT - Configuration of Customer Premises Cabling (CPC) for applications - Part 2: Integrated Services Digital Network (ISDN) primary rate Medium: Twisted pair | 1999 Amd 1 2005 |
| 11801 | IT- Generic cabling for customer premises Medium: Fibre, Coax, Twisted pair | Ed 2 2002 Amd 2 2010 |
| 15018 | IT - Generic cabling for homes Medium: Coax, Twisted pair | 2004 |
| 29108 | IT -- Terminology for intelligent homes Medium: Fibre, Coax. Twisted pair, Powerline, Wireless | 2013 |

7. Contacts

| - Body | - Contact person | - Link to the Web-Site | - Status of contact - Notes Liaison Tracking |
| --- | --- | --- | --- |
| - BBF Broadband Forum | - Michael Fargano - Broadband Forum Technical Committee Chair - michael.fargano@centurylink.com - Les Brown, Physical Layer Transmission Work Area co-director lesbrown@sympatico.ca | - www.broadband-forum.org/ | - TD 494 WP1 Jun-Jul 2015 - Liaison Rapporteur Frank Van der Putten frank.van_der_putten@alcatel-lucent.com - TD 596 WP1 February 2016 |
| - IEEE 802.3 | - David Law Chair, IEEE 802.3 Ethernet Working Group dlaw@hpe.com | - ieee802.org/3 | - TD 806 WP1 September 2016 - TD 682 WP1 February 2016 |
| - MoCA® Multimedia over Coax Alliance | - Rob Gelphman VP of Marketing and Member Relations MoCA Tel +1 408 838 7458 robgelphman@mocalliance.org | - www.mocalliance.org www.mocainyourhouse.com | - TD 617 WP1 February 2016 |
| - ITU-R WP1A | - John Shaw shawzone@usa.net | - www.itu.int/en/ITU-R/study-groups/Pages/default.aspx | - TD 733 WP1 September 2016 - TD 363 WP1 Nov-Dec 2014 |
| - ITU-R WP5A | - Gabrielle Owen Liaison Rapporteur ITU-R WP 5A Owengabrielle.owen@agentschaptelecom.nl | - www.itu.int/en/ITU-R/study-groups/rsg5/rwp5a/Pages/default.aspx | - TD 722 WP1 September 2016 |
| - ITU-R WP5C | - Pietro Nava WP 5C Chairman pietro.nava@huawei.com | - www.itu.int/en/ITU-R/study-groups/rsg5/rwp5c/Pages/default.aspx | - TD 292 GEN September 2016 |
| - ITU-R WP5B | - John Mettrop j.mettrop@caa.co.uk - John Shaw shawzone@gmail.com | - www.itu.int/en/ITU-R/study-groups/rsg5/rwp5b/Pages/default.aspx | - TD 355 WP1 Nov-Dec 2014 |
| - ITU-R SG6 | - John Shaw shawzone@usa.net | - http://www.itu.int/en/ITU-R/study-groups/rsg6/Pages/default.aspx | - TD 490 WP1 Jun-Jul 2015 |
| - ITU-T SG5 | - WP3/5 - Paolo Gemma paolo.gemma@huawei.com - Gianluca Griffa gianluca.griffa@telecomitalia.it - Ahmed Zeddam ahmed.zeddam@orange.com | - www.itu.int/en/ITU-T/studygroups/2013-2016/05/Pages/default.aspx | - TD 721 WP1 September 2016 - TD 357 WP1 Nov-Dec 2014 - TD 612 WP1 February 2016 |
| - ITU-T SG9 | - J.HiNoC Jingfei Cui cuijingfei@abs.ac.cn - Q1/9 Shigeyuki Sakazawa tkkim@etri.re.kr - Q7/9 TaeKyoon Kim tkkim@etri.re.kr - Liyue Zhu zhuliyue@abs.ac.cn - Q5/9 Mayumi Matsumot matsumoto@jlabs.or.jp - Q9/9 Jiansheng Zhang zhangjs@chinabtn.com | - www.itu.int/en/ITU-T/studygroups/2013-2016/09/Pages/default.aspx | - TD 395 WP1 Nov-Dec 2014 - TD 397 WP1 Nov-Dec 2014 |
| - ITU-T SG11 | - Martin Brand A1 Telekom Austria Austria Tel: + 43 664 663 1916 martin.brand@A1telekom.at | - www.itu.int/en/ITU-T/studygroups/2013-2016/11/Pages/default.aspx | - TD 137 GEN Nov-Dec 2014 |
| - ITU-T SG13 | - Gyu Myoung Lee - gmlee@kaist.ac.kr | - http://www.itu.int/en/ITU-T/studygroups/2013-2016/13/Pages/default.aspx | - TD 591 WP1 February 2016 |
| - ITU-T SG16 | Noah Luo Vice-Chairman SG 16 Huawei Technologies China Tel: +86 755 2878 6346 Fax: +86 755 2878 9166 noah@huawei.com | - www.itu.int/en/ITU-T/studygroups/2013-2016/16/Pages/default.aspx | - TD 725 WP1 September 2016 |
| - ITU-T SG17 | - Jonghyun Baek Rapporteur of Q6/17 Tel: +82 2 405 6540 Fax: +82 2 405 5219 - jhbaek@kisa.or.kr | - http://www.itu.int/en/ITU-T/studygroups/2013-2016/17/Pages/default.aspx | - TD 505 WP1 Jun-Jul 2015 |
| - ITU-T SG20 | - Nasser Al Marzouqi Chairman ITU-T SG20 Tel: +97 6118 468 Fax: +97 6118 484 Email: nasser.almarzouq@tra.gov.ae | - http://www.itu.int/en/ITU-T/studygroups/2013-2016/20/Pages/default.aspx | - TD 737 WP1 September 2016 |
| - ITU-D SG1 Q2/1 | - Laboni Patnaik Vice-Rapporteur Question 2/1 laboni@qualcomm.com | - http://www.itu.int/net4/ITU-D/CDS/sg/rgqlist.asp?lg=1&sp=2014&rgq=D14-SG01-RGQ02.1&stg=1 | - TD 719 WP1 September 2016 |
| - ISO/IEC JTC1/SC25 | - Dr. Walter P. von Pattay walter@pattay.com | - www.iso.org/iso/iso_technical_committee?commid=45270 | - TD 383 WP1 Nov-Dec 2014 |

__________________