Cost Comparisons of

Voice Frequency Cable  vs.  PCM System

Mr. G.Moumoulidis, OTE

Cost Comparison of Voice

Frequency Cable vs. PCM System

1.	The problem

Two analogue exchanges in a metropolitan area are linked by a voice frequency cable which is completely used up. There are two alternatives to provide facilities: either to use PCM systems equipped with the appropriate signalling interface unit on existing cable or to lay another voice frequency cable to expand the existing facilities.

The problem is to determine the “break-even distance” between the two alternatives.

To set up a PCM link, two voice-frequency pairs are needed. These pairs will be selected among those of the existing cable serving the traffic of the exchanges under consideration. The actual capacity K’ of PCM will be:

K'=K−2			(1)

Here, K is the capacity of PCM system.

2.	Evaluation of present worth of expenditures

2.1	PCM Equipment

Let CP be the cost of an installed pair of PCM terminals with the necessary signalling interface units and line terminating equipment. Charges associated to service life and maintenance costs have also been included.

Assume that CLE represents the cost per kilometer of the line equipment installed. Resultant costs due to service life and maintenance have been taken into account.

The cost of a complete PCM link is given by

CPCM=CP+CLE⋅l			(2)

Every time a system is used up, a new one should be installed to cater for the future demand. The period between successive installations is

tp=k'λ				(3)

l is the demand growth. This is considered constant over time.

For an infinite number of expansions, the cash-flow is illustrated in Figure 1.

Figure 1

The present worth of expenditures is written down

PWP=CPCM+CPCM(1+i)−tp+⋅=CPCM[1+(1+i)−tp+(1+i)−2tp+⋅]

The sum in brackets is the infinite geometric progression with ratio (1+i)−tp. We get:

PWp=CPCM1−(1+i)−tp					(4)

The above relationship can also be written down

PWP=CP+CLE⋅l1−e−rtp=CP+CLEl1−e−rKp'λ			(5)

where	r=ln(1+i).

2.2	Relay sets

Every year, the relay sets needed are 2λ.

Let CR be the total cost of an installed relay set. We point out that charges associated to service life and maintenance cost have been included. The cost of relay sets for one year is then:

2λCR

Assuming that the installation of relay sets takes place every year, the cash-flow is then as in Figure 2.

Figure 2

The present worth of expenditures is:

PWR=2λCR[1+(1+i)−1+(1+i)−2+(1+i)−3+⋅]PWR=2λCR1−(1+i)−1=2λCR1−e−r		(6)

2.3	Cable

The cost of installed cable of length l is:

Cc=(a+bs)l			(7)

a and b are the basic and incremental capital cost in which all kinds of charges have been taken into account. The optimal size is given by

S=λrln(1+p+2p)			(8)

where	p=arbλ

The provisioning time is

tc=Sλ			(9)

This means that the cable must be expanded every tc years.

The present worth of expenditures was found in previous paragraphs to be:

PWc=(a+bs)l1−e−rsλ			(10)

3.	Break-even distance

The break-even distance lo is the distance that the present worths of both alternatives become equal. Thus, we get

PWp=PWr+PWc			(11)

This equation eventually becomes:

CP+CLE⋅lo1−e−rk'=2λCr1−e−r+(a+bS)lo1−e−rsλ			(12)

By solving this equation with respect to lo , we get the break-even distance lo as a function of demand growth

lo=CP−Y(λ)X(λ)Z(λ)Y(λ)−CLE			(13)

where: 	X(λ)=1−e−rk'λ			(14)

Y(λ)=2λCR1−e−r			(15)

Z(λ)=a+bS1−e−rsλ			(16)

4.	Numerical example

Data regarding costs and service lines are as follows:

PCM Systems:

| - Cost of provisioning of two terminals equipped with the appropriate signalling interface units and line terminating equipment | 1800 MU |
| --- | --- |
| - Installation of two terminals and alignment of equipment | 500 MU |
| - Taxes | 20 % on the imported equipment |
| - Cost of one both-way regenerator | 100 MU |
| - Cost of housing for one regenerator | 25 MU |
| - Cost of installation and alignment of regenerator | 70 MU |
| - Average regenerator spacing | 1.81 km |
| - Operating plus maintenance | 5 % |
| - Service life | 20 years |
| - System capacity | 30 Ch |

Voice frequency cable:

| - Purchasing cost, basic cost | 100 MU/km |
| --- | --- |
| - Incremental cost | 4.8 MU/km/pair |
| - Digging cost | 500 MU/km |
| - Jointing cost | 40 MU/km |
| - Placement cost | 20 MU/km |
| - No taxes because cables are locally produced | -- |
| - Service life | 40 years |
| - Maintenance and operating cost | 2 % |

Relay sets:

| - Purchasing cost of relay sets | 20 MU/relay set |
| --- | --- |
| - Installation cost | 4 MU/relay set |
| - Taxes | 20 % of purchasing cost |
| - Service life | 20 years |
| - Maintenance | 7 % |
| - Interest rate | 10 % |

The scrap value for all dismantled equipment is assumed to be negligible.

Calculation of costs

PCM Systems

Total capital cost of terminal

Cp=1800⋅mp+(taxes + installation + alignment) 1+1(1+i)Tp−1

The mp for PCM is

mp=1+1(1+i)Tp−1+ui=1.675

Thus, we get

Cp=1800⋅1.675+[1800⋅0.2+500]1+1(1.1)20−1=4025MU

Total cost of line equipment per km

C = [Regenerators + housing] mp + [(Regenerators + housing) . taxes

+ Installation and alignment]1+1(1+i)Tp−1=(100+25)⋅1.675

+[(100+25)⋅0.2+70]⋅1+11⋅120−1=

321MU per regenerator spacing.

The cost of line equipment per km is:

CLE=3211.81=177MUkm

1.81 is the average repeater spacing.

Relay sets

Total capital cost CR=(purchasing cost)mr+(purchasing costtaxes+installation)1+1(1+i)Tr−1

The pvf is:

mr=1+11.120−1+0.070.1=1.875

Cr=20⋅1.875+(20⋅0.2+4)1+11.120−1=46.9MUrelayset

Cable

Total capital basic cost

a=(purchasing cost) mc + (digging + jointing + placement) 1+1(1+i)Tc−1

=100mc+(500+40+20)1+11.140−1

mc=1+11.140−1+0.020.1=1.222a=694MUkm

Total incremental capital cost

b= (purchasing) mc=4.8⋅1.222=5.87MUpairkm

With the above calculated costs and by using Eg (10), we elaborated Table 1, providing break-even distance as a function of l. Figure 3 illustrates the curve between l and lo.

| No. | l | tc | S | X(l) | Y(l) | Z(l) | 1o |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 17.7 | 90 | 0.410 | 4690 | 1492 | 4.80 |
| 2 | 10 | 13.5 | 130 | 0.230 | 9380 | 2016 | 6.50 |
| 3 | 15 | 11.5 | 180 | 0.162 | 14070 | 2573 | 7.30 |
| 4 | 20 | 10.1 | 200 | 0.124 | 18760 | 3046 | 8.46 |
| 5 | 25 | 9.2 | 230 | 0.101 | 23450 | 3508 | 9.47 |
| 6 | 30 | 8.5 | 250 | 0.085 | 28140 | 3952 | 10.30 |
| 7 | 35 | 7.9 | 280 | 0.073 | 32830 | 4390 | 11.35 |
| 8 | 40 | 7.45 | 200 | 0.064 | 37520 | 4818 | 12.36 |
| 9 | 45 | 7.1 | 320 | 0.057 | 42210 | 5238 | 13.32 |
| 10 | 50 | 6.7 | 340 | 0.052 | 46900 | 5652 | 13.60 |

Table 1

Figure 3