Forecasting Theories

(Solution to Exercises)

From TETRAPRO, edited by Mr. H. Leijon, ITU

SOLUTION AND EXERCISES - FORECASTING TECHNIQUES

1.	An object is to be forecasted.    The historical development was as follows:

| Year | Time Scale | Stock | Increase |  |
| --- | --- | --- | --- | --- |
| (at the end of) | (t) | (y) | Absolute % |  |
| 1968 | 1 | 583 |  |  |
| 1969 | 2 | 615 | 32 | 5.5 |
| 1970 | 3 | 646 | 31 | 5.0 |
| 1971 | 4 | 697 | 51 | 7.9 |
| 1972 | 5 | 738 | 41 | 5.9 |
| 1973 | 6 | 802 | 64 | 8.7 |
| 1974 | 7 | 844 | 42 | 5.2 |

Make a forecast for the expected stock 5 and 10 years ahead, in four different ways:

- Assuming an unchanged and constant absolute increase per year, based on the historical average increase.

- Assuming an unchanged and constant percentage increase per year, based on the total increase over the past six years.

- By adapting a linear trend line to the historical data.

- By adapting an exponential trend to the historical data.

Plot the historical data in a diagram and show in the same diagrams the two trend lines c) and d) expanded to year 16 (6 + 10), and also the two other forecasts a) and b).

Compare the outcome from the forecasts.

Check the trend lines as concerns statistical reliability. (Correlation coefficient, T - Test and Durbin - Watson Test)

Discuss the credibility of the forecasts.

1.	SOLUTION

a.	844−5836=43.5perannum

y(1979) =1061.5y(1984)=1279

b.	84458316=1.447716=1.0636

y(1979)=844⋅1.06365=1148.8

y(1984)=844⋅1.063610=1563.6

c.	y=a+b⋅xa=525.14b=44.607r=0.9945y(12)=1060.4y(17)=1283.5

d.	y=a+b⋅x

a=6.2946ea=541.7b=0.06336eb=1.0654COR=0.9978y(12)=7.054ey=1158.6y(17)=7.372ey=1590.4

|  | a. | b. | c. | d. |
| --- | --- | --- | --- | --- |
| y (12) | 1062 | 1149 | 1060 | 1159 |
| y (17) | 1279 | 1564 | 1283 | 1590 |

“a” is close to “c”
(slightly higher)

T-Test gives:

s2=Y2−a⋅y−b⋅x⋅yn−2

s2=3521423−25.14⋅4925−44.607⋅209497−2

s2=123.8786

s=11.1301

x−x2=x2−n⋅x2=28

T=b⋅(x−x)2s=112.2		(Large enough!)

The Durbin Watson Test gives	(Case c)

DW=2−2⋅wv

w=(yi−yi)⋅(yi+1−yi+1)=134v=(yi−yi)2=618

| x | y |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 583 | 570 | + 13 | DW = 1.57 |
| 2 | 615 | 614 | + 1 |  |
| 3 | 646 | 659 | - 13 |  |
| 4 | 697 | 704 | - 7 | (small enough!) |
| 5 | 738 | 748 | - 10 |  |
| 6 | 802 | 793 | + 9 |  |
| 7 | 844 | 837 | + 7 |  |

Confidence limits

u=s2⋅1+1n+t0−t2t−t2

s2=123.8786

|  | u |  | y |  |  |
| --- | --- | --- | --- | --- | --- |
| 12 | 424.73 | 20.61 | 1060 | 1019 | 1101 |
| 17 | 889.27 | 29.82 | 1283 | 1223 | 1372 |

Remarkably small confidence intervals!

Comments

The historical data seems to be reliable, the correlation coefficient was quite near unity (r = 0.9945) for the linear regression approach. The T-Test and the DW Test gave quite good values. The confidence levels for y(12) and y(17) were quite narrow for the linear regression.

All four forecasts are quite similar to y(12) but varies more for y(17). For y(17) cases a and c give about the same result, so does b and d. It may not be possible to say which level is the correct one at t=17 since the historical data concerns too few years to permit a reliable forecast 10 years ahead.

2.	Calculate the overall connection forecast for a local telephone network for 1983 (31.3.83)

Given:

GROWTH CHART OF DEMANDS IN A LOCAL TELEPHONE NETWORK

| As on | Equipped capacity | Working connect | Waiting list | Total demand |
| --- | --- | --- | --- | --- |
| 31/3/72 | 11800 | 9825 | 1901 | 11,726 |
| 31/3/73 | 12000 | 11114 | 1781 | 12,925 |
| 31/3/74 | 12100 | 11458 | 3555 | 15,013 |
| 31/3/75 | 12500 | 11530 | 5106 | 16,636 |
| 30/9/75 | 12500 | 11653 | 5662 | 17,315 |
| 31/3/76 | 12800 | 12275 | 2587 | 14,862 |
| 31/3/77 | 14150 | 13683 | 1624 | 15,307 |
| 31/3/78 | 15500 | 14437 | 1893 | 16,330 |

2.	SOLUTION

The total demand during the period 31/3/72 - 31/3/78 has been defined as the sum of working connections and the waiting list.    If we accept this definition of the demand, there are different ways of estimating the demand in 31/3/83.

a.	The increase from 31/3/72 to 31/3/78 is:

16330 - 11726 = 4604 for the 6 years.

If we assume the same annual increase in the future, we can expect the demand to increase with:

46046⋅5=3835

connections in the following 5 years.

The demand for connections will be:

16330 + 3835 = 20165    by 31/3/83

b.	Considering the values for 31/3/72 and 31//78 we find annual increase

16330117261/6=1.05675

If we assume the same percentual increase during the next 5 years, we arrive at.

16330⋅(1.05675)5=21520

Demanded connections by 31/3/83

c.	Applying linear regression on the given values we obtain:

| x | y |  |
| --- | --- | --- |
| 72 | 11726 |  |
| 73 | 12925 | we obtain |
| 74 | 15013 |  |
| 75 | 16636 |  |
| 75.5 | 17315 |  |
| 76 | 14862 |  |
| 77 | 15307 |  |
| 78 | 16330 |  |

and    y (83) = 20520

d.	Applying exponential regression lny=a+bx

gives    y (83) = 22077

Conclusion:    The four forecasts gave

- 20165

- 21520

- 20520

- 22077

It is impossible to conclude which forecast is most reliable.

Remarks: A waiting list may not always reflect the real unsatisfied demand, since it depends on how waiting subscribers are registered by the administration. It also depends on if people consider it worth while, or not, to ask for subscription.    A study of the waiting list and the annual increase of working connections shows that the waiting times for subscription is about three years if the list is handled in order of arrival.

The values for 30/9/75 are higher than the others and should be further investigated, before being included in the statistics. Maybe it depends on seasonal fluctuations on the waiting list?

3.	Forecast the total originating traffic for December 1986 in a local telephone exchange.

Given:

- Traffic records for January 1979 - December 1981 (Table A)

- Calling rate records for January 1979 - December 1981 (Table B)

- Connection forecast for December 1986 = 2969

(1170 subscribers in January 1979 and 1600 in December 1981)

A.	TOTAL ORIGINATING TRAFFIC

| MONTH | 1979 | 1980 | 1981 |
| --- | --- | --- | --- |
| JANUARY | 38.6 | 39.4 | 45.6 |
| FEBRUARY | 37.9 | 43.7 | 46.2 |
| MARCH | 42.1 | 48.7 | 47.2 |
| APRIL | 40.6 | 43.8 | 46.2 |
| MAY | 40.1 | 40.2 | 45.6 |
| JUNE | 38.1 | 42.6 | 48.5 |
| JULY | 37.7 | 41.1 | 44.4 |
| AUGUST | 39.9 | 44.2 | 47.4 |
| SEPTEMBER | 40.4 | 41.0 | 49.1 |
| OCTOBER | 40.7 | 43.8 | 48.7 |
| NOVEMBER | 40.8 | 41.8 | 45.0 |
| DECEMBER | 42.2 | 49.5 | 49.5 |

B.	OVERALL ORIGINATING CALLING RATES 	(Erlang/subscriber)

| MONTH | 1979 | 1980 | 1981 |
| --- | --- | --- | --- |
| JANUARY | .033 | .031 | .033 |
| FEBRUARY | .033 | .034 | .033 |
| MARCH | .036 | .038 | .034 |
| APRIL | .035 | .033 | .033 |
| MAY | .034 | .030 | .032 |
| JUNE | .032 | .032 | .035 |
| JULY | .031 | .031 | .032 |
| AUGUST | .033 | .032 | .034 |
| SEPTEMBER | .034 | .030 | .035 |
| OCTOBER | .034 | .032 | .032 |
| NOVEMBER | .033 | .030 | .028 |
| DECEMBER | .034 | .036 | .031 |

3.	SOLUTION

A study of table A shows that the monthly values are subject to seasonal variations. It can then be doubtful if this material is suitable for applying regression analyses on the monthly values.

Table B shows average originating traffic per subscriber. The variation is very little. The average of all 36 months is 0.032861 and the standard deviation only 0.001973. Therefore, if the connection forecast is already given, we can simply estimate the traffic in 1986 as the number of connections multiplied with the above average calling rate.

A(Dec.86)=2969⋅0.032861=97.6erlang

Table B will not be used further!

As concerns table A, it can be seriously discussed if only 3 years statistics permits extrapolation 5 years ahead. It would have been desirable to have 5-10 years statistics for this forecast.

A linear regression for the 36 monthly values give

y=38.18+0.2821⋅x(r=0.8188)

and    y (Dec 86) = y (96) = 65.26 erl.

Taking the average values for each year we arrive at:

|  | Year | Average | Dec. | Dec./Ave. |
| --- | --- | --- | --- | --- |
|  | 79 | 36.75 | 42.2 | 1.148 |
|  | 80 | 43.32 | 49.5 | 1.143 |
|  | 81 | 46.96 | 49.5 | 1.054 |
| Average | 79 - 80: | 42.34 | 47.07 | 1.112 |

Increase in annual average 79-81

46.9636.7512=1.1304

If same increase up till 1986 then annual average 1986=46.96⋅(1.1304)5=86.68

If December values still are 11.2% higher than annual average:

y(Dec.86)=86.68⋅1.112=96.4

This agrees rather well with the average value derived from the calling rates combined with the connection forecast.

4.	Consider a telephone network with two exchanges.

Given:

a.	The present traffic interest Aij(0)

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | Ai. (0) |
| 1 |  | 10 | 20 | 30 |
| 2 |  | 30 | 40 | 70 |
| A.j (0) |  | 40 | 60 | 100 |

b.	Forecasted values on the future total originating and terminating traffics per exchange [Ai.(t); A.j(t)]

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | Ai.(t) |
| 1 |  | ? | ? | 45 |
| 2 |  | ? | ? | 105 |
| A.j(t) |  | 50 | 100 | 150 |

Estimate the traffic values Aij(t)by using Kruithof’s method.

4.	SOLUTION

Iteration 1	Row multiplication. Ai.(t)is distributed according to the present traffic interest.

Result: Aij(1)

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | sum |
| 1 |  | 15 | 30 | 45 |
| 2 |  | 45 | 45 | 105 |
| sum |  | 60 | 90 |  |

Aij(1)=Aij(0)Ai.(0)⋅Ai.(t)

Iteration 2	Column multiplication. A.j(t)is distributed according to the traffic interest matrix given by iteration 1.

Result: Aij(2)

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | sum |
| 1 |  | 12.5 | 33.33 | 45.83 |
| 2 |  | 37.5 | 66.67 | 104.17 |
| sum |  | 50 | 100 |  |

Aij(2)=Aij(1)A.j(1)⋅A.j(t)

After column multiplication the sums of the rows differ from forecasted values.

Iteration 3	Row multiplication Ai.(t)is distributed according to the traffic interest matrix given by iteration 2.

Result: Aij(3)

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | sum |
| 1 |  | 12.27 | 32.73 | 45 |
| 2 |  | 37.80 | 67.20 | 105 |
| sum |  | 50.07 | 99.93 |  |

Aij(3)=Aij(2)Ai.(2)⋅Ai.(t)

Iteration 4	Column multiplication Aj(t) is distributed according to the traffic interest matrix given by iteration 3.

Result: Aij(4)

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | sum |
| 1 |  | 12.25 | 32.75 | 45 |
| 2 |  | 37.75 | 67.25 | 105 |
| sum |  | 50 | 100 | 150 |

Aij(4)=Aij(3)Aj(3)⋅Aj(t)

After four interactions the sums of the rows and the columns are equal to the forecasted values. We can put

Aij(t)=Aij(4)

Note:	Ai.=jAij ; Aj=iAij

5.	Calculate the traffic interest between two exchanges in a local area.

Given:

- A local area is divided into traffic areas No. 1, 2, 3, 4, ... The future traffics between all traffic areas are forecasted.

- The local area is to be divided into exchange areas. The exchange areas do not coincide with the traffic areas.

- Try to calculate the expected future traffic from exchange A to exchange B.

- We have the following information:

Exchange A will get

- 5000 subscriber lines from traffic area I, which totally has 10,000 subscriber lines.

- 8000 subscriber lines from traffic area II, which totally has 12,000 subscriber lines.

Exchange B will get

- 9000 subscriber lines from traffic area III, which is all of the subscriber lines that are there.

- 2000 subscriber lines from traffic area IV, which totally has 6000 subscriber lines.

- We also know from the forecast the total expected traffic from each one, to each one of the involved traffic areas, as following:

| From traffic area No. | To traffic area No. | Total expected traffic |
| --- | --- | --- |
| I | III | 100 erl. |
| I | IV | 90 erl. |
| II | III | 105 erl. |
| II | IV | 95 erl. |

5.	SOLUTION

A simple approach which we can easily handle with manual calculations, is to assume, that the traffic from one subscriber in a specific traffic area to one subscriber in another specific traffic area is constant.

Calculate the traffics from one subscriber in traffic area I, to one subscriber in traffic area III, etc.:

| From traffic area No. | To traffic area No. | Traffic between two subscribers |
| --- | --- | --- |
| I | III | r = |
| I | IV | s = |
| II | III | t = |
| II | IV | u = |

The expected total traffics from exchange A to exchange B can now be calculated as:

A=5000⋅9000⋅r+5000⋅2000⋅s+8000⋅9000⋅t+8000⋅2000⋅u=50.00+15.00+70.00+21.11=156.11erl.

6.	Estimation of traffic interests between exchanges: The objective of the example is to illustrate how a traffic matrix for exchanges can be computed from a given traffic matrix for traffic areas.

Given:

An area consisting of three traffic areas: I, II, III:

Traffic Area 1

| No. of subscribers | 10,000 |  |
| --- | --- | --- |
| Total originated traffic/sub | 0.06 | erl. |
| Distribution of this traffic | 60 % | I |
|  | 25 % | II |
|  | 15 % | III |

Traffic Area II

| No. of subscribers | 5,000 |  |
| --- | --- | --- |
| Total originated traffic/sub | 0.05 | erl. |
| Distribution of this traffic | 50 % | I |
|  | 30 % | II |
|  | 20 % | III |

Traffic Area III

| No. of subscribers | 5,000 |  |
| --- | --- | --- |
| Total originated traffic/sub | 0.04 | erl. |
| Distribution of this traffic | 50 % | I |
|  | 25 % | II |
|  | 25 % | III |

The area is served by a number of exchanges 1, 2, ... etc.

Exchange 1

| Total No. of subscribers | 8,000 |
| --- | --- |
| No. of subscribers belonging to traffic area I | 5,000 |
| No. of subscribers belonging to traffic area II | 3,000 |

Exchange 2

| Total No. of subscribers | 6,000 |
| --- | --- |
| No. of subscribers belonging to traffic area I | 4,000 |
| No. of subscribers belonging to traffic area III | 2,000 |

Task:

Calculate the expected total traffic flow from exchange 1 to exchange 2.

Hint:

Start with calculation of traffic from one subscriber in traffic area I to one subscriber in traffic area II, etc.

6.	SOLUTION

| a. | 1II ® 2I: | 0.06 0.60 |  |
| --- | --- | --- | --- |
| b. | 1II ® 2III: | 0.06 0.15 |  |
| c. | 1III ® 2I: | 0.05 0.50 |  |
| d. | 1III ® 2III: | 0.05 0.20 |  |
|  | Total traffic |  | 132 erl. |

The expected traffic flow from exchange 1 to exchange 2 is 132 erlang.

7.	Present traffic matrix Aij(0) has been estimated:

|  | To | Exchange No. j |  |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | Sum |
| 1 |  | 25 | 30 | 45 | 100 |
| 2 |  | 35 | 55 | 110 | 200 |
| 3 |  | 60 | 85 | 155 | 300 |
| Sum |  | 120 | 170 | 310 | 600 |

The number of main lines per exchange in year t has been forecasted:

| Exchange No. | Ni (0) | Ni (t) |
| --- | --- | --- |
| 1 | 2000 | 3000 |
| 2 | 3500 | 3500 |
| 3 | 6800 | 7500 |

The main lines have not been classified into different categories since the proportion of different subscribers is expected to be the same in the future.

The total originating and terminating traffic per exchange is therefore forecasted from the formulae:

Ai.(t)=Ni(t)⋅Ai(0)Ni(0)A.j(t)=Nj(t)⋅A.j(0)Nj(0)

| Exchange No. |  |  |
| --- | --- | --- |
| 1 | 150.0 | 180.0 |
| 2 | 200.0 | 170.0 |
| 3 | 331.9 | 341.9 |
| Sum | 681.9 | 691.9 |

Since the two sums of Ai.(t) and A.j(t) differ, we can use the mean value of these sums as an estimate of A⋅(t) and adjust the Ai.(t) andA.j(t). This will give:

| Exchange No. |  |  |
| --- | --- | --- |
| 1 | 151.1 | 178.7 |
| 2 | 201.5 | 168.8 |
| 3 | 334.3 | 339.4 |
| Sum | 686.9 | 686.9 |

- Now draw a traffic matrix for the future year t and fill in the total traffics calculated above.

- Calculate the different point-to-point traffics by using the weighted growth methods, from known data about present and future number of main lines and the present point-to-point traffics. The type of weights is your own choice.

- Fill in these values in a new traffic matrix and calculate the sums of columns and rows. You will see that these sums do not agree with the values in the matrix you first drew.

- If we consider the traffic per main line to be constant during the forecast period, then the obtained point-to-point traffics must be corrected so that their sums agree with the forecasted total traffics.

This correction can be made by Kruithof’s Double Factor method.

7.	SOLUTION

The weighted growth factor models can be used to forecast the point to point traffic.

The growth factors are equal to:

| Exchange 1 |  |
| --- | --- |
| Exchange 2 |  |
| Exchange 3 |  |

a.	Forecast according to Rapp’s formula 1:

|  | to |  | Exchange |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | sum |
| Exch. 1 |  | 37.5 | 38.1 | 62.4 | 138.0 |
| " 2 |  | 44.4 | 55.0 | 113.5 | 212.9 |
| " 3 |  | 83.1 | 87.7 | 170.5 | 341.3 |
| sum |  | 165.0 | 180.8 | 346.4 | 692.2 |

b.	Forecast according to Rapp’s formula 2:

|  | to |  | Exchange |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | sum |
| Exch. 1 |  | 37.5 | 38.6 | 65 | 141.1 |
| " 2 |  | 45.1 | 55.0 | 112.0 | 212.1 |
| " 3 |  | 86.7 | 92.6 | 170.5 | 349.8 |
| sum |  | 169.3 | 186.2 | 347.5 | 703.0 |

c.	Forecast according to APO’s formula:

|  | to |  | Exchange |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | sum |
| Exch. 1 |  | 37.5 | 38.8 | 62.8 | 139.1 |
| " 2 |  | 45.2 | 55.0 | 113.6 | 213.8 |
| " 3 |  | 83.8 | 87.8 | 170.5 | 342.1 |
| sum |  | 166.5 | 181.6 | 346.9 | 695.0 |

For exchange 1 one can see that the total originating traffic per main line has decreased from the present value of 0.050 to between 0.046 and 0.047.

For exchange 2, on the other hand, the total originating traffic per main line has increased from 0.057 to about 0.060.

As the traffic per main line in this case was considered as constant during the forecast period, the obtained matrices have to be reconciled with the estimate of total originating and total terminating traffic for each exchange.

Kruithof’s Double Factor Method is used for this purpose.

The reconciled traffic matrix forecast is:

- according to Rapp’s formula 1:

|  | to |  | Exchange |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | sum |
| Exch. 1 |  | 44.5 | 39.1 | 67.5 | 151.1 |
| " 2 |  | 45.8 | 49.0 | 106.7 | 201.5 |
| " 3 |  | 88.4 | 80.7 | 165.3 | 334.3 |
| sum |  | 178.7 | 168.8 | 339.4 | 686.9 |

- according to Rapp’s formula 2:

|  | to |  | Exchange |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | sum |
| Exch. 1 |  | 43.02 | 38.3 | 69.6 | 151.1 |
| " 2 |  | 46.2 | 48.5 | 106.7 | 201.5 |
| " 3 |  | 89.2 | 82.0 | 163.1 | 334.3 |
| sum |  | 178.7 | 168.8 | 339.4 | 686.9 |

- according to APO’s formula :

|  | to |  | Exchange |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | 1 | 2 | 3 | sum |
| Exch. 1 |  | 44.0 | 39.5 | 67.6 | 151.1 |
| " 2 |  | 46.2 | 48.8 | 106.5 | 201.5 |
| " 3 |  | 88.5 | 80.5 | 165.3 | 334.3 |
| sum |  | 178.7 | 168.8 | 339.4 | 686.9 |

8.	Suppose that a busy hour trunk traffic forecast for exchange “A” of 75 erlangs is to be distributed over 5 routes to exchanges “B”, “C”, “D”, “E” and “F”, and for which no traffic data is available. Suppose also that the connection forecasts for the exchanges are:

| A | 10 000 | (co) |
| --- | --- | --- |
| B | 5 000 | (c1) |
| C | 7 000 | (c2) |
| D | 4 000 | (c3) |
| E | 2 000 | (c4) |
| F | 10 000 | (c5) |

And that the distances from “A” are:

| B | 20 miles | (d1) |
| --- | --- | --- |
| C | 30 miles | (d2) |
| D | 10 miles | (d3) |
| E | 10 miles | (d4) |
| F | 50 miles | (d5) |

The situation is illustrated in the diagram below.

Estimate the distribution of the total trunk traffic over the five different routes A to B, A to C, ..., etc.

Hint:

The traffic forecast for a route can be obtained from

ti=co⋅cidi2j=1nco⋅cjdj2⋅T		where T is the total traffic.

8.	SOLUTION

c0⋅c1d12=10,000⋅5,000400=125,000

c0⋅c2d22=10,000⋅7,000900=77,778

c0⋅c3d32=10,000⋅4,000100=400,000

c0⋅c4d42=10,000⋅2,000100=200,000

c0⋅c5d52=10,000⋅10,0002,500=40,000

so that	j=15c0⋅cjdj2=842,778

Hence estimate traffic from		A to B    =    125000842778⋅75=11.1erl.

"	"		"		A to C    =    77778842778⋅75=6.9erl.

"	"		"		A to D    =    400000842778⋅75=35.6erl.

"	"		"		A toE    =    200000842778⋅75=17.8erl.

"	"		"		A toF    =    40000842778⋅75=3.6erl.

These precasted traffic values can then be used for dimensioning the routes.