-

Forecasting Theories

(Exercises included)

From TETRAPRO, edited by Mr. H. Leijon, ITU

Forecasting Theories

1.	GENERAL ASSUMPTIONS FOR FORECASTING

1.1	Why forecast ?

1.1.1	Telephone density and economic development

1.1.2	Purpose of forecasts

1.1.3	Forecast, planning and work programme

1.2	When is a forecast needed ?

1.3	What is to be forecast ?

1.3.1	Forecasting subjects

1.3.2	Scope of forecast

1.3.3	Data needed for planning

1.4	How is forecasting done ?

1.4.1	Basic requirements

1.4.2	Forecast check

1.4.3	How do we start ?

2.	FORECASTING METHODS

2.1	Introduction
2.2	Statistical demand analysis

2.2.1	Two-variable regression

2.2.2	Multiple regression

2.3	Trend methods

2.3.1	Time series analysis

2.3.2	Trend extrapolation

2.4	Individual judgment

2.5	Other methods

3.	FORECASTING FOR EXCHANGE PLANNING

4.	FORECASTING FOR NETWORK PLANNING

4.1	Introduction

4.2	Models for total traffic

4.3	Point-to-point forecast

4.4	Kruithof’s double factor method

5.	CONCLUSIONS

6.	REFERENCES

7.	EXERCISES

1.	GENERAL ASSUMPTIONS FOR FORECASTING

1.1	Why forecast ?

1.1.1	Telephone density and economic development

The telecommunication facilities of a country depend on the economic level in that country. In order to develop further the economy, certain basic telecommunication facilities are necessary. This holds for all countries. The correlation between the number of telephones per inhabitant and the Gross Domestic Product per capita exists, as shown in Figure 1.1/1. Certain deviations occur as a result of differences in the economic structure of different countries. It is easily understood that a country with an economy mainly based on agriculture needs fewer telephones per head than a highly industrialized country.

Figure 1.1/1 : Telephone density and Gross Domestic Product (GDP) per capita in some countries, 1993

The demand for telephones depends primarily upon the occupational specialisation of the population. The need for telephones is low in a self-supporting village, but increases as people leave the agricultural sector for other activities (industry, commerce, public services, etc.). Government and private commercial activities require telecommunications to function. The concentration of more and more people in urban areas may also give an increased demand for residential telephones, but this only concerns those persons who have reached a sufficiently hight standard of living. A certain demand for communication with rural areas will also follow from urbanization.. The demand is, of course, also dependent on national preferences and habits, as well as on the telephone tariffs. To avoid the influence of the latter, all investigations and prognosis should be made on the assumption that the telephone tariffs are not prohibitive but only sufficient to support adequate maintenance and sound expansion of the telephone network.

To estimate the future telecommunication demand, the expected general development and especially the economic development must be considered. Published data on the current economic status and information about plans for economic development are useful.

1.1.2	Purpose of forecasts

The extension of telephone services requires the provision of telephone apparatus, subscriber line plant, exchange equipment, junction and trunk circuits. However, there will always be a certain time lag between the identification of a need - or rather a future need - and the moment it can be met. The length of time between the identification of and the provision for this need is considerable. To avoid long waiting periods and high congestion, it is desirable for needs to be determined well in advance. This makes it possible to extend the plant at the right moment because the necessary action can be taken at the appropriate time.

A forecast will, therefore, primarily produce accurate estimates of the future demand for telecommunication facilities.

1.1.3	Forecast, planning and work programme

Forecasts provide the basis for the plans. The plans are considered by the management which makes its decisions. Following the decisions, a work programme is formulated. The work programme requires detailed planning of what needs to be done until all the equipment becomes operative. There is long-term, medium-term and short-term planning, each of which has its own requirements as concerns the details to be specifided at which point, if the programme’s time schedule is to be followed. Each type of planning must use, more or less, detailed forecasts of the required quantities.

We will use the following definitions:

Forecast :	A forecast is a prediction of the future demand, generally expressed in quantities.

Plan :	A plan is a proposal for action and estimates the cost of carrying out the plan. It may include alternative lines of action.

Decision :	The management decides what action shall be taken. It approves a plan which is then converted into a work programme.

Work programme :	A work programme is based on an approved plan and defines action to be taken as well as its timing.

Planning	A work programme generally requires detailed planning of all action to be taken to realise the work programme. The timing of each action is here of the utmost importance. More or less detailed work plans must be ready at a given time.

The relationship between the above concepts is illustrated in Figure 1.1/2

Figure 1.1/2 : Relationship between forecast and plan

Since the work programme and investments are based on forecasts, it is very important for these to be as accurate as possible. It is also desirable that the forecaster defines the degree of uncertainty of the forecast. This enables the planner to know when he can be flexible.

The situation in many developing countries may be such that the demand is far greater than the supply. In such a case, practically all extensions will be used immediately without essentially decreasing the gap between supply and demand. It is very difficult to forecast the demand in such cases since the existing demand cannot easily be measured or estimated. It is still the task of the forecaster to estimate this demand so that the decision-maker will know the effect on the demand/supply situation of his actions. Dealing with such situation is also a matter of what policy the administration will follow. Will it give priority to business subscribers, to development of local exchange areas, or to long distance services, etc.?

1.2	When is a forecast needed ?

The forecast periods depend on the planning periods. They, in turn, depend on the delivery times of the various types of equipment needed. We give a few examples below:

| Subscriber apparatus | 1-2 years |
| --- | --- |
| Exchange switching equipment | 3-4 years |
| Subscriber cables | 6-10 years |
| Ducts in local networks | 10-15 years |
| Buildings | 10-20 years |
| Sites for buildings | up to 50 years |

Planning, as well as forecasts, are usually divided into:

| Long-term planning/forecast | more than 10-15 years |
| --- | --- |
| Medium-term planning/forecast | 5-10 years |
| Short-term planning/forecast | less than 5 years |

It follows that the various types of forecast and planning concern different types of equipment dependent on when this equipment has to be used. Clearly, a short-term forecast must be more detailed as it serves the short-term planning in which every detail of the plant has to be specified.

No single forecasting method will be adequate for all time spans. Accordingly, different methods must be used for different periods, and this raises the problem of adjustment when forecasts for overlapping or non-overlapping time periods are derived with different methods since discrepancies may occur between the growth curves.

There is no standard procedure for matching contradictory forecasts. In most cases, judgment based on experience is used and a curve is drawn which bridges the inconsistency between the forecasts. This is shown in Figure 1.3/1.

Figure 1.3/1 : Matching contradictory forecasts

1.3	What is to be forecast ?

1.3.1	Forecasting subjects

Forecasting concerns four main areas:

| Accommodation | Site |
| --- | --- |
|  | Buildings |
| Lines | Subscriber direct exchange lines (subscriber network forecast) |
|  | Junction network |
|  | Trunk network |
| Equipment | Subscriber apparatus |
|  | Automatic switching equipment |
|  | Switchboard equipment |
| Numbering | Exchange numbering schemes (long-term strategy) |

Forecasts generally present the expected future number of direct exchange lines (DEL’s) and the future traffic, the latter being more or less detailed. Consequently, there are:

- subscriber forecasts;

- traffic forecasts.

1.3.2	Scope of forecast

A forecast may concern the total for a given entity, such as a country, a certain part of a country, a metropolitan area, etc. In this case, it is called an overall forecast and contains very few details for this entity. It is usually only sufficient for long-term planning purposes. A detailed forecast provides detailed data of the expected future development. It is separately made for each part of the country where the division is such that the units become workable.

Only the detailed subscriber forecast specifies the exact location of each subscriber.

A detailed traffic forecast specifies all future traffic within the area concerned. It is generally based on subscriber forecast and on measurement.

1.3.3	Data needed for planning

Dependent on the planning need, results of the forecast should be presented in different ways. We give the following examples:

Subscriber distribution within an area

- Exact location of the individual subscribers.

- Number of subscribers in each grid of a grid map.

- Number of subscribers per traffic area in the area concerned.

- Number of subscribers per subscriber category in each traffic area.

Traffic

- Originating and terminating traffic per subscriber.

- Originating and terminating traffic per subscriber in each traffic area.

- Originating and terminating traffic per subscriber for each subscriber category.

- Traffic streams between areas.

We may define:

A  =  total traffic for a group of subscribers

N  =  number of subscribers in the group

a  =  traffic per subscriber

y  =  call intensity in the group

h  =  average holding time

Then, A = y h, but also A = N a

Traffic A can now be forecasted in different ways:

- Directly, for example, by trend extrapolation of recorded A values for the past years.

- Indirectly, by first forecasting either N or a, or alternatively, y and h.

As concerns forecasting the traffic for a group of subscribers, it is generally preferable to make a forecast for each category of subscriber before the total traffic is estimated. The reason for this is that proportions between the different categories may change in the future.

1.4	How is forecasting done ?

1.4.1	Basic requirements

There are two main requirements for sound forecasting:

- An adequate supply relevant and accurate information of the past should be available. This will generally consist of records from measurements on existing equipment (connections, calls, traffic, etc.) supplemented by general background information.

- Systematic use of these data in the forecast.

- An educated guess as to future development. This estimate of future development may be an extrapolation of past development, sometimes adjusted to take into acccount available background information. The forecaster needs accurate historical data to improve his forecast.

Consequently, the basis for forecasting is the study of the past. The more we understand and mathematically describe the past development, the better are the chances of making a correct forecast.

It must also be stressed that the degree of uncertainty of forecast data should be given so that those who are going to use this data may take account of these uncertainties.

1.4.2	Forecast check

There is always a reason to be critical of the applicability of a forecast. The following questions should be asked:

- Is the forecast relevant (valid) ?

- How accurate is the forecast ?

- Is the outcome credible ?

The relevance of a forecast depends in some cases on the correct use of assumptions for future development.

The accuracy of a forecast may depend on the statistical precision of the historical data and on the extrapolation method used. Whether a forecast is credible or not is often a matter of personal judgment. Here, the human mind may, however, have its limitations when it comes to imagining future growth. Many forecasts have, therefore, been decreased in size since the forecaster could not envisage such growth. The development of many networks has been hampered because of the forecaster’s lack of imagination.

When judging the credibility of a forecast, it must be remembered that:

- Forecasts concerning population, economic and industrial development, etc., may be misleading.

- Available telecommunication statistics for past periods may contain errors and may not have always been collected under the same conditions.

- Available data on historical development may only cover a limited period.

- Past and present relationships between studied variables are not necessarily true in the future.

1.4.3	How do we start ?

The forecasting process can be divided into the following parts:

Definition of the problem

The purpose of and assumptions for forecasts have to be determined.

Collection of basic data

Various sources for basic data are investigated. The population and economic growth are studied. Results of recent forecasts are essential facts.

Choice of the forecast method

The method has to be chosen with regard to available information and required accuracy, etc.

Analysis and establishment of forecasts

The analysis consists of methodological preparation of the basic data and evaluation of the results obtained.

Documentation

The forecast has to be presented in an easily understandable format. The result should contain alternative forecasts. Besides the most likely forecast, there should be one optimistic and one pessimistic forecast, thus telling the planner where the upper and lower limits may be.

2.	FORECASTING METHODS

2.1	Introduction

Almost all forecasting methods assume that the future will in some way resemble the past. This may be interpreted in various ways. For example:

- For time trend forecasting methods, it is assumed that development will follow a curve which has been fitted to existing historical data.

- When using explicit relationships between demand and various determining factors, these will remain the same in the future.

- Comparing various steps of telecommunication development, it is assumed that the less-developed country (or area) will develop to the level of the more developed one.

- When applying personal (subjective) judgment in the forecast, the future will resemble the person’s previous knowledge and experience of past developments.

Although the future often resembles the past in this sense, it is never an exact reproduction. Forecasts must, therefore, never be accepted uncritically. This means that forecasts - as produced by some methods - are frequently modified before being accepted for planning purposes. While such amendments may appear subjective, they are necessary whenever there is reason to believe that the future will show divergences from the past, not accounted for by the forecasting method used. Examples of such situations are tariff changes, radical improvements of the service, etc.

It follows that the assumptions used in forecasting should be questioned, whenever possible changes occur in the environment in which the telephone system operates.

The importance of accurate and detailed data cannot be overemphasized. The quality of a forecast depends directly on the quality of the historical data used. If reliable data are not available, then the forecaster should first establish adequate data collection processes before applying forecasting methods. Many of the methods discussed in this chapter would be difficult, if not impossible, to use without adequate historical data.

2.2	Statistical demand analysis

One can assume that the development of the number of direct exchange lines in a country follows certain patterns. For example, it ought to be dependent on the number of inhabitants, living standard, technical and economic development, etc. If some variables are logically related to the telephone development, those variables may be used to explain the development. In the following, some such variables are described:

Population size - as the population grows, the number of consumers increases and the demand for subscriber lines increases.

Living standard - one measure of the living standard is the Gross Domestic Product per capita. In centrally planned economies, the Net Material Product per capita may be used. Both values should be converted into a fixed price level. When the living standard increases, the demand for subscriber lines also generally increases.

Building activity - as a measure of the change of structure in society due to urbanisation, high living standard, population development, etc., one can use the number of new offices and departments. The building activity can be used to estimate the future demand for subscriber lines.

Price - a high price for using a telephone (i.e. entrance fee, subscription cost and call charges) can be assumed to reduce the demand for subscriber lines.

The choice of explaining variables is dependent on the availability of reliable forecasts for them. Population and economic development are the most common explanatory variables for the number of subscriber lines in a country.

Figure 2.2/1 shows the telephone density and the GDP per capita for Sweden 1900-1965.

On a logarithmic scale, the relationship is approximately linear during the periods 1900-1915 and     1920-1965.

Figure 2.2/1 : Telephone density and GDP per capita in Sweden, 1900-1965

Statistical demand analysis by econometric modelling is a technique designed to obtain explicit quantitative relationships between interest and other variables which are believed to influence it.

An econometric model of demand is thus a mathematical representation of customer behaviour, and it is dependent on many more factors than those for which data can be obtained. Some factors, despite having a clear relationship with the forecast variables, are themselves difficult to forecast and other factors may be interdependent so their precise influence is unclear.

There are two main classes of model: single equation regression models and multi-equation regression models. The former comprises models in which the dependent variable is assumed to be a linear function of several explanatory and independent variables.

The second class of model, the multi-equation regression model, gives us the possibility of accounting simultaneously for the inter-relationship between a set of variables. The weakness of the multi-equation models is that the explanatory variables may be interdependent. The possible feedback between the dependent variable and the explanatory variables is neglected.

2.2.1	Two-variable regression

Assume that we want to study the relationship between two variables, x and y. In order to describe this relationship statistically, we need a set of observations of pairs of values, one from each variable:

(x1,y1),(x2,y2),..........,(xn,yn)

If we plot these quantities, we may find that they show a consistent pattern, and it may therefore be possible to draw a curve that “fits” these points. Two problems then arise:

- decide what type of curve to use;

- choose the “best” curve of that type.

The first problem is usually solved simply by looking at a graphical representation of the observed value pairs (xi,yi) and preferably by choosing on sight a simple type of curve. Straight  lines are very popular in this respect.

The second problem, choosing the optimal values for the parameters of the curve, can only be solved after we have decided on some sort of criterion that informs us about the quality of the curve. The least-squares criterion is mostly used. This is because of its intuitive appeal and its mathematical properties. We will also use it here. According to the least-squares criterion, a curve is good if S, the sum of squares of the “distances” from the given points to the curve, is small. The best curve is that for which S is the smallest.

The second problem can now be rephrased as: determine the parameter values that minimize S.

Suppose that we want to determine a straight line that fits the observations. This means that we assume a linear relationship between x, the independent variable, and y, the dependent variable. The fact that the observations (xi,yi) are not lying on a line is attributed to the influence of an error term.

In short, we assume:

y = a +b x + e					(2.2.1)

The parameters a and b are unknown. The error e is random but has an expected value equal to zero.

Let a and b be estimates of a, resp. b. The calculated value for y for a given value xi  using these estimates:

`	yÙi=a+bxi					(2.2.2)

However, we observed the pair (xi, yi), so yi−yÙi is the deviation of this particular point from the line. Thus, we have to find estimates a and b, that minimize:

S=i=1n(yi−yÙi)2

Figure 2.2/1 : The observations and the line

Note that we are only interested in the vertical deviations. This is not surprising as our ultimate goal is to predict the y- value when a new x- value is given. In general, we have the choice of three possibilities:

V:	Vertical distances

H:	Horizontal distances

P:	Distances perpendicular to the line

The following figure gives the results obtained, starting from the same observations in these three cases.

Figure 2.2/2 : Influence of the “distance” chosen

The estimates a and b follow by solving:

d Sd a=0andd Sd b=0

where

S=i=1n(yi−a−bxi)2

The result is:

b=n1nxiyi−1nxi1nyin1nxi2−1nxi2							(2.2.3)

a = y - b x								(2.2.4)

To judge the quality of the line so produced, we can calculate the coefficient of correlation r.

r2=(yÙi−y)2(yi−y)2=ExplainedvarianceTotalvariance					(2.2.5)

r has the same sign as b and lies in [-1, +1]. r = 0 indicates lack of correlation, in other words: a straight line cannot represent the relationship between y and x.

If r deviates from zero, there are difficulties in judging the goodness of fit between the observations and the straight line. Other methods, therefore, have to be applied.

If we are satisfied with the value of r obtained, we can use

y = a + b x

to provide point-estimates of y, given any value of x.

There are, however, other methods to check that the regression line gives a reliable description of how y depends on x. The first step is to estimate the variance of b and check that it lies within reasonable limits. The second step is to check against systematic errors with the Durbin-Watson test.

The first test requires calculation of the quantity

T=b(x−x)212s							(2.2.6)

where

s2=y2−ay−bxyn−2						(2.2.7)

For accurate checking, it is necessary to use statistical tables for the t-distribution, but T should be less than -2 or greater than + 2 to make the regression analysis acceptable.

For - 2 < T < 2, the value of b cannot be said to be significantly different from zero and we cannot be sure that our straight line with  b | 0  really represents a change of y with x.

If we want to design confidence intervals for the parameters a and b, or for our predicted y-values, we must make the following assumption:

The observations (xi, yi) form a random sample from a bivariate-Normal distributed population such that:

EY|X=x=α+βxVY|=x=s2							(2.2.8)

From now on, and without loss of generality, we shall also assume that x=y=0; this means that we choose a new origin so that before starting the calculations, we have to substract x from all the xi -values, and y from all yi -values.

The estimate for b can now be written as

b=xiyixi2								(2.2.3a)

and

a = 0									(2.2.4a)

The variance s2 is unknown, but we can estimate it by

se2=1n−2i=1n(yi−yÙ)2								(2.2.8)

se is called the standard error of the estimate. se2 is an unbiased estimator of s2.

Moreover,

(n−2)se2s2=c  2(n−2)		(Chi -square distributed with n-2 degrees of freedom)

a»Nα,s2nandb»Nβ,s2xi2,soa−αsnandb−βsxi2

are normal distributed, N(0,1), (with mean = 0 and standard deviation = 1).

It can be shown that a, b and se are independent of each other, so

a−αsns2se2andb−ssxi2s2se2

are t-distributed with n-2 degrees of freedom. Confidence intervals can be calculated and tests of hypotheses performed. To design a confidence interval for the value of y when x = x0, we must look at the variance of yo = bxo . Due to our new origin, which is different for each sample, the variance of yo includes the variance of the origin, V(a), and is equal to:

V(yo)=V(a)+xo2V(b)=s21n+xo2xi2			(2.2.9)

Again, we arrive at a t(n-2)-distribution by eliminating the unknown s2 because

yo−α−βxos1n+xo2xi2»N(0,1)

and so

y−α−βxose1n+xo2xi2»t(n−2)

In the above, we have restricted ourselves to a linear relation between x and y. The same method, however, can be applied in certain cases where a non-linear relationship seems more appropriate.

For this, it is sufficient that the relation can be brought in a linear form by a suitable transformation of the data.

For example, if we assume that the relationship can be expressed by

y=αeβx					(2.2.10)

then	lny=lnα+βx					(2.2.10a)

or	z=g+βx					(2.2.10b)

We determine the estimates c and b for g, resp. b, in the usual way and forecast with

y=ez=ec+bx=ecebx					(2.2.10c)

If transformation of the data to something like a linear relationship is impossible, curvilinear regression can be tried.

The underlying idea is now that the actual relationship between x and y can be represented by

y=βo+β1x+β2x2+,...,+βkxk				(2.2.11)

Our observations (xi,yi) are the result of random fluctuations eI around this line.

Applying the least-squares criterion for estimation of βo,β1,...,βk means again that ei has to be minimized.

Thus, we have tried to find the minimum of

i=1nyi−(bo+b1x1+b2xi2+...+bkxik)2

where bi should estimate βi.

Differentiating partially to bo,b1,...,bk, and equating all the derivatives to zero, gives the linear equation system:

yi=nbo+b1xi+b2xi2+,...,+bkxik

xiyi=boxi+b1xi2+b2xi3+,...,+bkxik+1

xikyi=boxik+b1xik+1+,...,+bkxi2k

The solution of this linear equation system with k+1 unknown gives the required estimates. It is advisable to limit the number of parameters bk  used.

2.2.2	Multiple regression

If we are not satisfied with the value of r, which means that the residual variance (= total variance - explained variance) is too large, we can look at the residues. The residual variance is

yi−yÙi2

and can have a high value only if at least some of the residues yi−yiÙ are large, meaning that vertical distances between observations and the line y = a + bx are wide.

If, however, the studied variable y is likely to depend on other variables than x, one can try to reduce the residuals by introducing one or more “explanatory” factors, x1, x2, etc. There are two main methods to apply, the Iterative Method and the Direct Method, which will be explained by two numerical examples.

Iterative Method

Example: Forecasting the number of local calls.

The method shown below is called the iterative method because it works in steps. This is preferable when explanatory factors to be used are not decideed in advance.

It should be noted that although the method involves a certain degree of approximation, the numerical results will be sufficiently accurate for practical cases like the one given here.

Step 1	Select a couple of possible explanatory factors, x1, ..., xn for which good forecasts are available (number of subscribers, inhabitants, etc.).

Step 2	Calculate the correlation coefficients between the dependent variable y (in the example below the number of local calls) and each of the explanatory factors x1, ..., xn.

Step 3	Select the explanatory factor which has the highest correlation coefficient with y1.

Step 4	If this factor is presumed to be x2, make your first attempt with the regression function

y1=a1+b1x2					(2.2.12)

and calculate the parameters a and b according to the methods in section 2.2.1.

Step 5	Calculate the residuals

r1=y1−(a1+b1x2)					(2.2.13)

Step 6	Calculate the correlation coefficients between the r1 :s and everyone of the remaining explanatory factors, in this case x1,x3,...,xm

Step 7	Select the remaining explanatory factor which has the highest correlation coefficient with r1.

Step 8	If this factor is called x1, make attempt No. 2 by using the regression function

r1=a2+b2x1					(2.2.14)

and calculate the parameters a2 and b2, according to the methods in Section 2.2.1.

Step 9	The function from attempt 1, together with the function from attempt 2, now gives

y1=(a1+b1x2)+r1=(a1+b1x2)+(a2+b2x1)=(a1+a2)+b2x1+b1x2				(2.2.15)

which is an improvement compared with the regression function from the first attempt.

Step 10	Calculate the residuals

r2=y1−[(a1+a2)+b2x1+b1x2]				(2.2.16)

Step 11	Calculate the correlation coefficients between the r2:s and each of the remaining explanatory factors, in this case x3, ..., xn

Step 12	Go on with attempt 3 and continue in the same way as long as the calculated residuals are not small enough.

Perfect fitting will not mean that all residuals are zero. This state will only be reached when the number of parameters is the same as the number of observation in the statistical material.

Numerical Calculations

|  |  | Long distance calls |  |  |
| --- | --- | --- | --- | --- |
| Financial year | Local calls | Manual service | Automatic service | Total |
|  | y1 |  |  | y2 |
| 1958/59 | 2305 | 110 | 125 | 235 |
| 1959/60 | 2423 | 103 | 165 | 268 |
| 1960/61 | 2605 | 95 | 220 | 315 |
| 1961/62 | 2852 | 84 | 267 | 351 |
| 1962/63 | 3040 | 78 | 322 | 400 |
| 1963/64 | 3376 | 73 | 372 | 445 |
| 1964/65 | 3642 | 55 | 445 | 500 |
| 1965/66 | 3700 | 38 | 530 | 568 |
| 1966/67 | 3884 | 21 | 609 | 630 |
| 1967/68 | 4059 | 13 | 650 | 663 |

Table 2.2/1 Telephone Calls (million)

Step 1:	Select possible explanatory factors

|  |  | Telephone Subscribers |  |  | Telephone Apparatus |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Year 1:st Jan. | Population | man. exch. | aut. exch. | Total | man. ex- exch. | aut. ex- exch. | Total |
|  | x1 |  |  | x2 |  |  | x3 |
| 1959 | 7 436 | 352 | 1 712 | 2 064 | 408 | 2 118 | 2 526 |
| 1960 | 7 471 | 311 | 1 839 | 2 150 | 359 | 2 278 | 2 637 |
| 1961 | 7 499 | 271 | 1 973 | 2 244 | 313 | 2 448 | 2 761 |
| 1962 | 7 542 | 207 | 2 143 | 2 350 | 236 | 2 668 | 2 904 |
| 1963 | 7 581 | 174 | 2 294 | 2 468 | 198 | 2 856 | 3 054 |
| 1964 | 7 627 | 144 | 2 454 | 2 598 | 163 | 3 060 | 3 223 |
| 1965 | 7 695 | 101 | 2 622 | 2 723 | 114 | 3 273 | 3 387 |
| 1966 | 7 773 | 75 | 2 785 | 2 860 | 86 | 3 487 | 3 573 |
| 1967 | 7 844 | 41 | 2 956 | 2 997 | 46 | 3 711 | 3 757 |
| 1968 | 7 894 | 27 | 3 095 | 3 122 | 32 | 3 903 | 3 935 |

Table 2.2/2	Explanatory Factors (thousands)

| 1:st | Population | Subscribers | Apparatus |
| --- | --- | --- | --- |
| Jan. |  | Total | Total |
|  | x1 | x2 | x3 |
| 1969 | 7 964 | 3 214 | 4 111 |
| 1970 | 8 042 | 3 335 | 4 288 |
| 1971 | 8 120 | 3 453 | 4 458 |
| 1972 | 8 201 | 3 566 | 4 626 |
| 1973 | 8 281 | 3 672 | 4 793 |
| 1974 | 8 360 | 3 776 | 4 957 |
| 1975 | 8 430 | 3 879 | 5 120 |

Table 2.2/3	Available Forecasts for Explanatory Factors (thousands)

Step 2:	Calculate the correlation coefficients. Note that the basic figures given in Tables 2.2/1-2 are rounded off in the following calculations.

Calculation of the correlation coefficients between the number of local calls y and the explanatory factors x1, x2 and x3 gave the results shown. Data is taken from Tables 2.2/1-2.

| y1 | x1 | x2 | x3 |
| --- | --- | --- | --- |
| 109 | 106 | 106 | 106 |
| 2.31 | 7.44 | 2.06 | 2.53 |
| 2.42 | 7.47 | 2.15 | 2.64 |
| 2.60 | 7.50 | 2.24 | 2.76 |
| 2.85 | 7.54 | 2.35 | 2.90 |
| 3.04 | 7.58 | 2.47 | 3.05 |
| 3.38 | 7.63 | 2.60 | 3.22 |
| 3.64 | 7.70 | 2.72 | 3.39 |
| 3.70 | 7.77 | 2.86 | 3.57 |
| 3.88 | 7.84 | 3.00 | 3.76 |
| 4.06 | 7.89 | 3.12 | 3.94 |
| 31.88 | 76.36 | 25.57 | 31.76 |

Table 2.2/4	Given Explanatory Factors

The following sums are calculated:

yi=31.88yi2=105.22      x1=76.36x12=583.31x1y1=244.32      x2=25.57x22=66.58x2y1=83.57     x3=31.76x32=102.97x3y1=103.96

The correlation coefficient is calculated from:

R(y1xi)=DiGiHi

where	Di=nxiy1−xiy1Gi=ny12−y12Hi=nxi2−xi2

which gives

R(x1y1)=0.979R(x2y1)=0.991R(x3y1)=0.989

Step 3:	Select the explanatory factor to be used first.

In this first attempt, we choose the explanatory factor with the highest correlation coefficient with y1. Here, it was the number of subscribers = x2.

Step 4:	Make attempt No. 1 with the regression function

y1=a1+b1x2

Formulae for calculation of a and b in Section 2.2 can be written as follows:

a1n+b1x2=y1a1x2+b1x2i2=x2y1

or with the numerical values:

10.00 a1 + 25.57 b1 = 31.88

25.57 a1 + 66.58 b1 = 83.57

The solution is:

a1 = - 1.196

b1 =    1.715

Step 5:	Calculate the residual r1.

The function arrived at in Step 4 (attempt 1) gives the following fitted values of y1:

Table 2.2/5	Fitted values of y from Step 4

| Financial |  | Fitted values | Comparison with the given values of y |  |
| --- | --- | --- | --- | --- |
| year |  | - 1.13 + 1.677 x = | Table 2.2/1 | residual |
|  | x2 | y1 | y | r1 |
| 1958/59 | 2.06 | 2.34 | 2.31 | - 0.03 |
| 1959/60 | 2.15 | 2.49 | 2.42 | - 0.07 |
| 1960/61 | 2.24 | 2.64 | 2.60 | - 0.04 |
| 1961/62 | 2.35 | 2.83 | 2.85 | + 0.02 |
| 1962/63 | 2.47 | 3.04 | 3.04 | + 0.00 |
| 1963/64 | 2.60 | 3.26 | 3.38 | + 0.12 |
| 1964/65 | 2.72 | 3.47 | 3.64 | + 0.17 |
| 1965/66 | 2.86 | 3.71 | 3.70 | + 0.01 |
| 1966/67 | 3.00 | 3.95 | 3.88 | - 0.07 |
| 1967/68 | 3.12 | 4.15 | 4.06 | - 0.09 |

Step 6:	Calculate the correlation coefficients with r1.

Before our second attempt, we calculate the correlation coefficients between the residuals from attempt 1 and the remaining explanatory factors x1  and x3.

Table 2.2/6	Calculation of correlation coefficients.

| Residuals from | x1 | x3 | Calculated sums |  |
| --- | --- | --- | --- | --- |
| attempt No. 1 | from |  |  |  |
| r1 | Table 2.2/2 |  | Variable | Value |
| - 0.03 | 7.44 | 2.53 | r12 | = 0.0642 |
| - 0.07 | 7.47 | 2.64 |  |  |
| - 0.04 | 7.50 | 2.76 | x12 | = 583.31 |
| - 0.02 | 7.54 | 2.90 |  |  |
| - 0.00 | 7.58 | 3.05 | x32 | = 102.97 |
| + 0.12 | 7.63 | 3.22 |  |  |
| + 0.17 | 7.70 | 3.39 | r1x1 | = - 0.0073 |
| - 0.01 | 7.77 | 3.57 |  |  |
| - 0.07 | 7.84 | 3.76 | r1x3 | = - 0.1199 |
| - 0.09 | 7.89 | 3.94 |  |  |
| Total | 76.36 | 31.76 |  |  |

The correlation coefficients were:

R (r1, x1) = - 0.029

R (r1, x3) = - 0.019

This indicates a very low correlation.

Step 7:	Select the next explanatory factor.

The highest correlation coefficient is the one between r1 and x1, so our second attempt will be to use the explanatory factor x1.

Step 8:	Make attempt 2 with the function

r1 = a2 + b2 x1

Parameters a1  and b2  are given from the following equations:

a2n+b2x1=r1a2x1+b2x12=r1x1

or	10.00 a2 + 76.36 b2  = - 0.04

76.36 a2 + 583.31 b2 = - 0.3089

The solution is then:

a2 = 0.18	0.113

b2 = - 0.023	- 0.0154	(cor - 0.028822)

Consequently	r1 = 0.113 - 0.0154 x1

Step 9:	The results from attempts 1 and 2 above-mentioned will now give the following regression function for local calls:

y1 = (-1.196 + 1.715 x2) 	+	r1	=

(from attempt 1)		(from attempt 2)

¯	¯

= (-1.196 + 1.715 x2) 	+	(0.113 - 0.0154 x1)

consequently

y1 = -1.083 - 0.0154 x1 + 1.715 x2

This result is compared with the given y1 -values in Table 2.2/1.

Step 10:	Calculate the residuals r2

| Financial | Fitted values for local calls | Given values of y |  |
| --- | --- | --- | --- |
| year | - 1.083 - 0.0154 x1 |  |  |
|  | + 1.715 x2 | Table 2.2/1 | Residual |
|  | = y1 | x2 | r2 |
| 1958/59 | 2.34 | 2.31 | - 0.03 |
| 1959/60 | 2.49 | 2.42 | - 0.07 |
| 1960/61 | 2.64 | 2.60 | - 0.04 |
| 1961/62 | 2.83 | 2.85 | + 0.02 |
| 1962/63 | 3.04 | 3.04 | 0.00 |
| 1963/64 | 3.26 | 3.38 | + 0.12 |
| 1964/65 | 3.46 | 3.64 | + 0.18 |
| 1965/66 | 3.70 | 3.70 | 0.00 |
| 1966/67 | 3.94 | 3.88 | - 0.06 |
| 1967/68 | 4.15 | 4.06 | - 0.09 |

From this regression function, y (x1, x2), we find the following future number of local calls (in million):

|  | x1 | x2 | y (x1x2) | y(x2) |
| --- | --- | --- | --- | --- |
| 1968/69 | 7.96 | 3.21 | 4300 | 4310 |
| 1969/70 | 8.04 | 3.34 | 4520 | 4530 |
| 1970/71 | 8.12 | 3.45 | 4710 | 4720 |
| 1971/72 | 8.20 | 3.57 | 4910 | 4930 |
| 1972/73 | 8.28 | 3.67 | 5080 | 5100 |
| 1973/74 | 8.36 | 3.78 | 5270 | 5290 |
| 1974/75 | 8.43 | 3.88 | 5440 | 5460 |

The result is given in Figure 2.2/3

Figure 2.2/3

From the residuals r2  follows that attempt 2 gave practically no difference as compared with attempt 1, y(x2) above. This followed already from the rather small correlation coefficient R(r1, x1).

We should, therefore, stop at step 8 and use the regression function.

y1 = -1.083 - 0.0154 x1 + 1.715 x2

Direct Method

For this method, we assume from the start that the variable we want to forecast depends on a number of explanatory variables.

Example:	Forecasting the number of long-distance calls.

The number of long-distance calls, y2, generally increases heavily by the introduction of automatic service (STD). It was frequently observed that introduction of STD between two areas caused a doubling of the number of calls.

Therefore, it seems appropriate to use the explanatory factor “degree of automixation” (denoted by x4), to forecast the number of long-distance calls. This degree is defined as the ratio between the number of long-distance calls and all long-distance calls.

It also seems likely that the number of telephone apparatus (denoted x3) could be a useable explanatory factor. After these considerations, we select a priori the two explanatory factors:

-  number of telephone apparatus	= x3

-  degree of automization		= x4

It is presumed that we need no further explanatory factors, so we can apply the “direction method” for calculation of the parameters in the regression function:

y2=a+b3x3+b4x4					(2.2.17)

| Financial year | Long-distance calls in thousand million | Telephone apparatus in million | Degree of automazation |
| --- | --- | --- | --- |
|  | y2 | x3 | x4 |
| 1958/59 | 0.235 | 2.53 | 0.53 |
| 1959/60 | 0.268 | 2.64 | 0.62 |
| 1960/61 | 0.315 | 2.76 | 0.70 |
| 1961/62 | 0.351 | 2.90 | 0.76 |
| 1962/63 | 0.400 | 3.05 | 0.81 |
| 1963/64 | 0.445 | 3.22 | 0.84 |
| 1964/65 | 0.500 | 3.39 | 0.89 |
| 1965/66 | 0.568 | 3.57 | 0.93 |
| 1966/67 | 0.630 | 3.76 | 0.97 |
| 1967/68 | 0.663 | 3.94 | 0.98 |
| Total | 4.375 | 31.76 | 8.03 |

Table 2.2./8	Long-distance calls and explanatory factors

| Financial year |  | Telephone apparatus in million | Degree of automazation |
| --- | --- | --- | --- |
|  |  | x3 | x4 |
| 1968/69 |  | 4.11 | 0.99 |
| 1969/70 |  | 4.29 | 0.99 |
| 1970/71 |  | 4.46 | 1.00 |
| 1971/72 |  | 4.63 | 1.00 |
| 1972/73 |  | 4.79 | 1.00 |
| 1973/74 |  | 4.96 | 1.00 |
| 1974/75 |  | 5.12 | 1.00 |

Table 2.2/9	Given forecasts for the explanatory factors:

From the figures for the period 1958-1968, we calculate the following totals:

x32=102.97x42=6.65		x3x4=26.13y2x3=14.55y2x4=3.71

The parameters a, b3 and b4 are estimated from the equation system:

an+b3x3+b4x4=y2ax3+b3x32+b4x3x4=y2x3ax4+b3x3x4+b4x42=y2x4

or	10.00 a +   31.76b3 +   8.03b4 =   4.38

31.76 a + 102.97b3 + 26.13b4 = 14.55

8.03 a +   26.13b3 +   6.65b4 =   3.71

The solution of the equation system is:

a   = -0.481

b3 =   0.222

b4 =   0.267

Consequently, the regression function will be:

y2 = -0.481 + 0.222 x3 + 0.267 x4

The result is compared with the given y2 -values in the following table:

| Financial | y2 = | Comparison with given values of y2 |  |
| --- | --- | --- | --- |
| year | -0.481 + 0.222 x3 + 0.267 x4 | from Table 2.2/8 | Residual r |
| 1958/59 | 0.223 | 0.235 | +0.012 |
| 1959/60 | 0.271 | 0.268 | -0.003 |
| 1960/61 | 0.319 | 0.315 | -0.004 |
| 1961/62 | 0.366 | 0.351 | -0.015 |
| 1962/63 | 0.412 | 0.400 | -0.012 |
| 1963/64 | 0.458 | 0.445 | -0.013 |
| 1964/65 | 0.510 | 0.500 | -0.010 |
| 1965/66 | 0.560 | 0.568 | +0.008 |
| 1966/67 | 0.613 | 0.630 | +0.017 |
| 1967/68 | 0.656 | 0.663 | +0.007 |

Table 2.2/10	Fitted values of long-distance calls (y2)

Extrapolating these figures into the future, we find the following forecast for the number of long-distance calls in thousands of millions:

1968/69		0.695

1969/70		0.735

1970/71		0.776

1971/72		0.814

1972/73		0.887

1974/75		0.923

The result is given in Figure 2.2/4.

Figure 2.2/4

2.3	Trend methods

A time series is a set of observations of a quantity as a function of time. The observations may follow a certain pattern and the forecaster wants to find out how the observed quantity will develop in the future, i.e. to estimate the trend.

This section will deal with possible curves for development, such as linear trend, exponential trend and trends towards saturation level. The section on time series deals with methods to explain an observed variation and growth pattern as a result of four factors: trend, cyclical variations, seasonal variations and random variations.

If correct historical data are available, the trend methods can be used to forecast future devleopment on one important assumption: the same dependence between the variables will exist in the future. This is the same assumption as is made for all forecasts based on extrapolation of curves describing the past.

2.3.1	Time series analysis`

Time series analysis is the name for a number of methods required to find an explanation for the fluctuations in the studied values (yi).

Usually, the values of y are thought to be the result of four factors:

T.	The trend T is the fundamental growth process (this growth can of course be negative or zero).

C.	The cyclical variations C are sinus-like quantities with a very low frequency. The business cycles, seven fat years followed by seven lean years, are an example of this.

S.	The seasonal variations S also oscillate, but with a higher frequency, and are generally more pronounced than the cyclical variations. Daily, weekly or yearly patterns are usually incorporated in S.

I.	The irregular or random movements (I) are due to chance events such as strikes, earthquakes, changes in tariff, etc., and may be regarded as residual, unexplained by T, C and S.

In Figure 2.3/1, the effects of T, C and S are shown.

Figure 2.3/1

Influence of cyclical and seasonal variations on the trend, in an idealized time series

The first three faccctors are studied separately. Fact I cannot be expressed as a function of time and acts as a residual variation.

We can express the combined effect of the four factors as

Y = T + S + C+ I							(2.3.1)

or by multiplication, expressed by the formula

Y = T´S´C´I							(2.3.2)

Combined addition and multiplication is also possible, e.g.

Y = T + S´C´I							(2.3.3)

It is usually sufficient for long-term forecasting to aggregate data into yearly figures and to attempt to fit a trend line through them.

A variety of mathematical curves can be fitted for analysing time series data ranging from freehand extrapolation of a graph of the series, fitting a simple straight line, fitting a straight line to transformed data series, to the use of a polynomial curves and on to still more complicated methods.

Linear trend

Any straight line giving the relationship between two variables can be expressed as:

y = a + bt								(2.3.4)

When a and b have their values specified, the line is uniquely defined and the value of y can be calculated for any value of t. A number of pairs of recorded values of y and t are available for the forecaster. These values do not in general lie exactly along a straight line. His task is to estimate a and b. Assuming that n records are available, the values of a and b calculated under the least squares criterion are obtained by solving the following pair of simultaneous equations:

y=na+bt							(2.3.5)

ty=at+bt2							(2.3.6)

where n = number of pairs of values (t, y);

Method

Calculate the following values:

ty=t1y1+t2y2+,...,+tnynt2=t12+t22+,...,+tn2t=t1+t2+,...,+tny=y1+y2+,...,yn

n = number of pairs = number of t-values = number of y-values

Here:	t=tn		y=yn

Finally, calculate a and b for the equation system (2.3.5) and (2.3.6):

b=ty−ntyt2−nt2					(2.3.7)

and

a=y−bt					(2.3.8)

For each value t, the y-value is given from:

y = a + b . t

which is the equation for the straight trend line we were seeking.

Exponential trend

Exponential growth can be expressed with the equation

y=a⋅eb⋅t					(2.3.9)

where y and t are the variables, and a and b are the parameters of the curve, and e = base for natural logarithms.

Taking the logarithm of the whole equation:

lny=lna+b⋅t					(2.3.10)

Introduce

z=lny	and	c=lna

The equation can then be written

z = c + b . t					(2.3.11)

We see that this equation is a straight line.

Method

Take the logarithm of all y-values and call these values z.

Then calculate:

tz	t2	t	z

t=tn	z=zn	nt2

as before !

Then, we have:

b=tz−ntzt2−nt2					(2.3.12)

and

c=z−btz=c+bt					(2.3.13)

For each t, the value y-value is now calculated as

y = anti-logarithm of z					(2.3.14)

Checking the regression model

A.	Testing the significance of the time parameters

This regression approach depends on the assumption that the quantity under consideration (either a measured traffic or a calling rate) is correlated with time. This may or may not be true, the values may be quite independent of time. The formulae given above will, however, always produce a numerical value for b. It is, therefore, necessary to test if this value is arrived at by pure chance and if its true mean should be zero. This can be performed by calculating the value

H=b(t−t)2s	where	s2=y2−ay−btyn−2	(2.3.15)

Calculate s2 first, then calculate s = square root of s2.

The notations above are valid for the straight line. For the exponential case, calculate s2 as

s2=z2−cz−btz(n−2)				(2.3.16)

For accurate checking, it is necessary to use statistical tables, but as an approximate guide, H should be greater than 2 or less than - 2 for the linear description to be valid. If values in the range - 2 to + 2 are obtained, then the value of b is not significantly different from zero; meaning that the traffic quantity does not change systematically with time and the analytical description is inappropriate. The best forecast for future values of y is then no change at all.

B.	Examining systematic errors

Even if the parameter b passes the above test, the model may be invalid due to the existence of systematic errors in the way the “best“ line is fitted to the data points. The following procedure should be used to check for systematic form of errors:

Denote the values on the fitted line corresponding to the recorded values of y by yÙ, and further denote the first readings by y1,yÙ1, the second by y2,yÙ2, and so on.

Calculate

w=i=1n−1(yi−yÙi)(yi+1−yÙi+1)				(2.3.17)

and	v=i=1n(yi−yÙi)2					(2.3.18)

Calculate the value 2−2wν

The statistic, known as the Durbin-Watson statistic, should lie between 1.5 and 2.5 and preferably between 1.7 and 2.3. If the value is outside this range, the data should be examined for indications of systematic deviation from the fitted curve. Common causes are either wrong curve shape (see Figure 2.3/2, Graph 3) or discontinuity in the data (see the same figure, Graph 4). If the Durbin-Watson statistic gives an unsatisfactory test result, suitable adjustments must be made to the data and the analysis be repeated. The forecasts may otherwise be subject to serious error.

Figure 2.3/2

Forecasting future levels

Assuming that the analytical description of the past successfully passes the tests described above, the next stage is to provide the required forecasts.

All that is required is to calculate the value of y from

y = a + bt

(If an exponential growth curve is fitted to the historical data, the actual forecast level is obtained by taking the natural anti-logarithm of the value of y.)

A general idea of the precision of the forecast can be obtained by designing a confidence interval for the forecast value, within which it can be stated with a pre-determined degree of probability that the future value will be.

For instance, if the forecast for a future time t0 is yo, then there is approximately a 95 % probability that the achieved value of y will be within the range.

yo±2u

where	u=s21+1n+(to−t)2(t−t)2				(2.3.19)

and s2 is calculated from (2.3.15).

It will be noticed (see Figure 2.3/3) that the further the forecast is made into the future, the wider the confidence interval, and accordingly the more uncertain the forecast.

Figure 2.3/3 : Confidence interval (95 %) for forecasted linear development

If the quantities plotted on a graph show a consistent pattern over time, it is possible to draw a curve of best fit. Future values are then forecast by extrapolation of this curve.

The first problem is to find the equation for the curve which best describes the relationship between quantity and time.

For many types of technical equipment, the history of its usage can be described in three phases. In the first phase, we observe an accelerated usage which in the second phase changes to a linear increase over time. In the third phase, the usage slows down and eventually reaches saturation level. Examples of such phenomena may be the number of households with radios, television sets and telephones.

For other cases, the saturation level may become zero. Example: Use of horse-drawn cabs in London, use of the telegram service, use of conventional sailing ships for transporting goods, use of stone axes in battle, etc.

We will not deal with this latter type of development curve.

For a quantity that in the future may reach a saturation level, it may be possible to describe the whole history with one single mathematical expression. It may however be simpler and more accurate to describe each phase separately.

Figure 2.3/4

Development over time for a telecommunication service

Curves describing the development trend over time are often called growth curves, even if the “growth” is sometimes really a decrease in quantity. Here are some common types of trend curves:

Linear:	y = a + b t			(2.3.20)

Parabolic:	y = a + b t + c t2		(2.3.21)

Exponential:	y = a ebt			(2.3.22)

Gompertz:	y = ea-br(t)			(2.3.23)

Notations

t =	point of time (independent variable)

a, b, c, r =	parameters to be calculated from historical data

y =	item to be forecast (dependent variable)

e =	the basis for natural logarithm

Some simple numerical examples are on trends:

1)	Linear trend	y = a + b t

The formula has the unknown parameters a and b which should be calculated from given historical data. To calculate two parameters, we need two equations.

These two equations might be obtained from two points in a diagram through which the straight line should pass.

Figure 2.3/5

Linear trend, estimated from 2 points

Assume the following points:	t =      0	t =    15

y = 100	y = 500

Insertion of the 2 points in the equation for the straight line (2.3.20) gives:

100 = a + b    0

500 = a + b 15

which gives	a = 100	and	b=500−10015=26.7

This gives the trend:

y = 100 + 26.7 t

2)	Exponential trend	y = a ebt

Assume the same points as in the previous case:

t =      0	t =    15

y = 100	y = 500

The two required equations will be

100 = a eb0

500 = a eb15	So: a = 100 and b is calculated from:

500 = 100 eb15

5 = eb15

ln5 = 1.609 = 15b	which gives		b = 0.1073

The trend is:	y = 100 e0.1073t

The curve is plotted in the diagram below:

Figure 2.3/6

Exponential trend determined from 2 points

3)	Gompertz’ trend	y=ea−brt

In this case, we have three parameters a, b and r, and the calculation requires three equations for determination of the parameters. We use the same two points as before:

t =      0	t =    15

y = 100	y = 500

A third equation can be obtained by assuming a saturation value in infinity, i.e. the point:

t = ¥	y = 3000 	(i.e. the value 3000 will be reached after an infinite number of years)

The parameters are calculated from:

100=ea−br0		500=ea−br15

The third equation gives since r < 1 and t = ¥

3000 = ea	therefore	a = 1n3000 = 8.006

If a = 8.006 is inserted in the first equation,

4.605 = 8.006 - b;	which gives	b = 3.401

Then r is calculated from the second equation for

a = 8.006	and	b = 3.401

6.215 = 8.006 - 3.401 r15

or	r=8.006−6.2153.401115	r = 0.958

Figure 2.3/7 : Numerical examples on trends

1)	Linear trend:	y=100+26.7t

2)	Exponential trend:	y=100e0.1073t

3)	Gompertz’ trend:	y=e8.006−3.401(0.958)t

(saturation value = 3000)

Other growth curves:

Consider

y=M−aebt				(2.3.24)

There are four configurations of this curve according to the signs of a and b. These are illustrated below.

Figure 2.3/8 : Modified exponential curves

When forecasting demand for telecommunications, only (ii) and (iii) are applicable for long term forecasting, of which (ii) is less frequently used. A decline towards a minimum as shown in (ii) might be used for such variables as the number of manual calls per subscriber or the number of telegrams per capita. However, few telecommunication services are characterized by steady decline. Thus (iii) is most applicable. Of course, the implied approach to a saturation level as shown in (iii) may cover a period stretching over several decades. It may possibly be applied to forecast telephone density in countries with very high penetration, but is is inapplicable where telephone development is at an earlier stage. An alternative to the Gompertz’ curve is the logistic curve, as described below.

Logistic curve

The simple logistic curve is given by the formula:

y=F+M−F1+ae−bt				(2.3.25)

The curve is S-shaped with a floor value F, and a maximum M. The curve increases to its maximum over time for a > o, b > o.

Figure 2.3/9 : Logistic curve

The curve has the following properties:

i.	The growth increases steadily to a peak and then decreases.

ii.	The point of inflexion at which the change in the absolute growth rate changes from an increase to a decrease occurs at a level midway between floor and ceiling, at time t = tc , where tc is given by 1/b 1n (1/a).

Comments

The purpose of a trend analysis may not always be to find out how the average for all traffic values changes with time. For dimensioning purposes, we are particularily interested in how the upper tail of the annual traffic distribution changes with time in order to obtain a representative value for the busy hour traffic. This busy hour traffic then determines the number of circuits and switches required in the future. To carry out a meaningful time series extrapolation, it is therefore essential that the data used really refers to values that are representative for our purpose. A set of historical data based on paid minutes per month, for example, would certainly not be worth the effort of any serious time series analysis. It is so because the relationship between the busy hour traffic and the monthly paid minutes is not generally established by experience but has to be calculated as a product of several inaccurate factors.

Another case where a time series analysis will not work is when the operational conditions have changed. For example, if the service is suddenly remarkably improved, the traffic ususally changes drastically. Such service improvement jumps may mean an increase in traffic of 100 % or more. To estimate the magnitude of such a sudden increase in traffic, information is required of the service performance before and after the change, how much traffic was suppressed before the change, etc. Such data are not generally available or are rather hard to collect.

2.4	Individual judgment

The mathematically simplest forecasting method is individual judgment. The forecast is based on experience and collected information. No systematic analysis is made.

However, the quality of a forecast is directly dependent on the quality of the basic data. Unless reliable basic data are available, the forecaster must establish an adequate data collection process before starting his forecasting work, or rather, to secure more reliable data for his future work.

A typical example of forecasts based on individual judgment is estimation of the future number of subscribers in a given area. These forecasts are generally related to existing and future buildings. Each type of buildings has its typical telephone density. The short-term forecast may concern existing buildings, buildings under construction and planned ones. The medium- and long-term forecasts may concern different types of areas as well, where areas with office buildings will require more telephones than residential areas. The forecaster may, as an aid, have tables showing the average number of subscribers in different types of buildings and areas. When carrying out this type of investigation, correct information from city planning authorities is essential, as well as past experience.

An example of the average number of subscribers for different types of buildings is shown in Table 2.4/1. The table should naturally be adjusted to agree with the situation in the country or towns concerned. For medium- and long-term forecasting, Table 2.4/2 shows typical values for different types of built-up area. The figures should also be adjusted here to agree with the country concerned. The figures given in the tables need to be updated when the telephone density increases.

| Type of buildings | Number of subscribers |
| --- | --- |
| Official buildings and offices, banks, insurance companies, large hotels, clubs, large restaurants, hospitals, large commercial houses | investigate |
| Small hotels, restaurants, food shops, boarding houses | 1 -2 |
| Chemists, doctors, lawyers, etc. | 1 - 1.5 |
| Shops | 0.5 - 1 |
| Large factories | investigate |
| Small factories, workshops | 0.5 - 1.5 |
| Cinemas, petrol stations | 1 - 2 |
| Private houses of highest class | 1 |
| Private houses of lower class | 0.3 - 0.5 |
| Terraced houses | 0.3 / apartment |
| Blocks of flats, highest class | 0.5 - 1 / apartment |
| Blocks of flats, lower class | 0.2 / apartment |

Table 2.4/1	Assessment of number of subscribers per different kind of buildings.This example is valid for a developing country with 4-8 subscribers per 100 inhabitants.

| Type of built-up area |  | Subscribers per hectare |  |
| --- | --- | --- | --- |
| A. | Slum |  | 0.25 |
| B. | Parks, gardens, etc. |  | 0.50 |
| C. | Old private houses with large gardens |  | 1.00 |
| D. | Poor workers’ residential districts |  | 1.50 |
| E. | Better class workers’ residential districts |  | 2.00 |
| F. | Modern private houses with large gardens |  | 3.00 |
| G. | Modern workers’ residential districts |  | 4.00 |
| H. | Industrial areas |  | 5.00 |
| I. | Modern private houses with small gardens |  | 7.00 |
| J. | Non-detached houses of older type |  | 8.00 |
| K. | Area consisting of working class dwellings and small workshops |  | 10.0 |
| L. | Modern non-detached houses |  | 13.0 |
| M. | Non-detached 1-2 storey residential buildings and small shops |  | 18.0 |
| N. | Blocks of flats up to 4 storeys |  | 25.0 |
| O. | Blocks of flats and shops of up to 4 storeys |  | 28.0 |
| P. | Business centre in residential area |  | 30.0 |
| Q. | Blocks of flats of more than 4 storeys |  | 40.0 |
| R. | Office buildings up to 3 storeys |  | 80.0 |
| S. | Office buidings of 4-6 storeys |  | 150 |
| T. | Office buildings of more than 6 storeys |  | 250 |

Table 2.4/2	Assessment of number of subscribers per hectare in built-up areas of different kinds.

This example is valid for a developing country with 4-8 subscribers per 100 subscribers.

2.5	Other methods

The following is a survey of some other methods used for forecasting.

Analytical comparisons

This method relies on data from other, preferably comparable areas where the development is more advanced. The forecast is then made by comparison and assuming that the development will be the same.

This method is used when the series of adequate historical data is too short to permit reliable trend analysis. This is frequently the case in developing countries. It is then assumed that the development will follow the same pattern as in the more developed country or area.

Table 2.5/1 shows growth in telephone penetration over the period 1960-1975, expressed in the number of telephones per 100 inhabitants for certain countries. It also shows during which period the United Kingdom developed at the same rate and how many years it took there to go from the penetration of the country concerned in 1960 to that in 1975.

| Country | Telephones per 100 inhabitants |  | Corresponding UK penetration |  |
| --- | --- | --- | --- | --- |
|  | 1960 | 1975 | Dates | Period |
| Argentina | 5.99 | 9.41 | 12 years | 1937-1949 |
| Chile | 2.43 | 4.26 | 8 years | 1923-1931 |
| Costa Rica | 1.34 | 5.02 | 25 years | 1910-1935 |
| Cyprus | 2.95 | 10.65 | 25 years | 1925-1950 |
| Fiji | 1.71 | 4.53 | 18 years | 1914-1932 |
| Greece | 2.30 | 20.71 | 45 years | 1922-1967 |
| Ireland | 5.13 | 12.78 | 20 years | 1935-1955 |
| Japan | 5.21 | 37.88 | 41 years | 1935-1976 |
| Mexico | 1.46 | 4.37 | 20 years | 1911-1931 |
| Spain | 5.47 | 12.46 | 18 years | 1936-1954 |

Table 2.5/1	Number of telephones per 100 inhabitants for certain countries.

Table 2.5/1 shows that there is no clear connection between the time it took for a particular country to develop from the 1960 value to the value of 1975, and the time that was needed in the United Kingdom to reach the same degree of development. It is therefore not recommended to compare countries with a too large difference in development. Table 2.5/2 gives the same information as the previous table but for a set of countries with approximately the same economic development.

| Country | Telephones per 100 inhabitants |  | Corresponding dates for UK |  |
| --- | --- | --- | --- | --- |
|  | 1960 | 1975 |  |  |
| Australia | 20.88 | 37.49 | 9 years | 1969-1976 |
| Denmark | 22.17 | 42.48 | 10 years | 1968-1978 |
| Finland | 12.89 | 35.78 | 20 years | 1955-1975 |
| Germany (F.R.) | 9.98 | 30.25 | 24 years | 1949-1973 |
| Iceland | 21.42 | 40.41 | 10 years | 1967-1977 |
| Netherlands | 13.15 | 34.41 | 19 years | 1955-1974 |
| Norway | 19.50 | 33.90 | 8 years | 1966-1974 |

Table 2.5/2	Telephone penetration 1960-1975 for some countries with approximately the same economic development.

This table shows a better agreement with the development in the United Kingdom than the previous one.

We can now conclude the following:

- Short-term projection of a country’s telephone density from a given level, by comparison with the growth of some more advanced country being at the same level earlier, cannot always be recommended since special circumstances may cause development not to follow the same trend. Such circumstances are, e.g., economic recession or expansion, supply limitations, different tariff policies, etc.

- For long-term projections, it is necessary to choose a country which is at least 15 years more advanced in order to have a series long enough for the possible future growth.

- If such a comparison is carried through, however, we assume that a comparison between one country’s performance over the last 15 years and another country’s over the previous 15 years is valid. This is unlikely because of the rapid changes in the world economy and because of disruptive events such as wars and major recessions which are likely to distort the comparison. Data confirms this.

Further Methods:

The eye-ball method

The method is simply that the forecaster inspects the plotted data in order to get an idea of some possible trend or relationship between the variables and then manually tries to draw a corresponding curve.

Moving average

The new value is calculated as the mean of a number of observed values. The purpose of this approach is to reduce irregularities caused by, for instance, seasonal variations.

Exponential smoothing

Similar to the moving average method but weight is given to the observations in such a manner that more recent observations get more weight.

Market research

This method involves a study of the market by asking questions to potential customers.

Combined methods (including structural analysis)

A combination of different methods such as market research, growth curves, projection and econometric models.

3.	FORECASTING FOR EXCHANGE-PLANNING

For exchange-planning, the demand should be forecast for all particular items for each exchange separately. This includes the following, per category of subscriber:

- the number of direct subscriber lines (DEL)

- originating traffic

- terminating traffic

- local traffic

- long-distance traffic

- international traffic

- special services

Once these forecasts have been made, corresponding figures from different exchanges can be added to produce results for greater areas or for the country as a whole. This is known as forecasting “from the botton up”.

When a forecast for a an area shall be made, all information available about future changes in this area must be collected. The relevant information obtained in this way can make it possible to foresee discontinuities in the development of the demand. On the other hand, fluctuations of the quantities of interest are relatively great when we are looking at a small area. The number of reliable historic records is also usually very limited.

These two factors combine to make the small area forecast less reliable. Adding the results to obtain the corresponding figures for a wider area can lead to grave errors. To guard against this, the “from the bottom up” forecast is compared with a “from the top down” forecast.

Starting from the forecast for the country as a whole or for a large area, one can of course arrive at forecasts for smaller regions by splitting up the amounts forecasted. Usually, more extensive and reliable data are available for the forecasts of large areas. Therefore, the “from the top down” forecast is generally regarded as more reliable than the “from the bottom up” forecast.

There are two ways in which agreement between the two forecasts can be reached:

1.	By changing the division of the total amount over the various areas.

2.	By reducing the local forecasts by a given factor, either uniformly or after some system.

Clearly, this procedure of matching the two forecasts is rather subjective. It is also clear that an area which has submitted a high forecast will have to reduce its figures unless a satisfactory reason for this high forecast can be given. Such a reason may be a known development plan for this area, which motivates a sudden increase in the demand. The large-area forecast does not always take into account such local peculiarities.

4.	FORECASTING FOR NETWORK PLANNING

4.1	Introduction

The planning of a telephone network is based on estimates of future traffic need.A long-term forecast is necessary for the development plan to ensure coordinated extension over a period of 15-25 years. This development plan has to be brought up to date approximately every 2-4 years.

Within the development plan, short-term forecasts are needed to provide the basic data for planning the actual extension steps. They should contain estimates of the traffic needs for four to six years ahead to cater for delivery times, etc. The short-term forecast should be brought up to date every year.

The traffic needs are expressed in erlang. To forecast these needs, the traffic is estimated between each pair of exchanges, generally separately for each direction. The traffic within each exchange area is also estimated.

4.2	Traffic matrix

To specify the traffic needs in a region with n exchanges, n2  traffic values are required. A standard way to specify these traffics is to present them in a matrix, the so-called traffic matrix.

|  | to |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| from |  | 1 | i | j | n | SO |
| 1 |  | A(11) |  |  | A(1n) | O(1) |
| i |  |  | A(ii) | A(ij) |  | O(i) |
| j |  |  | A(ji) | A(jj) |  | O(j) |
| n |  | A(n1) |  |  | A(nn) | O(n) |
| ST |  | T(1) | T(i) | T(j) | T(n) | A(11) |

Here:

A(ij)	is the traffic from i to j;

A(ji)	is the traffic from j to i;

A(ii)	is the local traffic in exchange i;

O(i)	is the sum of all traffic originating in i;

T(j)	is the sum of all traffic terminating in j.

Adding all the row-totals O(i), i.e. the entries in column SO (sum originating traffic) give the total traffic A.. The same result is obtained by adding all the column-totals T(j), i.e. the entries in the row ST (sum terminating traffic). In short:

iO(i)=jT(j)=A..

As long as no confusion arises, the symbol A(i, j) can be used. However, it will frequently be necessary to distinguish between the present traffic from i to j A(i, j/0) and the estimated traffic at some future date t: A(i, j/t). Then, of course,

O(it)=jA(ijt)	and	T(jt)=iA(ijt)

4.3	Point-to-point forecast

Various methods exist to estimate A(i, j/t) based on the expected growth of the number of subscribers in areas (i) and (j), expected changes in traffic per subscriber, tec. The traffic matrix can then be completed by adding the entries row-wise to obtain the O(i/t) and column-wise to obtain the T(j/t).

For estimation of the future point-to-point traffics in a network, one usually bases the calculations on forecasted growth of subscriber lines and the present traffic matrix. Different formulae are used, of which the most common ones will be given below. It cannot be stated that one formula is more accurate than the other. Only feedback from future records may indicate which formula is the best for a particular case. When this has been found out, there is however no guarantee that this will be so for ever.

Estimation of total traffic

Taking into account that different categories of subscribers initiate different amounts of traffic, it may sometimes be possible to estimate a future traffic from:

A(t)=N1(t)⋅α1+N2(t)⋅α2+--			(4.3.1)

where N1(t), N2(t), etc., are the forecasted number of subscribers of category 1, 2, etc., and a1, a2, etc., are the traffic per subscriber of category 1, 2, etc.

If it is not possible to separate the subscribers into categories with different traffic, the future traffic may simply be estimated as:

A(t)=A(0)N(t)N(0)					(4.3.2)

where N(t) and N(0) are the number of subscribers at times t and zero.

Estimation of point-to-point traffic

For estimation of the traffic from one exchange to another, various formulae may be applied. The main idea is to take into account the increase of subscribers in the two exchanges and to apply certain weight factors to these growths.

Aij(t)=Aij(0)WiGi+WjGjWi+Wj				(4.3.3)

where Wi and Wj are the weights and Gi is the growth of subscribers in exchange i, and Gj in exchange j.

Gi=Ni(t)Ni(0)		Gj=Nj(t)Nj(0)			(4.3.4)

Different methods exist for Wi and Wj.

Rapp’s Formula 1

Wi=Ni(t)Wj=Nj(t)				(4.3.5)

The assumption here is that the traffic per subscriber from exchange i to exchange j is proportional to the number of subscribers in exchange j.

Rapp’s Formula 2

Wi=Ni(t)2Wj=Nj(t)2				(4.3.6)

This formula assumes that the change of originated and terminated traffic per subscriber is as small as possible.

Australian Telecom Formula

Wi=Ni(0)+Ni(t)2Wj=Nj(o)+Nj(t)2			(4.3.7)

This formula is a modification of Rapp’s Formula 1.

A fourth formula is derived as from the following assumption: the traffic per one subscriber in exchange i to all subscribers in exchange j is constant.

Aij(t)Ni(t)⋅Nj(t)=Aij(0)Ni(0)⋅Nj(0)Aij(t)=Aij(0)⋅Gi⋅Gj				(4.3.8)

These four formulae can be further adjusted by introducing N in the weighting factors.

Gravity Model

The traffic between two exchanges can be expressed as:

Aij=K(dij)⋅Ni⋅Nj					(4.3.9)

where K(dij)= community of interest factor.

It has been found that this factor, to a certain degree, depends on distances between the exchanges dij We can, therefore, write K(dij) as follows:

K(dij)=e−gdij					(4.3.10)

or	K(dij)=dij−g					(4.3.11)

The value for the parameters g or g can be calculated from a known traffic matrix. It may be necessary to adjust the expression for Aij for pairs of exchanges with special relations to each other; for example, a big factory in one part of a country and the head office in another part.

The formulae given here are all deducted under certain assumptions. One assumption is that the traffic per subscriber remains constant, another is that a subscriber has the same interest to call every other subscriber. It is, however, found in developed countries that new subscribers may have less traffic than the old ones. This is partly taken into account in the Australian formula.

It is in principle impossible to say that one formula is more reliable than the other. The forecaster should try different formulae and try to find out which one gives the most credible result for his particular case.

4.4	Kruithof’s double factor method

Kruithof’s method enables us to estimate the future individual traffic values,  A(i, j) in a traffic matrix. The values, at present, are assumed to be known and so is the future row and column sums.

The procedure is to adjust the individual A(i, j) so as to agree with the new row and column sums, i.e.

A(i, j) is changed to A(i,j)S1So

where So is the present sum and S1  the new sum for the individual row or column.

If we start with adjusting the A(i, j), with regard to the new row sums, Sj, these sums will agree, but the column sums will not. Next step is then to adjust the found values of A(i, j) to agree with the column sums. This makes the row sums disagree, so the next step is to adjust the new A(i, j) to agree withthe row sums. The procedure is continued until sufficient accuracy for both column and row sums has been reached. The iteration is rather fast and gives in general a satisfying result after about three corrections.

The procedure is better understood from the numerical example given below.

The method is applicable when the change of proportions between the individual A(i, j) is not expected to change much and in cases where it is not possible to forecast the individual A(i, j) in other ways.

Example of the use of Kruithof’s Double Factor Method

Consider a telephone network with two exchanges:

Given:

1.	The present traffic interests Aij(0)

| i | j | 1 | 2 | sum |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 10 | 20 | 30 |  |  |
| 2 |  | 30 | 40 | 70 |  |  |
| sum |  | 40 | 60 | 100 |  | (4.4.4) |

2.	Forecast of the future total originating and terminating traffic per exchange:

Ai⋅(t)andAj(t):

| i | j | 1 |  | 2 | sum |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 45 |  |
| 2 |  |  | ? |  | 105 |  |
| sum |  | 50 |  | 100 | 150 | (4.4.5) |

Problem:

Estimate the traffic values A(i, j/t) with Kruithof’s method.

Solution:

Iteration 1:		Row multiplication.
		Ai⋅ is distributed as given by present traffic interest.

Result:

| i | j | 1 | 2 | sum |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | 15 | 30 | 45 |  |
| 2 |  | 45 | 60 | 105 |  |
| sum |  | 60 | 90 | 150 |  |

Aij(1)=Aij(0)Ai⋅(0)Ai⋅(t)					(4.4.6)

After row multiplication, the sums of columns differ from the forecast. Next iteration will be column multiplication.

Iteration 2:		Column multiplication.
		Aj(2) is distributed as given by iteration 1.

Result:

| i | j | 1 | 2 | sum |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | 12.5 | 33.33 | 45.83 |  |
| 2 |  | 37.5 | 66.67 | 104.17 |  |
| sum |  | 50 | 100 | 150 |  |

Aij(2)=Aij(1)Aj(0)Aj(t)				(4.4.7)

After column multiplication, the sums of rows differ from the forecasted values. Next iteration will be row multiplication.

Iteration 3:		Row multiplication.
		Ai⋅ is distributed as given by iteration 2.

Result:

| i | j | 1 | 2 | sum |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | 12.27 | 32.73 | 45 |  |
| 2 |  | 37.80 | 67.20 | 105 |  |
| sum |  | 50.07 | 99.93 | 150 |  |

Aij(3)=Aij(2)Ai⋅(0)Ai⋅(t)					(4.4.8)

Iteration 4:		Column multiplication.

Aj is distributed as given by iteration 3.

Result:

| i | j | 1 | 2 | sum |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | 12.25 | 32.75 | 45 |  |
| 2 |  | 37.75 | 67.25 | 105 |  |
| sum |  | 50 | 100 | 150 |  |

Aij(4)=Aij(3)Aj(0)Aj(t)				(4.4.9)

After 4 iterations, the sums of rows and columns are equal to the forecasted values. We can now write:

Aij(t)=Aij(4)					(4.4.10)

5.	CONCLUSIONS

This chapter has been concerned mainly with the mathematics behind the forecasting methods. This will make it possible for the reader to be able to assist in the forecasting work, especially as concerns preliminary treatments of historical data.

Besides explaining when and why forecasts are needed, this chapter contained some frequently used methods of describing historical data in mathematical formulae. It should be emphasized again that the treatment of historical data is not forecasting. It is only treatment of historical data. Forecasting, on the other hand, is the art of guessing future development. Only in certain cases for short-term purposes will extrapolation of curves, obtained from historical data, provide reliable forecasts. In most cases, it must be judged if the future conditions and assumptions are the same as for the past. Adjustments of extrapolated values may be necessary for various reasons. Improved service performance may double the traffic within a month or two. New activities in society (e.g. industries) may cause drastic changes in traffic dispersion, etc.

Forecasting generally concerns the demand for subscriptions and traffic caused by various telecommunication services. Since demand quantities are difficult or impossible to measure, the historical data given to the forecaster are generally measured quantities, such as carried traffic. Therefore, the forecaster should first of all transfer the given data into demand data. If they concern a country and a service where the demand is well satisfied, the demand may be estimated with acceptable accuracy.

If the demand is far greater than the supply, it is very difficult to estimate the real demand. The forecaster has to take this difficulty into account as long as conditions do not permit a balance between supply and demand.

Another difficulty in using historical data is that they may not be sufficiently reliable to permit an analysis. One reason for this is that the data have been collected on different principles, for different purposes and at different times. It may even happen that there are no historical data available at all. Forecasting must then be based on other indirect indications, which does not generally improve the reliability of the forecast.

6.	REFERENCES

| BEAR, D. | Some Theories on Telephone Traffic Distribution; A critical survey (ITC-paper, Stockholm, 1973) |
| --- | --- |
| LEIJON, K.H. | Introduction to Practical Teletraffic Engineering. Section F. Forecasting (ITU) REM/72/038, Benghazi, 1976) |
| CCITT (GAS 5) | Methods Used in Long-term Forecasting of Domestic Telecommunication Demand and Required Resources (1980) |
| LONNQUIST, I. | Theory and Methods for Traffic Forecasts (TELE, Stockholm, 1968) |
| LONNQUIST, I. | Forecasting. Teletraffic Engineering Seminar, Istanbul, 1980 (ITU) |
| LONNSTROM, M.M. MARKLUND, F. MOO, I. | A Telephone Development Project, Stockholm, 1967 |
| NYGAARD, J. | An Integrated Telephone Traffic Forecating Tool for Use in Planning, I. (Teleteknik, Vol. XIV, 1970, No. 1) |
| RAPP, Y. | Some Economic Aspects on the Long-term Planning of Telephone Networks (Stockholm, 1969) |
| TURNER, W.M. | Forecasting. ITU/SIDA Seminar, New Delhi, 1975. SEM/ASIA/75 |
| CCITT (GAS 5) | Economic Studies at the National Level in the Field of Telecommunications |

Note:	This chapter has been edited from a manuscript prepared by K.H. Leijon, L.M. Ericsson, Sweden.

7.	EXERCISES

FORECASTING TECHNIQUES

An object is to be forecasted. The historical development was as follows:

| Year | Time Scale | Stock | Increase Absolute |  |
| --- | --- | --- | --- | --- |
| (at the end of) | (t) | (y) | % |  |
| 1968 | 1 | 583 |  |  |
| 1969 | 2 | 615 | 32 | 5.5 |
| 1970 | 3 | 646 | 31 | 5.0 |
| 1971 | 4 | 697 | 51 | 7.9 |
| 1972 | 5 | 738 | 41 | 5.9 |
| 1973 | 6 | 802 | 64 | 8.7 |
| 1974 | 7 | 844 | 42 | 5.2 |

Make a forecast for the expected stock 5 and 10 years ahead, in four different ways:

a.	Assuming an unchanged and constant absolute increase per year, based on the historical average increase.

b	Assuming an unchanged and constant percentage increase per year, based on the total increase over the past six years.

c.	By adapting a linear trend line to the historical data.

d.	By adapting an exponential trend to the historical data.

Plot the historical data in a diagram and show in the same diagrams the two trend lines c) and d) expanded to year 16 (6 + 10), and also the two other forecasts a) and b).

Compare the outcome from the forecasts.

Check the trend lines as concerns statistical reliability.

(Correlation coefficient, T - test and Durbin - Watson Test)

Discuss the credibility of the forecasts.

2.	Calculate the overall connection forecast for a local telephone network for 1983 (31.3.83)

Given:

GROWTH CHART OF DEMANDS IN A LOCAL TELEPHONE NETWORK

| As on | Equipped capacity | Working connect | Waiting list | Total demand |
| --- | --- | --- | --- | --- |
| 31/3/72 | 11800 | 9825 | 1901 | 11,726 |
| 31/3/73 | 12000 | 11114 | 1781 | 12,925 |
| 31/3/74 | 12100 | 11458 | 3555 | 15,013 |
| 31/3/75 | 12500 | 11530 | 5106 | 16,636 |
| 30/9/75 | 12500 | 11653 | 5662 | 17,315 |
| 31/3/76 | 12800 | 12275 | 2587 | 14,862 |
| 31/3/77 | 14150 | 13683 | 1624 | 15,307 |
| 31/3/78 | 15500 | 14437 | 1893 | 16,330 |

3.	Forecast the total originating traffic for December 1986 in a local telephone exchange.

Given:

a.	Traffic records for January 1979 - December 1981 (Table A)

b.	Calling rate records for January 1979 - December 1981 (Table B)

c.	Connection forecast for December 1986 = 2969

(1170 subscribers in January 1979 and 1600 in December 1981)

A.	TOTAL ORIGINATING TRAFFIC

| MONTH | 1979 | 1980 | 1981 |
| --- | --- | --- | --- |
| JANUARY | 38.6 | 39.4 | 45.6 |
| FEBRUARY | 37.9 | 43.7 | 46.2 |
| MARCH | 42.1 | 48.7 | 47.2 |
| APRIL | 40.6 | 43.8 | 46.2 |
| MAY | 40.1 | 40.2 | 45.6 |
| JUNE | 38.1 | 42.6 | 48.5 |
| JULY | 37.7 | 41.1 | 44.4 |
| AUGUST | 39.9 | 44.2 | 47.4 |
| SEPTEMBER | 40.4 | 41.0 | 49.1 |
| OCTOBER | 40.7 | 43.8 | 48.7 |
| NOVEMBER | 40.8 | 41.8 | 45.0 |
| DECEMBER | 42.2 | 49.5 | 49.5 |

B.	OVERALL ORIGINATING CALLING RATES 	(Erlang/subscriber)

| MONTH | 1979 | 1980 | 1981 |
| --- | --- | --- | --- |
| JANUARY | .033 | .031 | .033 |
| FEBRUARY | .033 | .034 | .033 |
| MARCH | .036 | .038 | .034 |
| APRIL | .035 | .033 | .033 |
| MAY | .034 | .030 | .032 |
| JUNE | .032 | .032 | .035 |
| JULY | .031 | .031 | .032 |
| AUGUST | .033 | .032 | .034 |
| SEPTEMBER | .034 | .030 | .035 |
| OCTOBER | .034 | .032 | .032 |
| NOVEMBER | .033 | .030 | .028 |
| DECEMBER | .034 | .036 | .031 |

4.	Consider a telephone network with two exchanges.

Given:

a.	The present traffic interest [Aij(0)]

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | 02 (o) |
| * |  | 10 | 20 | 30 |
| 2 |  | 30 | 40 | 70 |
| Tj (o) |  | 40 | 60 | 100 |

b.	Forecasted values on the future total originating and terminating traffics per exchange [0; (t) T; (t)]

|  | j |  |  |  |
| --- | --- | --- | --- | --- |
| i |  | 1 | 2 | oi (E) |
| 1 |  | ? | ? | 45 |
| 2 |  | ? | ? | 105 |
| Tj (t) |  | 50 | 100 | 150 |

Estimate the traffic values Aij(t)by using Kruithof’s method.

5.	Calculate the traffic interest between two exchanges in a local area.

Given:

a.	A local area is divided into traffic areas No. 1, 2, 3, 4, ... The future traffics between all traffic areas are forecasted.

b.	The local area is to be divided into exchange areas. The exchange areas do not coincide with the traffic areas.

c.	Try to calculate the expected future traffic from exchange A to exchange B.

d.	We have the following information:

Exchange A will get

- 5000 subscriber lines from traffic area I, which totally has 10,000 subscriber lines.

- 8000 subscriber lines from traffic area II, which totally has 12,000 subscriber lines.

Exchange B will get

- 9000 subscriber lines from traffic area III, which is all of the subscriber lines that are there.

- 2000 subscriber lines from traffic area IV, which totally has 6000 subscriber lines.

e.	We also know from the forecast the total expected traffic from each one, to each one of the involved traffic areas, as following:

| From traffic area No. | To traffic area No. | Total expected traffic |
| --- | --- | --- |
| I | III | 100 erl. |
| I | IV | 90 erl. |
| II | III | 105 erl. |
| II | IV | 95 erl. |

6.	Estimation of traffic interests between exchanges: The objective of the example is to illustrate how a traffic matrix for exchanges can be computed from a given traffic matrix for traffic areas.

Given:

An area consisting of three traffic areas: I, II, III:

Traffic Area 1

No. of subscribers		10,000

Total originated traffic/sub	0.06 erl.

Distribution of this traffic	60 %	I

25 %	II

15 %	III

Traffic Area II

No. of subscribers		5,000

Total originated traffic/sub	0.05 erl.

Distribution of this traffic	50 %	I

30 %	II

20 %	III

Traffic Area III

No. of subscribers		5,000

Total originated traffic/sub	0.04 erl.

Distribution of this traffic	50 %	I

25 %	II

25 %	III

The area is served by a number of exchanges 1, 2, ... etc.

Exchange 1

Total No. of subscribers	8,000

No. of subscribers belonging

to traffic area I		5,000

No. of subscribers belonging	3,000

to traffic area II

Exchange 2

Total No. of subscribers	6,000

No. of subscribers belonging	4,000

to traffic area I

No. of subscribers belonging	2,000

to traffic area III

Task:

Calculate the expected total traffic flow from exchange 1 to exchange 2.

Hint:

Start with calculation of traffic from one subscriber in traffic area I to one subscriber in traffic area II, etc.

7.	Present traffic matrix Aij(0) has been estimated:

|  | To |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| From |  | Exchange No. j |  |  |  |
|  |  | 1 | 2 | 3 | Sum |
| 1 |  | 25 | 30 | 45 | 100 |
| 2 |  | 35 | 55 | 110 | 200 |
| 3 |  | 60 | 85 | 155 | 300 |
| Sum |  | 120 | 170 | 310 | 600 |

The number of main lines per exchange in year t has been forecasted:

| Exchange No. | Ni (0) | Ni(t) |
| --- | --- | --- |
| 1 | 2000 | 3000 |
| 2 | 3500 | 3500 |
| 3 | 6800 | 7500 |

The main lines have not been classified into different categories since the proportion of different subscribers is expected to be the same in the future.

The total originating and terminating traffic per exchange is therefore forecasted from the formulae:

Ai⋅(t)=Ni(t)Ai(0)Ni(0)Aj(t)=Nj(t)Aj(0)Nj(0)

| Exchange No. | Ai (t) | Aj (t) |
| --- | --- | --- |
| 1 | 150.0 | 180.0 |
| 2 | 200.0 | 170.0 |
| 3 | 331.9 | 341.9 |
| Sum | 681.9 | 691.9 |

Since the sum of Ai⋅(t) and Aj(t) differ, we can use the mean value of these sums as an estimate of A⋅(t) and adjust the Ai⋅(t) andAj(t). This will give:

| Exchange No. | Ai (t) | A.j(t) |
| --- | --- | --- |
| 1 | 151.1 | 178.7 |
| 2 | 201.5 | 168.8 |
| 3 | 334.3 | 339.4 |
| Sum | 686.9 | 686.9 |

a.	Now draw a traffic matrix for the future year t and fill in the total traffics calculated above.

b.	Calculate the different point-to-point traffics by using the weighted growth methods, from known data about present and future number of main lines and the present point-to-point traffics. The type of weights is your own choice.

c.	Fill in these values in a new traffic matrix and calculate the sums of columns and rows. You will see that these sums do not agree with the values in the matrix you first drew.

d.	If we consider the traffic per main line to be constant during the forecast period, then the obtained point-to-point traffics must be corrected so that their sums agree with the forecasted total traffics.

If time permits it, do that correction by using Kruithof’s Double Factor method.

8.	Suppose that a busy hour trunk traffic forecast for exchange “A” of 75 erlangs is to be distributed over 5 routes to exchanges “B”, “C”, “D”, “E” and “F”, and for which no traffic data is available. Suppose also that the connection forecasts for the exchanges are:

A	10 000	(co)

B	  5 000	(c1)

C	  7 000	(c2)

D	  4 000	(c3)

E	  2 000	(c4)

F	10 000	(c5)

And that the distances from “A” are:

B	20 miles	(d1)

C	30 miles	(d2)

D	10 miles	(d3)

E	10 miles	(d4)

F	50 miles	(d5)

The situation is illustrated in the diagram below.

Estimate the distribution of the total trunk traffic over the five different routes A to B, A to C, ..., etc.

Hint:

The traffic forecast for a route can be obtained from

ti=cocidi2j=1ncocjdj2

where T is the total traffic.