TELECOMMUNICATION INDICATOR HANDBOOK

International Telecommunication Union

Organisation for Economic Co-operation and Development

D R A F T

µ1.	Introduction	1

Public telecom sector indicators	1

Reporting period	3

Demographic and macro-economic data	3

Derived indicators	4

Aggregated indicators	4

Telecom organizations	5

Telecom services	5

2.	Definitions	7

Telephone network size and dimension	7

1.	Telephone main lines in operation	7

2.	Total capacity of local public switching exchanges	8

3.	Percentage of main lines connected to digital exchanges	8

4.	Percentage of main lines for residential use	8

5.	Percentage of main lines in urban areas	8

6.	Public pay phones	9

Other services	9

7.	Number of telex subscriber lines	9

8.	Number of mobile telephone system subscribers	9

9.	Number of private leased circuits	9

10.	Subscribers to public data networks	10

Quality of Service	10

11.	Un-met applications for telephone main lines	10

12.	Percentage of telephone service faults cleared by next working day	10

13.	Percentage of calls which fail during the busy hour	10

14.	Faults per 100 telephone main lines per year	10

15.	Response time for operator services	10

Traffic	11

16.	Local telephone traffic	11

17.	National telephone traffic	11

18.	Total international traffic	11

19.	National telegrams	11

20.	International outgoing telegrams	12

21.	National telex traffic	12

22.	International telex traffic	12

Tariffs	12

23.	Telephone service installation charge	12

24.	Telephone service monthly subscription charge	12

25.	Telephone service national call charges	12

26.	Telephone service international call charges	14

27.	Mobile communication charges	14

28.	Leased line charges	14

29.	Packet-switched data communications network charges	15

Staff	16

30.	Total full-time staff in telecommunications services	16

Revenue and expenses	16

31.	Total revenues from all telecom services	16

32.	Revenues from telephone connection charges	17

33.	Revenues from telephone subscription charges	17

34.	Revenues from local and national calls	17

35.	Revenues from international calls	17

36.	Revenues from telegram and telex services	17

37.	Revenues from other data/text services	17

38.	Revenue from leased lines	17

39.	Revenue from mobile communications services	17

40.	Other revenues	17

41.	Total current expenditure for all telecommunication services	17

42.	Operational expenditure	17

43.	Interest	18

44.	Taxes	18

45.	Depreciation	18

46.	Other expenditures	18

Investment	18

47.	Total annual investment in telecommunication including land and buildings	18

48.	Total annual investment in telecommunication excluding land and buildings	18

49.	Annual investment in research and development	18

50.	Annual investment in telephone switching equipment.	18

BIBLIOGRAPHY	19

# 1.	Introduction

## Public telecom sector indicators

The purpose of this Handbook is to identify and define the most important indicators useful for analyzing the public telecommunication sector.  The term "Public telecommunication sector" refers to the telecommunications infrastructure and services provided on this infrastructure for the public at large.  This would include telecommunication networks (e.g., telephone, telex, telegraph, data)  which consist of exchanges (switches) linked by transmission circuits that connect subscribers to each other and with subscribers abroad.  Anyone can subscribe to the network; hence the term public which refers to the access arrangement rather than the ownership of the network. The public telecom sector does not include private networks that do not automatically connect to the public network or which have limitations on membership. The public telecom sector also excludes telecommunications equipment manufacturing and broadcasting. This Handbook provides a basic list of indicators that every network operator and regulatory agency should strive to collect and disseminate. Definitions are included to assist telecom analysts as well as the growing number of non-specialists interested in telecommunications to understand the data. Guidelines are provided for those responsible for collecting and disseminating the statistics in a particular country.

The list of indicators is shown in Table 1. The indicators are classified into different areas that provide a broad view of the public telecommunication sector. Telephone network size and dimension shows parameters for what is typically the largest component of the telecom sector, the telephone network. The parameters measure the total size and the number of users, how modern the network is and the composition of the user base. Enhanced and other services shows subscribers of newer services such as mobile telephones and data networks as well as the long standing telex service. Quality of service measures the reliability of the telephone network. Traffic measures the usage of the telephone, telegram and telex networks both nationally and internationally. Tariffs show the costs to the subscriber for using telecom services. Staff shows the number of people employed in the public telecom sector. Revenue and costs show the income and costs associated with providing telecom services. Investment shows expenditure for expanding and modernizing the network.

TABLE 1. Telecommunication indicators

|  | TELEPHONE NETWORK SIZE AND DIMENSION |
| --- | --- |
| 1. | Telephone main lines in operation |
| 2. | Total capacity of local public switching exchanges |
| 3. | Percentage of main lines connected to digital exchanges |
| 4. | Percentage of main lines which are for residential use |
| 5. | Percentage of main lines in urban areas |
| 6. | Public pay phones |
|  | Percentage of households with telephone |
|  | Number of telephone subscribers |
|  | Voice mail subscribers |
|  | OTHER SERVICES |
| 7. | Number of telex subscribers |
| 8. | Mobile telephone subscribers |
| 9. | Number of private leased circuits |
| 10. | Subscribers to public data networks of which: |
| 10a) | Subscribers to public packet switched data networks |
|  | Videotex subscribers |
|  | ISDN subscribers |
|  | Number of radio pager subscribers |
|  | QUALITY OF SERVICE |
| 11. | Un-met applications for telephone main lines/Time to meet new applications |
| 12. | Percentage of telephone service faults cleared by next working day |
| 13. | Percentage of calls which fail during the busy hour |
| 14. | Faults per 100 telephone main lines per year |
| 15. | Percentage of calls for operator service answered within 15 seconds |
|  | Customer satisfaction rate |
|  | Time to achieve dial tone |
|  | Pay phone availability |
|  | Billing queries |
|  | TRAFFIC |
| 16. | Local telephone traffic |
| 17. | National telephone traffic |
| 18. | International telephone traffic (outgoing/incoming) |
| 19. | Number of national telegrams |
| 20. | Number of international outgoing telegrams |
| 21. | National telex traffic |
| 22. | International outgoing telex traffic |
|  | TARIFFS |
| 23. | Telephone service installation charge |
| 24. | Telephone service monthly subscription charge |
| 25. | National call charges |
| 26. | International call charges |
| 27. | Mobile communications charges |
| 28. | Leased line charges |
| 29. | Packet-switched data communications network charges |
|  | STAFF |
| 30. | Total full-time staff in telecommunication services |
|  | Total full-time staff in telephone service |

|  | REVENUE AND COSTS |
| --- | --- |
| 31. | Total revenues from all telecommunication services of which: |
| 32. | Revenues from telephone connection charges |
| 33. | Revenues from telephone subscription charges |
| 34. | Revenues from local and national calls |
| 35. | Revenues from international calls |
| 36. | Revenues from telegrams and telex services |
| 37. | Revenues from other data/text services |
| 38. | Revenue from leased lines |
| 39. | Revenue from mobile communications services |
| 40. | Other revenues |
| 41. | Total expenditure for telecommunication services of which: |
| 42. | Operational expenses |
| 43. | Net interest payments/receipts |
| 44. | Taxes |
| 45. | Depreciation |
| 46. | Other expenditures |
|  | INVESTMENT |
| 47. | Total annual investment in telecommunications, including land and buildings |
| 48. | Total annual investment in telecommunications, excluding land and buildings of which: |
| 49. | Annual investment in research and development |
| 50. | Annual investment in telephone switching equipment |
|  | Fixed assets |

## Reporting period

The indicators should be collected at least on an annual basis. Historical data should be retained for measuring trends and forecasting future demand. The use of a common reporting year (e.g., end of the calendar year) is desirable for comparative purposes. While this is not always possible due to differing national practices, all the data should at least refer to the same end period to enhance the precision of derivations. For example, operational data and financial data should refer to the same end period to measure revenue per subscriber line accurately.

## Demographic and macro-economic data

The list of indicators does not contain demographic or macro-economic statistics. These data are needed for gauging telecom network penetration and impact on the overall economy. Demographic and macro-economic statistics are regularly collected and disseminated by national statistical agencies and economic and finance ministries as well as regional and international organizations (e.g., OECD, UN, World Bank, International Monetary Fund). Some of the main publications containing demographic and economic statistics are shown in the bibliography.

Demographic and macro-economic data useful for telecommunication analysis are described in Table 2:

Table 2. Useful demographic and macro-economic indicators

| Demographic | Population size is essential for measuring network penetration (e.g., telephone lines per 100 inhabitants). Number of households and percentage of population in urban areas is also useful. |
| --- | --- |
| Employment | Useful for expressing telecommunications employment as a percentage of total employment. |
| Exchange rate | Needed for converting financial data to a standard unit for inter-country comparisons such as the US dollar or the ECU. It is preferable to use an average exchange rate since telecom financial operations typically accrue over a period of time. For tariff comparisons, it is preferable to use Purchasing Power Parties (PPPs) which compensate for the different cost of living between countries. |
| Gross Domestic Product (GDP) | Useful for gauging the size of the telecom sector vis-a-vis the overall domestic economy (e.g., telecom revenues as a percent of GDP). |
| Gross Fixed Capital Formation (GFCF) | Useful for measuring the share of telecom investment to total investment in the economy (e.g., telecom investment as a percent of GFCF). |
| Inflation Price Index | Inflation price indices are useful for correcting financial data for the effects of inflation. Common prices indices include the consumer price index or the GDP deflator though for investment time-series it is preferable to use a GFCF Deflator. |

## Derived indicators

No derived statistics are shown since these can be calculated from the primary indicators. For example, main lines per 100 inhabitants can be calculated from main lines and population, telecom staff per 1,000 main lines can be derived from full-time staff and main lines, etc.

## Aggregated indicators

The telecom indicators are useful for intra-country (when more than one national operator) as well as inter-country comparisons. If there is more than one operator in a country, their results need to be added to obtain an overall total so that international comparisons can be made with compensation made for inter-company traffic and revenue flows to avoid double-counting. Common reporting periods and indicators (as contained in the list) are important for obtaining accurate country totals and enhancing transparency in comparisons. The United States Federal Communications Commission (FCC) annual Yearbook provides a good example of how a country total can be obtained where there are many operators providing service. See Table 3.

Table 3. Aggregation Example

| 31.12.1990 | TOTAL US | AT&T Communications, Inc. | Seven Regional Bell Operating Companies | Other Reporting Local Exchange Companies |
| --- | --- | --- | --- | --- |
| Full-time employees | 639470 | 76869 | 440466 | 122135 |
| Total switched access lines | 126388961 |  | 103874183 | 22514778 |
| Total operating revenues (thousands of dollars) | 117423237 | 33533605 | 67010637 | 16878995 |

Source: Adapted from FCC. Statistics of Communications Common Carriers. 1990/91 Edition. US Government Printing Office, Washington D.C.

## Telecom organizations

In order to obtain a complete picture of the public telecom sector in the country, it is also useful to know the names of the agency responsible for enacting the telecom laws of the country (typically a ministry), the entity responsible for supervising and regulating the operators and a list of operators which have been licensed and the services they provide.

Table 4. Telecom organizations example

| Ministry | Department of Trade and Industry |
| --- | --- |
| Regulator | Office of Telecommunications (OFTEL) |
| Operator(s) | British Telecom |
|  | Mercury Communications, Inc. |
|  | Kingston Telecommunications |

## Telecom services

It is also useful to know the various telecom services available in a country. A sample list is shown in Table 5.

Table 5. Telecommunication Services

CALL WAITING

CALL FORWARDING

GROUP CALLING

COUNTRY DIRECT SERVICE

800 (GREEN) NUMBERS

ISDN

VIDEO CONFERENCE

VIDEOTEX

ELECTRONIC MAIL

900 SERVICE

RADIO PAGING

COBRO REVERTIDO

MOBILE TELEPHONE

PACKET-SWITCHING NETWORK

TELEX

HIGH SPEED DATA COMMUNICATIONS

ELECTRONIC YELLOW PAGES

TELECOMMUNICATION CREDIT CARDS

Note:  This list is intended to be indicative rather than comprehensive.

# 2.	Definitions

## Telephone network size and dimension

### 1.	Telephone main lines in operation

A “main line” is a telephone line connecting the subscriber's terminal equipment to the public switched network and which has a dedicated port in the telephone exchange equipment. This term is synonymous with the term “main station” or "Direct Exchange Line" (DEL) which are commonly used in telecommunication documents. It may not be the same as an access line or a subscriber (see below).

It is understood that:

-the line connected to the telephone exchange may be either an exclusive exchange line or a shared line;

-when a subscriber's equipment has several extensions (private branch exchange), the number of main lines is equal to the number of lines connecting the installation to the telephone exchange, whether these lines are operated in one direction or in both directions.

Example:

A subscriber's equipment with extensions is served by 50 lines which connect it to the telephone exchange. The installation has ten operating positions (and therefore ten “operator's stations”) and 500 extensions. In accordance with the above definition this installation must be counted as having 50 main lines (i.e. as many as there are lines connecting the subscriber installation to the exchange).

In other words, it will not be counted as having:

-either one main line (which would refer to the installation),

-or ten main lines (which would correspond to the number of operator's stations),

-or 500 main lines (which correspond to the extensions).

A distinction should be noted between subscriber and main line. Subscribers (e.g., customer's which are billed individually) may share the same line (e.g., party line). Thus one main line could serve several subscribers. Some operators are not in a position to report main lines and report the number of subscribers. This should be described in a note.

Similarly, some operators/administrations report access lines rather than main lines. Access lines typically include extensions on Private Automatic Branch Exchanges (PABXs) that can be billed separately or that have their own telephone number. Operators/administrations that report access lines rather than main lines should provide their definition; if possible they should specify the number of extensions counted as access lines.

A further pattern arises in the definition of high speed lines which contain many voice channels compressed on the same circuit. For instance, a large organization may lease a single 1.5 or 2.0 Mbit/s circuit giving multiple voice channels. In this instance, for purposes of comparison, it would be better to count the number of channels (multiple) rather than the line (single)

### 2.	Total capacity of local public switching exchanges

The total capacity of public switching exchanges corresponds to the maximum number of main lines which can be connected. This number includes, therefore, main lines already connected and main lines available for future connection, including those used for the technical operation of the exchange (test numbers).The measure should be the actual capacity of the system rather than the theoretical potential when the system is upgraded or compression technology is employed.

### 3.	Percentage of main lines connected to digital exchanges

This percentage is obtained by dividing the number of main lines connected to digital  telephone exchanges by the total number of main lines. It should be made clear that this indicator does not measure the percentage of exchanges which are digital, the percentage of inter-exchange lines which are digital are even the percentage of digital network termination points, though these may be useful measures in their own right.  Respondents should indicate of the main lines included in the definition are only those in operation (indicator 1) or the total capacity (indicator 2).

### 4.	Percentage of main lines for residential use

This percentage is obtained by dividing the number of main lines serving households (i.e., lines which are not used for business, government or other professional purposes or as public telephone stations) by the total number of main lines.  Respondents should indicate the definition of "households" which is being applied.

### 5.	Percentage of main lines in urban areas

This percentage is obtained by dividing the number of main lines in urban areas by the total number of main lines in the country.

The definition of "urban" used by the country should be supplied. See table 6 for an illustrative example.

Table 6. Distribution of Telephones (DELs) between Urban and Rural Areas - 31st March 1991

|  | No. of DELs (000 lines) | DELs (%) |
| --- | --- | --- |
| Urban |  |  |
| 2 M.T.N.L. | 1213 | 23.90 |
| 2 Metro City Districts | 434 | 8.55 |
| 15 Major City Districts | 777 | 15.31 |
| 281 Other Cities | 1241 | 24.46 |
| 3396 Towns with population 5000 & above | 880 | 17.34 |
| Total Urban | 4545 | 89.56 |
| Rural | 530 | 10.44 |
| All India | 5075 | 100.00 |

Note: Villages: (=Population less than 5,000)

Source: Department of Communications. Indian Telecommunication Statistics 1991. New Delhi. p. 5.

### 6.	Public pay phones

Public telephones refers to the total number of all types of public telephones including coin and card operated and public telephones in call offices. Public phones installed in private places should also be included as should mobile public telephones. All public telephones regardless of capability (e.g., local calls or national only) should be counted.  Where the national definition of "pay phone" differs from that above (e.g. by excluding pay phones in private places) then respondents should indicate their own definition.

## Other services

### 7.	Number of telex subscriber lines

A telex subscriber line is a line connecting the subscriber's terminal equipment to the public telex network and which has a dedicated port in the telex exchange equipment.

### 8.	Number of mobile telephone system subscribers

Subscribers refer to users of portable telephones subscribing to an automatic public mobile telephone service which provides access to the Public Switched Telephone Network (PSTN). This can be further sub-divided by non-cellular systems and cellular systems (analogue, digital). Subscribers to public mobile data services or radio paging services should not be included in this definition.

### 9.	Number of private leased circuits

Private leased circuits refer to a two-way link for the exclusive use of a subscriber regardless of the way it is used by the subscriber (e.g., switched subscriber or non-switched, or voice or data). Private circuits, also referred to as leased lines, can be either national or international in scope. In reporting this indicator, only the number of lines should be included, not the number of network termination points. A separate indicator should be calculated for the number of private networks.

### 10.	Subscribers to public data networks

The number of subscribers to public data networks including packet-switched networks, circuit-switched networks, videotex networks, ISDN (Integrated Services Digital Networks) and dial-up data networks. Separate indicators can be reported for each. Countries should specify in a note what networks they are including.

## Quality of Service

### 11.	Un-met applications for telephone main lines/Time to meet new applications

Applications for connection to the PSTN which have had to be held over (e.g., waiting list) owing to a lack of technical facilities (equipment, lines, etc.). It should be specified what is the normal period for responding to requests for a new line (for instance, no more than two weeks from the date of the request).  Separate indicators should be collected for waiting times to other services, e.g., leased lines, mobile communications, packet switched data communications.

### 12.	Percentage of telephone service faults cleared by next working day

Faults cleared by the next working day refers to the percentage of faults reported to the PSTN that have been corrected by the end of the next working day. (e.g., not including non-working days (weekends)). Separate indictors of fault clearance should be collected for other services.

### 13.	Percentage of calls which fail during the busy hour

The percentage of unsuccessful calls refers to the number of calls that could not be completed due to technical problems for which the operator is responsible (e.g., network congestion, line failure) divided by the total number of calls (successful and unsuccessful). Numbers that are engaged, called party not available or subscriber error should not be included as incomplete calls. Separate indicators of call failure should be collected for other services. For leased lines, percentage of time the line is un-available to the customer is a better indicator.

### 14.	Faults per 100 telephone main lines per year

The total number of reported faults per 100 main telephone lines for the year. This is calculated by dividing the total number of reported faults for the year by the total number of main lines in operation and multiplying by 100. Countries should specify whether faults due to faulty terminal equipment on the customer's premises is included in the indicator or not. In a liberalized terminal market, these faults which are not the direct responsibility of the public telecommunications operator should probably be excluded.

### 15.	Response time for operator services

The percentage of calls for operator services answered within 15 seconds. This should include directory inquiry and operator assistance calls. A separate indicator should be collected for response times for emergency service calls.

Possible additions:

Customer satisfaction rate

Pay phone availability (i.e. number in working order on a given day or over a given period).

Time to achieve dial tone (Percentage of time dial-tone available within three seconds)

Billing queries

## Traffic

### 16.	Local telephone traffic

Local traffic consists of effective (completed) traffic exchanged within the local charging area in which the calling station is situated. This is the area within which one subscriber can call another on payment of the local charge (if applicable).  This indicator should be reported in the number of calls (messages) and/or minutes. If the indicator is reported in meter units or pulses, then an appropriate conversion figure to calls/minutes of traffic should be supplied.

Each country should include a footnote explaining its definition of the “local charging area” and indicating the number of such areas and their average size (in km2).

### 17.	National telephone traffic

National trunk (toll) traffic consists of effective (completed) national traffic exchanged with a station outside the local charging area of the calling station. As above, the indicator should be reported as the number of calls and/or minutes of traffic.

### 18.	Total international traffic

This covers all the effective (completed) traffic originating in a given country to destinations outside that country. The indicator should be reported in number of calls or in minutes of traffic. If available, the total traffic terminating in the country should also be reported, where possible, bilateral traffic to particular countries should be reported (see Table 7).

Table 7. Sample table for providing international bi-lateral telephone traffic (minutes)

|  | Incoming/Receiving | Outgoing/Calling |
| --- | --- | --- |
| Country 1 |  |  |
| Country 2 |  |  |
| Country 3 |  |  |
| Country n |  |  |

### 19.	National telegrams

The number of charged telegrams which both originate and terminate within the same country.

### 20.	International outgoing telegrams

The number of charged outgoing full rate telegrams originating in a given country with a destination outside the country.

### 21.	National telex traffic

All the telex traffic which both originates and terminates within the same country.

### 22.	International telex traffic

All the outgoing telex traffic originating in a given country with a destination outside the country. If available, the total incoming telex traffic terminating in a given country should also be reported.

## Tariffs

Note: All tariffs should be expressed in local currency at current prices. A separate statement should be made concerning tax rates (e.g., sales tax, value-added tax); whether tax is included or not, at what rate it is levied and whether it can be reclaimed by businesses.

### 23.	Telephone service installation charge

Installation refers to one-off charges involved in applying for basic telephone service. Where there are different charges for different exchange areas the charge for the largest urban area should be used and specified in a note. Where there are different installation charges for residential and business consumers or for first and subsequent lines, these should be stated separately.

### 24.	Telephone service monthly subscription charge

Monthly subscription refers to the recurring fixed charge for subscribing to the PSTN. The charge should cover the rental of the line but not the rental of the terminal (e.g., telephone set) where the terminal equipment market is liberalized. Separate charges should be stated where appropriate, for residential and business subscribers or for first and subsequent lines. Responders should also indicate if the rental charge includes any allowance for free or reduced rate call units, or the largest urban area should be used and specified in a note.

### 25.	Telephone service national call charges

Local calls refers to the cost of a peak rate 3-minute call within the same exchange area using the subscriber's own terminal (i.e., not from a public telephone) and national (long-distance) calls within the country. See Table 8 for an example of how this information might be provided.

Table 8:  Tariff information required for national telephone basket

| Charges for PSTN calls at 1 January 1992 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country: | Austria |  |  |  |  |  |  |  |  |  |
| Charges in local currency |  |  |  |  |  |  |  |  |  |  |
|  |  | Business |  | Residential |  |  |  |  |  |  |
| Fixed Charge |  |  |  |  |  |  |  |  |  |  |
| Installation |  |  | 1200 |  |  | 1200 |  |  |  |  |
| Rental (per month) |  |  | 160 |  |  | 160 |  |  |  |  |
| Free units included |  |  | 0 |  |  | 0 |  |  |  |  |
| Taxes |  |  | 0% |  |  | 0% |  |  |  |  |
| Comment: | No tax |  |  |  |  |  |  |  |  |  |
| Call Charge |  | Minimum charge per call |  | Charge per minute |  |  |  |  |  |  |
| Distance |  |  |  |  |  |  |  |  |  |  |
| 3 km |  | 0.80 |  |  | 0.67 |  |  |  |  |  |
| 7 km |  | 0.80 |  |  | 0.67 |  |  |  |  |  |
| 12 km |  | 0.80 |  |  | 0.67 |  |  |  |  |  |
| 17 km |  | 0.80 |  |  | 0.67 |  |  |  |  |  |
| 22 km |  | 0.80 |  |  | 0.67 |  |  |  |  |  |
| 27 km |  | 0.80 |  |  | 4.00 |  |  |  |  |  |
| 40 km |  | 0.80 |  |  | 4.00 |  |  |  |  |  |
| 75 km |  | 0.80 |  |  | 4.00 |  |  |  |  |  |
| 110 km |  | 0.80 |  |  | 6.00 |  |  |  |  |  |
| 135 km |  | 0.80 |  |  | 6.00 |  |  |  |  |  |
| 175 km |  | 0.80 |  |  | 6.00 |  |  |  |  |  |
| 250 km |  | 0.80 |  |  | 6.00 |  |  |  |  |  |
| 350 km |  | 0.80 |  |  | 6.00 |  |  |  |  |  |
| 490 km |  | 0.80 |  |  | 6.00 |  |  |  |  |  |
| Discounts by time of day or week (as percentage of peak rate) |  |  |  |  |  |  |  |  |  |  |
|  |  | Mon-Fri | Mon-Fri |  | Mon-Fri | Mon-Fri |  | Sat |  | Sun |
|  |  | 11:00 | 15:00 |  | 20:00 | 03:00 |  | 11:00 |  | 13:00 |
| Distances |  |  |  |  |  |  |  |  |  |  |
| 3,7 km |  | 100% | 100% |  | 100% | 100% |  | 100% |  | 100% |
| 12 km |  | 100% | 100% |  | 100% | 100% |  | 100% |  | 100% |
| 17 km |  | 100% | 100% |  | 100% | 100% |  | 100% |  | 100% |
| 22 km |  | 100% | 100% |  | 100% | 100% |  | 100% |  | 100% |
| 27 km |  | 100% | 100% |  | 67% | 67% |  | 67% |  | 67% |
| 40 km |  | 100% | 100% |  | 67% | 67% |  | 67% |  | 67% |
| 75 km |  | 100% | 100% |  | 67% | 67% |  | 67% |  | 67% |
| 110, 135, 175 km |  | 100% | 100% |  | 67% | 67% |  | 67% |  | 67% |
| 250km+ |  | 100% | 100% |  | 67% | 67% |  | 67% |  | 67% |

Note:  The same basic tariff information can be used for the business, residential and mobile communications baskets.

### 26.	Telephone service international call charges

This is the cost of a peak rate 3-minute direct dialled (i.e., without operator intervention) call from a destination within the country to a destination outside the country. The rate should be supplied for peak rate time calls and off-peak (discount) rate calls (if applicable). The cost should be reported in national currency, with a statement on what taxes are applied. See Table 9 for an example of how this data might be provided.  In countries where the first minute is charged at a higher rate than subsequent minutes, the first three minutes of a four minute call could be quoted.

Table 9. Sample table for providing international call charges (3 minute direct-dialled peak rate call in local currency and including any taxes)

|  | Standard/Peak rate | Economy/Discount rate |
| --- | --- | --- |
| Country 1 |  |  |
| Country 2 |  |  |
| Country 3 |  |  |
| Country n |  |  |

### 27.	Mobile communication charges

The connection charge and monthly subscription charge and call charges by distance, duration and time of day/week, for mobile telephone service.  Separate information should be provided for analogue (e.g. TACS,  NMT, AMPS)) and digital (e.g. GSM) services where applicable (see table 8).

### 28.	Leased line charges

Connection charge and monthly rental charge. Costs should be specified for different speeds (e.g., 2.4, 4.8, 9.6, 19.2, 56/64 kbit/s and 1.5/2.0 Mbit/s) and different distances (see table 10).

Table 10:  Tariff information required for  leased lines basket

| Charges for leased lines at 1 January 1992 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country: | Australia |  |  |  |  |  |  |
| Charges in local currency |  |  |  |  |  |  |  |  |  |
| Tax: |  | 0.00 |  |  |  |  |  |
| Comment: |  | No tax |  |  |  |  |  |
| Connection charge: |  |  |  |  |  |  |  |  |
|  | Distance | 2 km | 20 km | 50 km | 100 km | 200 km |  |
| Capacity |  |  |  |  |  |  |  |
| 9.6 kbit/s |  | 440.00 | 440.00 | 645.00 | 645.00 | 645.00 |  |
| 56/64 kbit/s |  |  | 3000.00 | 3000.00 | 3000.00 | 3000.00 | 3000.00 |  |
| 1.5/2.0 Mbit/s |  |  | 5028.00 | 5028.00 | 5028.00 | 5028.00 | 5028.00 |  |
| Rental per line per month |  |  |  |  |  |  |  |  |  |
|  | Distance | 2 km | 20 km | 50 km | 100 km | 200 km |  |
| Capacity |  |  |  |  |  |  |  |
| 9.6 kbit/s |  | 74.00 | 225.50 | 412.50 | 450.00 | 525.00 |  |
| 56/64 kbit/s |  |  | 216.00 | 303.00 | 498.00 | 773.00 | 782.00 |  |
| 1.5/2.0 Mbit/s |  |  | 1729.00 | 3421.00 | 4962.00 | 6221.00 | 7542.00 |  |

### 29.	Packet-switched data communications network charges

Connection, monthly rental charge and call set-up charges for packet-switched data communication (see table 11)

Table 11:  Tariff information required for  X.25 data communications basket

| Charges for X.25 Packet Switched data communications at 1 January 1992 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Country: | Australia |  |  |  |  |  |  |
| Charges in local currency |  |  |  |  |  |  |  |  |  |
| Connection charge: |  |  | 1275.00 |  |  |  |  |  |
| Rental Charge: |  |  | 686.00 |  |  |  |  |  |
| Tax: |  | 0.00 |  |  |  |  |  |
| Comment: |  | No tax |  |  |  |  |  |
|  |  |  | local |  | long distance |  |  |  |
| Call set-up charge |  |  |  | 0.00 |  | 0.00 |  |  |
| Charges per segment |  |  |  | local |  | long distance |  |  |  |
|  | peak rate |  | 0.0012 |  | 0.0012 |  |  |
|  | discount (cheapest) |  |  | 0.00012 |  | 0.00012 |  |  |
| Cost per minute |  |  |  | local |  | long distance |  |  |  |
|  | peak rate |  | 0.0063 |  | 0.0063 |  |  |
|  | discount (cheapest) |  |  | 0.00063 |  | 0.00063 |  |  |

## Staff

### 30.	Total full-time staff in telecommunications services

Full-time staff employed by telecommunication network operators in the country for the provision of public telecommunication services. Part-time staff should be expressed in terms of full-time staff equivalents.  As far as possible, staff not working principally for the provision of telecommunications services (e.g. those working in postal services or cable TV operations) should be excluded.

An indication of the percentage of functions carried out by contractors could be specified in a footnote.

## Revenue and expenses

All items in this section should be reported in national currency at current prices.

Revenue (turnover) consists of telecommunications service earnings during the financial year under review. Revenue should not include monies received in respect of revenue earned during previous financial years, neither does it include monies received by way of loans from governments, or other external investors, nor monies received from repayable subscribers' contributions or deposits.

### 31.	Total revenues from all telecom services

This is the total revenue earned and is the sum of items 32-40. This should exclude revenues from non-telecom services (e.g., broadcasting, cable TV) except where these services are provided on an interactive basis (e.g., videotex) or on an end-to-end basis (e.g., programme exchange). The revenue figure should include indirect payment (e.g., repayments from investments in regional or international satellite operating companies) and should take into account any net surplus or deficit on international settlements with foreign operators.

### 32.	Revenues from telephone connection charges

Revenue received for connection (installation) of telephone service. This may include charges for transfer or cessation of service.

### 33.	Revenues from telephone subscription charges

Revenues from recurring charges for subscription to PSTN including equipment rentals.

### 34.	Revenues from local and national calls

Revenues from local and national telephone calls.

### 35.	Revenues from international calls

Revenues for telephone calls originating/terminating outside the country. This may include any surplus income from subscribers, other national and foreign telecommunication operators, governments, etc., after deduction of the share of this income to be paid to other organizations for outgoing telecommunication traffic (Administrations of the incoming and possibly transit countries).

### 36.	Revenues from telegram and telex services

Revenues for transmittal of telegraphic and telex messages, both national and international.

### 37.	Revenues from other data/text services

Revenues from the data and text services such as data communications (e.g., packet switching) but not telegram or telex.

### 38.	Revenue from leased lines

Revenues from the provision of leased lines (circuits).

### 39.	Revenue from mobile communications services

Revenues from the provision of mobile communications services.

### 40.	Other revenues

Any other revenues not accounted for elsewhere for the provision of public telecommunication services. Responders should indicate in a note what are the main sources or "other" telecommunications revenues.

### 41.	Total current expenditure for all telecommunication services

Current expenditure means expenditure other than investments; it consequently refers to the running of telecommunication services on an annual basis. It is further sub-divided (see 42 to 46 below).

### 42.	Operational expenditure

Operational expenditure would include:

-salaries, benefits, etc. of operational staff, pensions, and other labour costs,

-materials for operations including rental of premises, office supplies etc.,

-labour and material costs of maintenance and repair of the existing telecommunications systems.

### 43.	Interest

Interest refers to the financial year for loans associated with fixed and current assets. The sum should be interest payable by the PTO on debts minus the any income gained from investments.

### 44.	Taxes

Taxation refers to taxes on the operator's income, expenditure, profit or capital (e.g. corporation tax, income tax, excise taxes, non-recoverable VAT and local land taxes) raised by central or local government. This item does not include pay-related (e.g. social) taxes which should be incorporated under current expenditures

### 45.	Depreciation

Depreciation covers the expected devaluation of capital assets insofar as this is regarded as an item of current expenditure.

Financial charges made in the year for the loss of value of installed equipment,. It is normally calculated on hypotheses based on the useful life of the different categories of equipment.

### 46.	Other expenditures

Any other expenditures not connected with operation, depreciation, interest or taxation.

## Investment

The term “investment” means the expenditure associated with acquiring the ownership of property (including intellectual and non-tangible property such as computer software) and plant. These include expenditure on initial installations and on additions to existing installations where the usage is expected to be over an extended period of time. Also referred to as capital expenditure.

### 47.	Total annual investment in telecommunication including land and buildings

The annual investment for acquiring property and plant.

### 48.	Total annual investment in telecommunication excluding land and buildings

The annual investment for acquiring plant (e.g., switching equipment, transmission equipment, office machinery, motor vehicles) but not including land or buildings.

### 49.	Annual investment in research and development

Annual investment (in current prices and national currency) for research and development including software development and the preparation of new service offerings.

### 50.	Annual investment in telephone switching equipment.

The annual investment for telephone switching equipment such as local, national (trunk) and international exchanges.

# BIBLIOGRAPHY

Department of Telecommunications. 1992. Indian Telecommunication Statistics 1991. New Delhi. Useful example of telecom indicator collection for large developing country.

Federal Communications Commission. 1991. Statistics of Communications Common Carriers. Washington D.C. Useful examples of aggregation of data from many operating agencies.

International Monetary Fund. International Financial Statistics. Various years. Washington D.C. Source of macro-economic data.

ITU. Various years. Yearbook of Statistics. Geneva. Historical raw data of public telecommunication sector for most countries and territories.

----. Various years. Africa, Americas, Arab States, Asia-Pacific and Europe Telecommunications Indicators. Geneva. Detailed regional coverage with totals and averages.

OECD. 1990. Performance Indicators for Public Telecommunication Operators. Paris. Policy-oriented uses of performance indicators.

OECD. Communications Outlook 1993. Paris. Policy-oriented uses of performance indicators for OECD member countries.

UN. Department of International Economic and Social Affairs. Various years. Monthly Bulletin of Statistics. New York. Source of population, consumer price index and exchange rate data.

World Bank. Various years. World Development Report. Washington D.C. Demographic and macro-economic data.

____________________________